3GPP TSG-RAN RAN2 #123						R2-2308631

Toulouse, France, 21 – 25 August, 2023

Agenda Item:		7.16.2.1

Source:			Huawei, HiSilicon

Title:			Discussion on general aspects

Document for:		Discussion and Decision

1   Introduction

According to the agenda for 7.16.2.1, this paper is to discuss more details for model Id and mapping of functionality to entities.

7.16.2.1	Architecture and General

Discussion on Model ID is postponed. Based on RAN1 progress, Can discuss the AIML algorithm dependency on locality (e.g. cell specific), UE-side AIML dependency on gNB configuration etc, dependency on other aspects such as UE speed, Network-side AIML dependency to be UE specific etc, and the related procedure impacts. Can discuss the expected impacts for Network Side-algorithms.

UE Cap: On a high level, Identify potential impacts to RRC and LPP UE capabilities or equivalent functionality if any.

Progress the logical arch (if needed).

Mapping of Functionality to entities, general aspects.

Including [Post122][060][AIML] Mapping of functions to physical entities (CMCC)

In this paper, we provide our analysis for the above areas.

2   Discussion

2.1 Discussion on AIML dependency

At RAN1#112b meeting, the following agreements were made. The highlighted parts are about the additional conditions for applying functionality or model.

Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). - FFS: relationship between functionality-based LCM and model-ID-based LCM - Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

Based on the above RAN1 progress, we observe that RAN1 is still discussing the necessity, content, and relations to model/functionality identification aspects. To our understanding, these requirements are important before RAN2 discussing anything on the additional conditions.

Observation 1: For additional conditions for model-based LCM and functionality-based LCM, RAN1 is still discussing the necessity, content, and other aspects.

From RAN2 point of view, it is hard to see which LCM components may be impacted for now. For example, if RAN1 identifies that some conditions are helpful for functionality-based LCM, then whether/what conditions are needed at UE side and NW side may be discussed later.

In addition, for the following statements, we think they are related to evaluation discussion and thus RAN1 can provide more feedbacks, if applicable. For example, whether UE-sided models are dependent on UE speed, and if yes, how to co-ordinate a UE-sided model and UE speed information.

UE-side AIML dependency on gNB configuration etc, dependency on other aspects such as UE speed, Network-side AIML dependency to be UE specific etc

Observation 2: For AIML dependency, more RAN1 inputs on evaluations are helpful for RAN2 discussions.

2.2 Mapping of functionality to entities

In the email discussion, the mapping of functionality to entities is being discussed. In our RAN2 paper [2][3], we have provided some analysis on entity mapping for model transfer/delivery and control respectively.

2.3 Functionality-based LCM

In the RAN1#112bis-e meeting, for AIML functionality identification and functionality-based LCM, the following agreement is achieved:

Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda.

We observe that RAN1 has separate discussions on model-based LCM and functionality-based LCM, and we think the two directions should be separately treated and concluded in the future.

We understand that some common discussions have been made in RAN1 and RAN2 as there are some overlaps between two directions and common discussions may be more efficient. However, it is important to understand the specialities for both directions.

In our understanding, the following LCM components may be needed for functionality-based LCM:

- Data collection, e.g. for monitoring

- Control

Other LCM components (e.g. model transfer/delivery) are not needed for functionality-based LCM.

Proposal 1: RAN2 to discuss whether model-based LCM and functionality-based LCM should be separately discussed (e.g. technical discussions, RAN2 part TR, conclusions).

For a specific use case, we think it is beneficial to link it to a functionality, and it can be left to UE implementation how to use the functionality. For example, if the functionality can support different levels of UE speed, the UE can just use this functionality in different scenarios without any interactions with NW side. In general, we think the granularity of functionality can be a use case, and then UE can decide to use a suitable functionality.

Proposal 2: A functionality is linked to a use case, and it can be left to UE implementation how to use the functionality.

2.4 UE capability reporting

For UE capability discussion, RAN2 made the following progress at RAN2#121b-e:

FFS if For UE capability for AIML methods we use the UE capability mechanisms as defined for RRC reported and LPP reported capabilities.

Currently, TS 38.306 and TS 38.331 have defined some UE capability reporting mechanisms/frameworks, and here are some examples:

- For a specific feature, there is a set of capabilities, e.g. csi-ReportFramework

- The combination of some capabilities which can be configured simultaneously, e.g. CA-ParametersNR

For CSI and BM cases, we think that the existing RRC reported UE capability mechanisms/frameworks as defined in TS 38.306 and 38.331 can be used a starting point.

For positioning, currently, UE reports related capability via LPP message to LMF. Likewise, it is desirable to take the exiting LPP reported capability as a starting point.

For this study AI for air interface, we think RAN1/RAN2 should firstly identify what enhancements are needed, e.g. per LCM component, or per use case. And then, we may check UE capability aspects while current frameworks can be considered. Normally, the UE capability discussions can be initiated in WI phase, and it seems not urgent in SI phase.

Proposal 3: The UE capability discussion is pending for the discussions on enhancements, e.g. per LCM per use case. Current UE capability frameworks (e.g. as defined for RRC reported and LPP reported capabilities) can be re-used.

In addition, at previous RAN2 meetings, there were some proposals about report of updates of functionality or applicability of functionality. In our understanding, if the current used functionality is not suitable, the UE may either perform functionality control by itself or may let network know. The UE reported information can be considered as additional assistance information, and thus some existing mechanism can be re-used, e.g. UAI.

Proposal 4: For report of updates of functionality or applicability of functionality, if needed, the UE reported information can be considered as additional assistance information, and existing mechanism can be re-used, e.g. UAI.

3   Conclusion

In this paper, we follow the RAN2#122 agenda, and provide technical analysis on general aspects. Our observations and proposals are listed as below:

AIML dependency

Observation 1: For additional conditions for model-based LCM and functionality-based LCM, RAN1 is still discussing the necessity, content, and other aspects.

Observation 2: For AIML dependency, more RAN1 inputs on evaluations are helpful for RAN2 discussions.

Functionality-based LCM

Proposal 1: RAN2 to discuss whether model-based LCM and functionality-based LCM should be separately discussed (e.g. technical discussions, RAN2 part TR, conclusions).

Proposal 2: A functionality is linked to a use case, and it can be left to UE implementation how to use the functionality.

UE capability reporting

Proposal 3: The UE capability discussion is pending for the discussions on enhancements, e.g. per LCM per use case. Current UE capability frameworks (e.g. as defined for RRC reported and LPP reported capabilities) can be re-used.

Proposal 4: For report of updates of functionality or applicability of functionality, if needed, the UE reported information can be considered as additional assistance information, and existing mechanism can be re-used, e.g. UAI.

4   Reference

[1] [Post122][060][AIML] Mapping of functions to physical entities (CMCC)

[2] R2-2308633, Discussion on model transfer and delivery, Huawei, HiSilicon

[3] R2-2308634, Discussion on control and LCM other, Huawei, HiSilicon