3GPP TSG-RAN WG2 Meeting #123        	          R2-2308632

Toulouse, France, 21 – 25 August, 2023

Agenda Item:	7.16.2.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on data collection

Document for:	Discussion and decision

1 Introduction

For RAN2#123, the draft agenda lists the following guidance:

7.16.2.2	Data Collection

Postpone evaluation discussion unitil RAN1 reply is received. Can continue to discussion Open issues.

Mapping of functionality to entities, for Data collection (i.e. do we use the existing data collection frameworks as is or what modifications do we expect, any aspects that is not covered that may be important?)

In this paper, we plan to discuss the following aspects for data collection:

- Mapping of functionality of entities for data collection

- Discussion on existing data collection frameworks and possible modifications

2 Discussion

  - 2.1 Mapping of functionality of entities for data collection

In the email discussion [1], the latest progress is shown as below.

For data collection part, so far training, inference and monitoring are involved, so the rest of LCM components (i.e. b) model transfer/delivery, e) control) is marked in grey.

For CSI compression with two-sided model, for training aspect, RAN1 is still discussing type 1 and type 3, and then there may be lots of options for data collection, e.g. UE/OTT server based training. More RAN1 progress will be helpful for RAN2 discussions on possible solutions.

For the listed entities for data collection, we think at least UE/OTT server/gNB can be considered. Whether to involve other entities needs more RAN2 discussions.

CSI compression with two-sided model

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training | gNB, OAM, OTT server, UE |
| b) | Model transfer/delivery | For training Type 1: gNB->UE, or OAM->gNB&UE, or OTT server->gNB&UE, or UE->gNB For training Type 3: - For UE part of two-sided model, OTT server->UE if the model is trained at OTT server, or no model transfer/delivery if the model is trained at UE; - For NW part of two-sided model, no model transfer/delivery if the model is trained at gNB, or OAM->gNB if the model is trained at OAM; |
| c) | Inference | NW part of two-sided model: gNB UE part of two-sided model: UE |
| d) | Model/functionality monitoring | NW-side: gNB monitors the performance UE-side: UE monitors the performance and reports to NW |
| e) | Model/functionality control (selection, (de)activation, switching, updating, fallback) | gNB |

Observation 1: For CSI compression with two-sided model, the training aspect is still under RAN1 discussions (e.g. training type 1 and type 3), so more RAN1 inputs are helpful for RAN2 discussions on solutions. For the listed entities for data collection, we think at least UE/OTT server/gNB can be considered. Whether to involve other entities needs more RAN2 discussions.

CSI prediction with UE-sided model

Wait for RAN1 progress.

For BM with UE-sided model, the following progress has been made in the email discussion [1].

BM with UE-sided model

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training | [FFS: gNB, OAM], OTT server, UE |
| b) | Model transfer/delivery | [FFS: gNB->UE, or OAM->UE], or OTT server->UE, or no model transfer/delivery if the model is trained at UE |
| c) | Inference | UE |
| d) | Model/functionality monitoring | UE (UE monitors the performance, and may report to gNB), gNB (gNB monitors the performance) |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB if monitoring resides at UE or gNB, UE if monitoring resides at UE |

At RAN1#111 meeting, the following agreements were made:

Agreement For the sub use case BM-Case1 and BM-Case2, at least support Alt.1 and Alt.2 for AI/ML model training and inference for further study: - Alt.1. AI/ML model training and inference at NW side - Alt.2. AI/ML model training and inference at UE side - The discussion on Alt.3 for BM-Case1 and BM-Case2 is dependent on the conclusion/agreement of Agenda item 9.2.1 of RAN1 and/or RAN2 on whether to support model transfer for UE-side AI/ML model or not - Alt.3. AI/ML model training at NW side, AI/ML model inference at UE side

For BM-Case1/Case2, RAN1 has agreed to at least support model training/inference at UE side for UE-sided model. To our understanding, the training data may be UE specific, and UE/OTT server should be suitable entities for training. For UE-sided model, if gNB or other network entities do the training, we would like to understand the extra benefits compared with UE/OTT server based approaches. For example, signalling overhead, performance differences.

For now, we observe that the following use cases are about UE-sided model:

- CSI prediction with UE-sided model

- BM with UE-sided model

- Positioning with UE-sided model

For all use cases with UE-sided model, we think RAN2 can focus on UE/OTT server based training. Whether other entities should be involved for the study should be discussed.

Proposal 1: For UE-sided model for BM and positioning, training can be done at UE or OTT server.

  - 2.2 Discussion on existing data collection frameworks

The latest table for data collection mechanisms is listed as below.

|  | Involved Network entity | RRC state to generate data | Max payload size per reporting* | Contents to be collected | End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Logged MDT | TCE/OAM (It can be utilized by gNB) | RRC_IDLE/RRRC_INACTIVE | <9kbyte | L3 cell/beam measurements, location info, sensor info, timing info | - Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency****: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message, Privacy via user consent |
| Immediate MDT | TCE/OAM (It can be utilized by gNB) | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements, location info, sensor info | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Event triggered report, Periodic reporting | AS security via RRC message, Privacy via user consent |
| L3 measurements | gNB | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - 20ms (RRC) | Event triggered report, Periodic reporting | AS security via RRC message. |
| L1 measurement (CSI reporting) | gNB | RRC_CONNECTED | <1706bit in PUCCH, <3840bit in PUSCH | L1 CSI measurement | - Procedure latency: - Report interval: - 4-320 slot for periodic report and semi-persistent report - 0-32 slot after reception of DCI for aperiodic report - Air interface signaling latency: - 1 TTI (PUCCH) | Aperiodic report, Semi-persistent report, Periodic report | No AS security |
| UAI | gNB | RRC_CONNECTED | <9kbyte | Assistance information to show UE preference | - Procedure latency: - Upon generation of UE's preference - Air interface signaling latency: - ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Early measurements | gNB | RRC_IDLE/RRC_INACTIVE | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency: - ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| LPP | LMF | RRC_CONNECTED | <9kbyte | Location info | - Procedure latency: - Latency to get upper layer trigger (for UE triggered) - Or latency to receive NW request message (~20ms) - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and LMF | UE-triggered, NW-triggered | AS security via RRC message |

For CSI and BM related use cases, we think immediate MDT is a possible candidate solution, and there are the following reasons:

- Immediate MDT can be used to transmit L3 cell/beam measurements from the UE to the NW. If RAN1 has identified L3 cell/beam measurements requirements, RAN2 may discuss possible solutions

- It can provide privacy protection via user consent

In addition, L1 measurement is a possible candidate solution, and it is up to RAN1 for the decision.

For positioning use cases, RAN2 can discuss whether to use LPP or use immediate MDT for data collection purpose. In our understanding, if the data collection requirement is between UE and gNB, immediate MDT mechanism can be considered; and if the data collection requirement is between UE and LMF, LPP mechanism can be considered.

At the last RAN2 meeting, RAN2 agreed on the LS [2] to RAN1. Assumption 3 is about RRC states. If RAN1 does not confirm the need of enhancement of the non-connected state, we can just focus on the RRC_Connected state. Otherwise, we can see more analysis from RAN1.

In general, for data collection discussions in RAN2, we suggest to wait for RAN1 feedbacks on data collection requirements. If RAN1 can identify some requirements, we think some existing frameworks can be considered for possible solutions.

Observation 2: RAN1 feedbacks on data collection requirements (per LCM component per use case) are helpful for RAN2 discussions on data collection.

Proposal 2: Existing data collection mechanisms (as identified by the previous RAN2 meeting) should be used to support potential data collection requirements.

3 Conclusion

In this paper, we provide our views on data collection. We have the following observations and proposals:

Observation 1: For CSI compression with two-sided model, more RAN1 inputs are helpful for RAN2 discussions on solutions. For the listed entities for data collection, we think at least UE/OTT server/gNB can be considered. Whether to involve other entities needs more RAN2 discussions.

Observation 2: RAN1 feedbacks on data collection requirements (per LCM component per use case) are helpful for RAN2 discussions on data collection.

Proposal 1: For UE-sided model for BM and positioning, training can be done at UE or OTT server.

Proposal 2: Existing data collection mechanisms (as identified by the previous RAN2 meeting) should be used to support potential data collection requirements.

4 References

- [Post122][060][AIML] Mapping of functions to physical entities (CMCC)

- R2-2306906, LS out on Data Collection Requirements and Assumptions, Source: RAN2, To: RAN1