3GPP TSG-RAN WG2 Meeting #123        	          R2-2308633

Toulouse, France, 21 – 25 August, 2023

Agenda Item:	7.16.2.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on model transfer and delivery

Document for:	Discussion and decision

Introduction

In the email discussion [1], the mapped entities for model transfer/delivery are also discussed. In this paper, we discuss the necessity of model transfer/delivery.

Discussion

Discussion on the necessity of model transfer/delivery

On UE-sided model

In [3], for UE-sided model for BM and Positioning, so far only UE-side OTT server and UE are listed. We think that RAN1 has agreed to at least support model training/inference at UE side for UE-sided model. To our understanding, the training data may be UE specific, and UE/OTT server should be suitable entities for training. For UE-sided model, if gNB or other network entities do the training, we would like to understand the necessity and the extra benefits compared with UE/OTT server based approaches. For example, signalling overhead, performance differences.

Proposal 1: For UE-sided model for BM and positioning, training can be done at UE or OTT server. Model transfer/delivery can be initiated by UE or OTT server, and there is no need to consider model transfer/delivery solution 1/2/3.

On Two-sided model

In [3], for CSI compression with two-sided model, the mapped entities for model transfer/delivery are listed as below.

******************************************************************************************

For CSI feedback enhancement:

Proposal 1: The Table 1 can be used as starting point for discussion on mapping of AI/ML functions to physical entities for CSI compression with two-sided model.

Table 1: The mapping of functions to physical entities for CSI compression with two-sided model

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training(offline training) | gNB, OAM, OTT server, UE, [FFS: CN] |
| b) | Model transfer/delivery | For training Type 1: gNB->UE, or OAM->gNB&UE, or OTT server->gNB&UE, or UE->gNB, [FFS: CN->gNB&UE] For training Type 3: - For UE part of two-sided model: OTT server->UE, [FFS: CN->UE]; - For NW part of two-sided model: OAM->gNB, [FFS: CN->gNB]; |
| c) | Inference | NW part of two-sided model: gNB UE part of two-sided model: UE |
| d) | Model/functionality monitoring | NW-side: NW monitors the performance UE-side: UE monitors the performance and may report to NW |
| e) | Model/functionality control (selection, (de)activation, switching, updating, fallback) | gNB, [FFS: UE] |

******************************************************************************************

For all use cases identified by RAN1, we observe that only CSI compression with two-sided may need model transfer/delivery, but it still needs more RAN1 progress.

For “OTT server->gNB&UE”, we are unclear about the feasibility of direction “OTT server->gNB”, e.g. how the procedure works, who deploys and controls OTT server. In our understanding, we rarely discussed the interaction between gNB and an entity outside 3GPP, so “OTT server->gNB” should be removed.

Observation 1: For all AI related use cases, only CSI compression with two-sided may need model transfer/delivery.

Proposal 2: For OTT server to gNB for CSI compression with two-sided model, we suggest to remove it as the feasibility is unclear.

For [FFS: CN], we are unclear about the necessity of involving CN. CSI compression with two-sided model is mainly about PHY layer, and it is natural to involve UE and RAN. Usually CN is not involved with PHY layer features, so we think the necessity of involving CN is unclear.

Proposal 3: For CSI compression with two-sided model, the necessity of involving CN can be discussed.

Analyses for Solution 1a and 1b for CSI compression with two-sided model

Based on the above observation 1, we think RAN2 can have some technical discussion on model transfer/delivery solution 1a/1b, i.e. gNB transfers/delivers models to UE.

In solution 1a, the models are transferred/delivered via RRC signalling. In general, comparing to DRBs, SRBs have higher priorities and can transfer model with less latency comparing to other candidate solutions. However, if the data amount of the transferred model is much larger than traditional RRC signalling, e.g. ten to hundreds MB, there will be some challenges for solution 1a.

According to TS 38.331, for each DL RRC message, it can at most be divided into 5 segments and each segment supports at most 9kB data amount, due to the PDCP SDU limitation. Thus, within one RRC message, at most 45kB model data can be included. Meanwhile, the data size of a complete AI model is hard to specialized and can vary from several to tens of MBs. For example, in the table of CSI prediction with generalization, the model size may be near 30MB, which means there should be more than 3k RRC segments.

In this term, solution 1a may bring extra workload in RAN2.

Observation 2: Based on the current specification, solution 1a can only transfer models with size less than 45KB.

A straightforward solution would be extending the number limitation of the segments while this will introduce another issue. According to TS 38.300, quoted as the follows, even the model is transferred via segments without segment number limitation, the RRC message segments should be transferred continuously. Therefore, if an RRC message containing AI models with a large number of segments is transferred on the air-interface, the other conventional RRC messages, e.g. RRC reconfigurations for UE mobility, will be delayed, and thus causing severe problems like RLF.

## 7.10 Segmentation of RRC messages An RRC message may be segmented in case the size of the encoded RRC message PDU exceeds the maximum PDCP SDU size. Segmentation is performed in the RRC layer using a separate RRC PDU to carry each segment. The receiver reassembles the segments to form the complete RRC message. All segments of an RRC message are transmitted before sending another RRC message. Segmentation is supported in both uplink and downlink as specified in TS 38.331 [12].

Besides, if the model is transferred via SRB1/SRB2, the data transmission via DRBs will be delayed for a long time until the model is completely transferred. And if the model is transferred via SRBs with a low priority (like SRB4), it will minimize the impacts on data transmission via DRBs, but the model transfer latency may be large.

If SRB4 is to be considered for transmitting model related information, the network still needs to understand the data transmission requirements (e.g. QoS characteristics), which puts extra complexities.

Observation 3: For solution 1a, the feasibility of new SRB is unclear. Solution 1a may introduce extra delay for DRB data transmission or may result in large model transfer latency.

Then for the mobility scenario, according to the current specification, once the UE has HO to the target cell before the model transfer with the source cell is finished, the UE will delete the RRC signalling received from the source cell, including the received segments of models, and then starts the model transfer in the target cell from the beginning. Then, this may slow down the transfer progress and cannot meet the demands of the time sensitive case. This predicament also happens if when RLF occurs.

Observation 4: For mobility and RLF case, solution 1a may not meet the demands of the time sensitive case.

In summary, we have the following proposals:

Proposal 4: For CP-based solution 1a, how it work in case of large model size is unclear, so the feasibility should be discussed.

Proposal 5: The new SRB option may bring some issues, and RAN2 should evaluate them.

In solution 1b, the gNB can transfer/deliver AI models to UE via UP data. Naturally, solution 1b can support large-size model delivery. To meet different model delivery latency demands, diverse QoS parameters can be applied, how to deal with the UP transmission of the models and legacy UP data is still unclear. Overall, solution 1b can have the potential capacity to deliver large-size models while meet the latency demands. Currently, for solution 1b, there are the following possibilities:

- (a) A new UP terminated at gNB. It may mean gNB can transfer/deliver AI/ML model(s) to UE via data radio bearer.

- (b) gNB first transfers/delivers AI/ML models to CN, and then CN transfers/delivers the models to UE via UP.

- (c) A new AI layer may be needed, and the motivation is FFS. Some companies think that introduction of a new layer is out of the SI scope.

- (d) Some companies think that the application function (AF) hosting the AI/M models and UPF can be collocated with the RAN, and the protocol stack can be implemented at any network entity. While some companies think such case seems identical as Option 2 and Option 4, and thus it is better to clarify and probably discuss them separately.

Currently the DRB setup procedure is part of PDU Session Establishment, which can be firstly initiated between UE and AMF. For (a), it seems that gNB can be allowed to directly initiate the DRB setup procedure. If so, we think RAN2 may need to discuss more details about it, e.g. how gNB decides on the QoS related information.

For (b), one benefit is that air interface may not be impacted. For this option, the overall latency will be larger, and it may raise the leakage of model privacy between vendors and operations.

For (c), it is FFS whether this option is out of the SI scope or not. For the new AI layer, if it is considered as UP layer, we have the same comments as for (a); if it is considered as CP layer, what are the differences between this option and solution 1a (and also pros/cons) needs to be discussed.

For (d), whether and how such deloyments can be achieved are unclear, and they should be clarified.

In summary, there are some possibilities proposed by companies, and there are some ambiguities to be clarified. We think the feasibility should be discussed.

Observation 5: How solution 1b works is unclear.

Proposal 6: For UP-based solution 1b, the feasibility should be discussed, e.g. how it works and how it may impact different network entities.

Conclusion

In this paper, we provide our views on the necessity of model transfer/delivery. We have the following proposals:

Proposal 1: For UE-sided model for BM and positioning, training can be done at UE or OTT server. Model transfer/delivery can be initiated by UE or OTT server, and there is no need to consider model transfer/delivery solution 1/2/3.

Proposal 2: For OTT server to gNB for CSI compression with two-sided model, we suggest to remove it as the feasibility is unclear.

Proposal 3: For CSI compression with two-sided model, the necessity of involving CN can be discussed.

For all use cases identified by RAN1, we observe that only CSI compression with two-sided may need model transfer/delivery, but it still needs RAN1 progress. We have the following observations and proposals:

Observation 1: For all AI related use cases, only CSI compression with two-sided may need model transfer/delivery.

Observation 2: Based on the current specification, solution 1a can only transfer models with size less than 45KB.

Observation 3: For solution 1a, the feasibility of new SRB is unclear. Solution 1a may introduce extra delay for DRB data transmission or may result in large model transfer latency.

Observation 4: For mobility and RLF case, solution 1a may not meet the demands of the time sensitive case.

Observation 5: How solution 1b works is unclear.

Proposal 4: For CP-based solution 1a, how it work in case of large model size is unclear, so the feasibility should be discussed.

Proposal 5: The new SRB option may bring some issues, and RAN2 should evaluate them.

Proposal 6: For UP-based solution 1b, the feasibility should be discussed, e.g. how it works and how it may impact different network entities.

References

- [Post122][060][AIML] Mapping of functions to physical entities (CMCC)

- R2_121 Chair Notes EOM.

- R2-2302268, Report of Offline 027 model transfer delivery, Huawei.

- RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm