3GPP TSG-RAN WG2 Meeting #123          	          R2-2308634

Toulouse, France, 21 – 25 August, 2023

Agenda Item:	7.16.2.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on control and LCM other

Document for:	Discussion and decision

Introduction

In the RAN2#123 agenda, there are some guidance for Control and LCM other:

Expect to continue evaluation for cases of methods, ambition level, mapping of functionality to entities. Determine tangible open issues if any.

In this paper, we firstly list the latest progress of control (for model-based and functionality-based) in RAN1 and RAN2, and then we list open issues for discussions.

Discussion

Discussion related to [Post122][060]

For UE-sided model for BM, P2 is made:

********************************************************

Proposal 2: The Table 2 can be used as starting point for discussion on mapping of AI/ML functions to physical entities for beam management with UE-side model.

Table 2: The mapping of AI/ML functions to physical entities for beam management with UE-side model

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training(offline training) | UE-side OTT server, UE, [FFS: gNB, OAM, CN] |
| b) | Model transfer/delivery | UE-side OTT server->UE, [FFS: gNB->UE, or OAM->UE, or CN->UE] |
| c) | Inference | UE |
| d) | Model/functionality monitoring | UE (UE monitors the performance, and may report to gNB), gNB (gNB monitors the performance) |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB if monitoring resides at UE or gNB, UE if monitoring resides at UE |

********************************************************

For e), we think that “gNB if monitoring resides at UE” can be discussed. If gNB relies on UE reported information in order to perform monitoring, it means the model control at gNB side is dependent on UE reported information. For UE reported information, we think RAN1 is discussing the requirements (e.g. necessity, content), so more RAN1 inputs are helpful for RAN2 discussions.

Observation 1: Model/functionality control at gNB side is dependent on UE reported information. For the UE reported information, more RAN1 inputs are helpful for RAN2 discussions.

Our suggestion is to change the wording into:

For e), gNB if monitoring resides at gNB, if resides at UE and reported (if needed) to gNB.

Proposal 1: For UE-sided model for BM, for model/functionality control, the mapped entities are:

gNB if monitoring resides at gNB, if resides at UE and reported (if needed) to gNB, UE if monitoring resides at UE.

For UE-sided model for Positioning, P4 is made:

********************************************************

For Positioning accuracy enhancement:

Proposal 4: The Table 4 can be used as starting point for discussion on mapping of AI/ML functions to physical entities for positioning with UE-side model (case 1 and 2a).

Table 4: The mapping of functions to physical entities for positioning with UE-side model (case 1 and 2a)

| Use case | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training (offline training) | UE-side OTT server, UE, [FFS: LMF, OAM, CN] |
| b) | Model transfer/delivery | UE-side OTT server->UE, [FFS: LMF->UE, OAM->UE, CN->UE] |
| c) | Inference | UE |
| d) | Model/functionality monitoring | UE, LMF |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | UE if monitoring resides at UE, LMF if monitoring resides at UE or LMF |

********************************************************

For e), we think that “LMF if monitoring resides at UE” can be removed, because there is no UE reported information listed in d).

Proposal 2: For UE-sided model for Positioning, for model/functionality control, the mapped entities are:

UE if monitoring resides at UE, LMF if monitoring resides at LMF.

Discussion related to functionality control

The RAN2#121b-e meeting has made some agreements as below, and model and functionality are mentioned together.

For the CSI compression and beam management use cases, model/function selection/(de)activation/switching/fallback can be UE-initiated or gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model).

For the positioning use case, model/function selection/(de)activation/switching/fallback can be UE-initiated or LMF-/ gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model).

For functionality selection and switching, we are unclear about the necessity. In our understanding, for a specific use case, there is a corresponding functionality, and then it is sufficient to consider functionality activation/deactivation/fallback. If there are no use cases for supporting functionality selection/switching, it is not suggested to just use the same terminologies for both model/functionality control.

Proposal 3: Functionality selection/switching can be removed since the use cases are unclear.

For UE-side monitoring for functionality-based LCM, we understand that the UE can monitor the performance, and the performance requirements can be left to RAN4 discussions. In this case, UE can perform functionality selection/(de)activation/switching/fallback based on the defined requirements.

Proposal 4: UE makes functionality (de)activation/fallback based on the defined requirements (FFS to be defined by RAN4).

Conclusion

In this paper, we mainly discuss some issues related to [Post122][060] and functionality control. We have the following observations and proposals:

Observation 1: Model/functionality control at gNB side is dependent on UE reported information. For the UE reported information, more RAN1 inputs are helpful for RAN2 discussions.

Proposal 1: For UE-sided model for BM, for model/functionality control, the mapped entities are:

gNB if monitoring resides at gNB, if resides at UE and reported (if needed) to gNB, UE if monitoring resides at UE.

Proposal 2: For UE-sided model for Positioning, for model/functionality control, the mapped entities are:

UE if monitoring resides at UE, LMF if monitoring resides at LMF.

Proposal 3: Functionality selection/switching can be removed since the use cases are unclear.

Proposal 4: UE makes functionality (de)activation/fallback based on the defined requirements (FFS to be defined by RAN4).

References

- [Post122][060][AIML] Mapping of functions to physical entities (CMCC)