3GPP TSG-RAN WG2 Meeting #123	R2-2308637

Toulouse, France, 21 - 25 August, 2023

Agenda Item:	7.25.1

Source:	Huawei, HiSilicon

Title:	Discussion on MAC-CE based indication for cross-RRH TCI state switch

Document for:	Discussion and decision

1. Introduction

NR_HST_FR2_enh (enhanced NR support for high speed train scenario in FR2) is a RAN4-led Rel-18 topic [1], and one of the objectives with RAN2 impact is the following:

- Specify UL timing adjustment solution, including explicit NW signalling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE [RAN4, RAN2].

RAN4 made a number of agreements and sent an LS to RAN2 [2]. In the LS, RAN2 is asked to introduce 1bit indication in MAC-CE to inform UE on the TCI state switch cross non-collocated RRHs. In RAN2 #122 meeting, RAN2 confirmed that it seems feasible to specify a new MAC CE. Since there are still issues that need further clarification from RAN4, a reply LS [3] was sent to ask further questions. Before receiving the reply, in this contribution, we discuss the potential RAN2 impacts based on the current information.

2. Discussion

2.1 Impacts to MAC layer

In NR, there are multiple TCI state indication/activation MAC CEs related to PDCCH reception, including:

- TCI State Indication for UE-specific PDCCH MAC CE (Clause 6.1.3.15 in TS 38.321),

- Enhanced TCI States Indication for UE-specific PDCCH MAC CE (Clause 6.1.3.44 in TS 38.321),

- Unified TCI States Activation/Deactivation MAC CE (Clause 6.1.3.47 in TS 38.321).

In the LS [3], RAN2 asked to which MAC CE(s) the cross non-collocated RRHs indication is applicable. At this stage, we believe that at least the MAC CE in Clause 6.1.3.15 in TS 38.321 is a target one, because it has the basic function of TCI state switching from one RRH to another RRH. As for the MAC CEs in Clause 6.1.3.44 and Clause 6.1.3.47 in TS 38.321, we can wait for the reply from RAN4 to finalize the design. Regarding the TCI State Indication for UE-specific PDCCH MAC CE (Clause 6.1.3.15 in TS 38.321)

RAN2 focus on the discussion on TCI State Indication for UE-specific PDCCH MAC CE (Clause 6.1.3.15 in TS 38.321).

We provide two alternatives for cross-RRH TCI state switch indication.

- Alternative 1 is that we use a new LCID in Table 6.2.1-1 for cross-RRH TCI state switch indication and modify the MAC CE description of Clause 6.1.3.15 to include the cross-RRH TCI state switch indication case. One benefit of using LCID is that the MAC sub header length is one byte shorter, so the UE may get this MAC CE more reliable when the channel quality is not that good. However, the number of reserved bits is limited, so it may be not very friendly for future features which need the shorter length header with LCID.

- Alternative 2 is that we re-use the format of the existing TCI State Indication for UE-specific PDCCH MAC CE, but it will be identified by a MAC sub header with eLCID (see the Table 6.2.1-1b Values of one-octet eLCID for DL-SCH). In this case, the number of reserved bits will not be a problem. Therefore, we prefer Alternative 2 for this MAC CE design, and it is friendly if RAN4 replies there are more than one TCI state switch MAC CE needs the cross-RRH indication.

RAN2 to agree on re-using the existing structure of TCI State Indication for UE-specific PDCCH MAC CE, and discuss how it is identified. Prefer the following Alt2 in the MAC CE design.

- Alt1: The MAC CE is identified by a MAC sub header with LCID. In Table 6.2.1-1 Values of LCID for DL-SCH, use a reserved LCID value for cross-RRH TCI state switch indication.

- Alt2: The MAC CE is identified by a MAC sub header with eLCID. In Table 6.2.1-1b Values of one-octet eLCID for DL-SCH, use a reserved one-octet eLCID for cross-RRH TCI state switch indication.

- In the LS [3], there is a question to RAN4 about the UE behaviour upon reception of the MAC CE: “Is it possible to update timing advance upon reception of the MAC CE with indication on the TCI state switch across RRHs and if not should UE stop uplink transmissions?” Based on our understanding, the UE will do one-shot uplink transmit timing adjustment (if needed) upon reception of the MAC CE. With this uplink timing adjustment, the uplink transmission should be good to be received by the gNB, and the UE does not stop uplink transmission.

- Question 3: What is the intended UE behavior (e.g. regarding timing advance handling) upon reception of the MAC CE with indication on the TCI state switch across RRHs? For example, Does UE behavior also depend on the existing RRC parameter highSpeedDeploymentTypeFR2? Is it possible to update timing advance upon reception of the MAC CE with indication on the TCI state switch across RRHs and if not should UE stop uplink transmissions?

The UE does not stop uplink transmission after receiving the cross-RRH TCI state switch indication MAC CE.

2.2 Impacts to RRC layer

In Rel-17, the RRC field highSpeedLargeOneStepUL-TimingFR2-r17 was introduced, and it can be used to enable or disable the one-shot uplink timing adjustment feature. When it is enabled, the UE evaluates the DL timing difference between two RRHs/TCI states, and then if the difference is larger than the threshold, the UE does the one-shot uplink timing adjustment.

According to the RAN4 agreement as extracted below, we observe that RAN4 has agreed to re-use the Rel-17 RRC signalling highSpeedLargeOneStepUL-TimingFR2-r17 to enable or disable the Rel-18 cross-RRH indication feature.

R4-2310042, WF on FR2 HST RRM requirements (part 2) Issue 1-1-2: Conditions to apply one-shot large timing adjustment Agreement: - The condition for UE to apply one-shot large uplink timing adjustment at TCI state switch: - UE capability to support the cross-RRH TCI state switching with MAC-CE based network signaling assistance - NW flag signaling highSpeedMeasFlagFR2-r17 to indicate UE be in the HST scenario - Reuse Rel-17 large one step UL timing adjustment signalling (highSpeedLargeOneStepUL-TimingFR2-r17 NW signalling and [largeOneStepUL-timingFR2-r17] UE capability)

However, for the UE supporting both Rel-17 one-shot UL timing adjustment feature and Rel-18 UL timing adjustment (with the cross-RRH indication), if we just reuse the Rel-17 RRC configuration signalling, the UE may not know whether to enable Rel-18 feature or not. The same problem also applies to gNB side.

In order to solve the issue, a new RRC field for enabling the Rel-18 uplink timing adjustment feature can be considered.

Observation 1: If the UE supports both Rel-17 one-shot UL timing adjustment feature and Rel-18 UL timing adjustment with the cross-RRH indication feature, reusing the Rel-17 configuration signalling is not sufficient.

Introduce a new RRC field for enabling the Rel-18 cross-RRH TCI state switch indication.

2.3 Impacts to UE capability aspect

- In Rel-17, the UE capability parameter ue-OneShotUL-TimingAdj-r17 (the name is captured as [largeOneStepUL-timingFR2-r17] in TS 38.133) was introduced to indicate whether the UE supports this Rel-17 feature. According to the RAN4 agreement below, RAN4 agrees to re-use this UE capability.

- However, as described in Section 2.2, the gNB needs to distinguish which uplink timing adjustment mechanism that the UE supports. Then the gNB can correspondingly configure a RRC parameter to enable one UL timing adjustment method. We observe that reusing the Rel-17 UE capability is not sufficient, therefore, a new Rel-18 UE capability parameter is necessary.

R4-2310042, WF on FR2 HST RRM requirements (part 2) Issue 1-1-2: Conditions to apply one-shot large timing adjustment Agreement: - The condition for UE to apply one-shot large uplink timing adjustment at TCI state switch: - UE capability to support the cross-RRH TCI state switching with MAC-CE based network signaling assistance - NW flag signaling highSpeedMeasFlagFR2-r17 to indicate UE be in the HST scenario - Reuse Rel-17 large one step UL timing adjustment signalling (highSpeedLargeOneStepUL-TimingFR2-r17 NW signalling and [largeOneStepUL-timingFR2-r17] UE capability)

Introduce a new UE capability parameter for Rel-18 cross-RRH TCI state switch indication.

3. Conclusion

Based on the discussion in this contribution, we have the following observation and proposals:

Observation 1: If the UE supports both Rel-17 one-shot UL timing adjustment feature and Rel-18 UL timing adjustment with the cross-RRH indication feature, reusing the Rel-17 configuration signalling is not sufficient.

- RAN2 focus on the discussion on TCI State Indication for UE-specific PDCCH MAC CE (Clause 6.1.3.15 in TS 38.321).

- RAN2 to agree on re-using the existing structure of TCI State Indication for UE-specific PDCCH MAC CE, and discuss how it is identified. Prefer the following Alt2 in the MAC CE design.

- Alt1: The MAC CE is identified by a MAC sub header with LCID. In Table 6.2.1-1 Values of LCID for DL-SCH, use a reserved LCID value for cross-RRH TCI state switch indication.

- Alt2: The MAC CE is identified by a MAC sub header with eLCID. In Table 6.2.1-1b Values of one-octet eLCID for DL-SCH, use a reserved one-octet eLCID for cross-RRH TCI state switch indication.

The UE does not stop uplink transmission after receiving the cross-RRH TCI state switch indication MAC CE.

Introduce a new RRC field for enabling the Rel-18 cross-RRH TCI state switch indication.

Introduce a new UE capability parameter for Rel-18 cross-RRH TCI state switch indication.

- Draft CRs for the above proposals are provided in [4], [5] and [6].

4. Reference

[1] RP-220985, New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)

[2] R4-2306399, LS on MAC-CE Based Indication for Cross-RRH TCI State Switch

[3] R2-2306865, Reply LS on MAC-CE Based Indication for Cross-RRH TCI State Switch

[4] R2-2308638, Draft CR to 38.321 for HST-FR2, Huawei, HiSilicon

[5] R2-2308639, Draft CR to 38.331 for HST-FR2, Huawei, HiSilicon

[6] R2-2308640, Draft CR to 38.306 for HST-FR2, Huawei, HiSilicon