3GPP TSG-RAN WG2 Meeting #123	R2-2308648

Toulouse, France, August 21-25, 2023

Agenda Item:	6.1.3.2

Source:	Huawei, HiSilicon

Title:	Discussion on newly introduced FR2 CA BW Classes

Document for: Discussion and Decision

Introduction

RAN4 sent LS R4-2119966 [1] (RAN4#101 meeting, Nov 2021) to RAN2 on release independence aspects of newly introduced FR2 CA BW Classes and CBM/IBM UE capability “both”:

RAN4 has intorduced new FR2 FBG2 CA BW classes R, S, T, U into 38.101-2 [1], see below. Table 5.3A.4-1: CA bandwidth classes NR CA bandwidth class; Aggregated channel bandwidth; Number of contiguous CC; Fallback group. D; 200 MHz < BWChannel_CA ≤ 400 MHz; 2; 2. E; 400 MHz < BWChannel_CA ≤ 600 MHz; 3. F; 600 MHz < BWChannel_CA ≤ 800 MHz; 4. R; 800 MHz < BWChannel_CA ≤ 1000 MHz; 5. S; 1000 MHz < BWChannel_CA ≤ 1200 MHz; 6. T; 1200 MHz < BWChannel_CA ≤ 1400 MHz; 7. U; 1400 MHz < BWChannel_CA ≤ 1600 MHz; 8. Furthermore RAN4 has agreed to define these new FR2 CA FBG2 BW classes R, S, T, U as release independent from REL-15 pending confirmation from RAN2 that this is possible [2]. Second topic RAN4 has decided that signaling is needed for the case that UE supports both independent beam management (IBM) and common beam management (CBM). In RE16 signaling UE can just indicate either IBM or CBM altough in REL16 only IBM is possible as CBM requrements are not yet defined. 2. Actions: To RAN2 RAN4 respectfully asks RAN2 to respond if it is possible to define new FR2 CA BW classes R, S, T and U as release independent from release 15. Also RAN4 asks RAN2 to develop signalling for a UE to be able to indicate that it supports both IBM and CBM and respond if it is possible to define this capability as release independent from release 16.

However, still now RAN2 didn’t develop the signaling and didn’t send reply LS to RAN4. In this contribution, we discuss how to process this issue in RAN2.

Discussion

The background and RAN2 discussion history are listed below:

- RAN4 sent LS R4-2119966 to ask RAN2 to define new FR2 CA BW classes R, S, T and U.

- In RAN2#119 meeting (August, 2022), RAN2 discussed this issue and had a baseline CRs in R2-2207974 [2] and R2-2207975 [3]. The CR was postponed because RAN4 were still discussing whether the value R, S, T, U will be discarded.

FR2 BW Class [AT119-e][023][NR17] FR2 BW classes (Nokia) Scope: Treat R2-2208510, R2-2208511, R2-2207974, R2-2207975, R2-2207973, Determine agreeable parts. For the agreeable parts, agree CRs. Intended outcome: Report, Agreed CRs (if any), LS out (if applicable) Deadline: Schedule 1 R2-2209082 Report of [AT119-e][023][NR17] FR2 BW classes (Nokia) Nokia, Nokia Shanghai Bell [023] Noted [023] RAN2 agrees there is no backward compatibility issue for network in introducing new FR2 FBG5 BW classes in the CA-BandwidthClassNR field (proposed in CR R2-2208511). [023] RAN2 waits for further progress of RAN4 on FR2 BW classes R, S, T, U (i.e., if they will be deleted from FBG2) before deciding to discard the signalling agreed in the baseline CRs in R2-2207974 and R2-2207975 R2-2208510 Discussion on FR2 new bandwidth classes Huawei, HiSilicon discussion Rel-17 NR_RF_FR2_req_enh2-Core [023] Noted R2-2208511 Introduction of FR2 FBG5 CA BW classes Huawei, HiSilicon CR Rel-17 38.331 17.1.0 3432 - B NR_RF_FR2_req_enh2-Core R2-2207974 Introduction of FR2 FBG2 CA BW classes Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Ericsson, ZTE Corporation, Sanechips, Qualcomm, Xiaomi Communications CR Rel-17 38.331 17.1.0 2867 4 B NR_RF_FR2_req_enh2-Core R2-2204851 Moved from 6.0.2 R2-2207975 Introduction of FR2 FBG2 CA BW classes Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Ericsson, ZTE Corporation, Sanechips, Qualcomm, Xiaomi Communications CR Rel-17 38.306 17.1.0 0678 3 B NR_RF_FR2_req_enh2-Core R2-2204850 Moved from 6.0.2 [023] All CRs are postponed R2-2207973 Reply LS on release independence aspects of newly introduced FR2 CA BW Classes and CBM/IBM UE capability Nokia, Nokia Shanghai Bell LS out Rel-17 NR_RF_FR2_req_enh2-Core R2-2204854 To:RAN4 Moved from 6.0.2 [023] Noted, not needed

- In RAN2#119-bis meeting (October, 2022), the baseline CRs were resubmitted, the RAN2 conclusion is still wait for RAN4.

FBG2 BW Classes Wait for RAN4 R2-2210243 Introduction of FR2 FBG2 CA BW classes Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Ericsson, ZTE Corporation, Sanechips, Qualcomm, Xiaomi Communications CR Rel-17 38.331 17.2.0 2867 5 B NR_RF_FR2_req_enh2-Core R2-2207974 R2-2210245 Introduction of FR2 FBG2 CA BW classes Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Ericsson, ZTE Corporation, Sanechips, Qualcomm, Xiaomi Communications CR Rel-17 38.306 17.2.0 0678 4 B NR_RF_FR2_req_enh2-Core R2-2207975 R2-2210241 Reply LS on release independence aspects of newly introduced FR2 CA BW Classes and CBM/IBM UE capability Nokia, Nokia Shanghai Bell LS out Rel-17 NR_RF_FR2_req_enh2-Core R2-2207973 To:RAN4

- In RAN4#105 meeting (November, 2022) [4], RAN4 further discussed this issue, the conclusion was: not pursue the removal of value R, S, T, U.

R4-2218823 Removal of the CA bandwidth classes R-U Type: CR For: Agreement 38.101-2 v17.7.0 CR-0515 rev Cat: F (Rel-17) Source: Ericsson Abstract: Draft CR to remove the CA BW classes R-U superseded by R5-R8. Verizon: we support this proposal. R2~R12. We want companies to consider to remove these. ZTE: there is no consensus. We can come back next meeting. Samsung: we share the similar view as Verizon. We do not think there is any technical issue to block removing these. Xiaomi: We agree with ZTE. Decision: Not pursued.

- RAN2 didn't open this discussion again, the baseline CRs were not agreed in RAN2 and the reply LS was not sent to RAN4.

Observation 1: RAN4 sent LS R4-2119966 to ask RAN2 to define new FR2 CA BW classes R, S, T and U, but still now no signalling was introduced in RAN2 and no reply LS was sent to RAN4.

Currently, the value R, S, T, U are captured in TS 38.101-2, but no UE capability signaling in TS 38.331. The RAN4 specification and RAN2 specification are not aligned, this issue should be fixed.

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| V (Note 4) | 1200 MHz < BWChannel_CA ≤ 1600 MHz | 4 |  |
| W (Note 4) | 1600 MHz < BWChannel_CA ≤ 2000 MHz | 5 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| R | 800 MHz < BWChannel_CA ≤ 1000 MHz | 5 |  |
| S | 1000 MHz < BWChannel_CA ≤ 1200 MHz | 6 |  |
| T | 1200 MHz < BWChannel_CA ≤ 1400 MHz | 7 |  |
| U | 1400 MHz < BWChannel_CA ≤ 1600 MHz | 8 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 |  |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| R2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| R3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| R4 | 400 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| R5 | 500 MHz ≤ BWChannel_CA ≤ 1000 MHz | 5 |  |
| R6 | 600 MHz ≤ BWChannel_CA ≤ 1200 MHz | 6 |  |
| R7 | 700 MHz ≤ BWChannel_CA ≤ 1400 MHz | 7 |  |
| R8 | 800 MHz ≤ BWChannel_CA ≤ 1600 MHz | 8 |  |
| R9 | 900 MHz ≤ BWChannel_CA ≤ 1800 MHz | 9 |  |
| R10 | 1000 MHz ≤ BWChannel_CA ≤ 2000 MHz | 10 |  |
| R11 | 1100 MHz ≤ BWChannel_CA ≤ 2200 MHz | 11 |  |
| R12 | 1200 MHz ≤ BWChannel_CA ≤ 2400 MHz | 12 |  |

Observation 2: The RAN4 and RAN2 specification are not aligned, the value R, S, T, U are captured in TS 38.101-2, but no UE capability signaling in TS 38.331.

In our view, the RAN4 conclusion is clear, the value R, S, T, U are introduced, so RAN2 should define the UE capability signalling as requested by RAN4 LS. However, it is safer to double check with RAN4. Thus, it is suggested to send reply LS to RAN4: 1) inform RAN4 that RAN2 has discussed the signalling for introduction of FR2 BW classes R, S, T, U; 2) ask RAN4 whether the RAN4 will discard the FR2 BW classes R, S, T, U or not.

Proposal 1: RAN2 to send reply LS to RAN4:

1) inform RAN4 that RAN2 has discussed the signalling for introduction of FR2 BW classes R, S, T, U;

2) ask RAN4 whether the RAN4 will discard the FR2 BW classes R, S, T, U or not.

The content of draft reply LS is provided as below.

1. Overall Description: RAN2 thanks RAN4 for the LS in R4-2119966 and for new FR2 FBG2 CA BW classes R, S, T, U: 1) RAN2 would answer the questions in the LS as follows: RAN2 confirms that the above bandwidth classes have been considered to be introduced starting Rel-17 but allowing earlier implementation from Rel-15. (The baseline CRs are R2-2207974 and R2-2207975.) 2) RAN2 would like to know whether RAN4 will discard the FR2 BW classes R, S, T, U or not, so that RAN2 can decide whether to define new signaling for FR2 BW classes R, S, T, U. 2. Actions: To RAN4 RAN2 respectfully asks RAN4 to take the above answer into account, and to respond whether RAN4 will discard the FR2 BW classes R, S, T, U or not.

Conclusion

In this contribution, we discussed the RAN4 LS on FR2 CA BW Classes.

Observation 1: RAN4 sent LS R4-2119966 to ask RAN2 to define new FR2 CA BW classes R, S, T and U, but still now no signalling was introduced in RAN2 and no reply LS was sent to RAN4.

Observation 2: The RAN4 and RAN2 specification are not aligned, the value R, S, T, U are captured in TS 38.101-2, but no UE capability signaling in TS 38.331.

Proposal 1: RAN2 to send reply LS to RAN4:

1) inform RAN4 that RAN2 has discussed the signalling for introduction of FR2 BW classes R, S, T, U;

2) ask RAN4 whether the RAN4 will discard the FR2 BW classes R, S, T, U or not.

References

- R4-2119966, LS on release independence aspects of newly introduced FR2 CA BW Classes and CBM/IBM UE capability “both”, RAN4.

- R2-2207974, Introduction of FR2 FBG2 CA BW classes, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Ericsson, ZTE Corporation, Sanechips, Qualcomm, Xiaomi Communications	.

- R2-2207975,	Introduction of FR2 FBG2 CA BW classes, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Ericsson, ZTE Corporation, Sanechips, Qualcomm, Xiaomi Communications

- RAN4_105_Main_Session_report_v08_Friday-EOM.