3GPP TSG-RAN WG2 Meeting #123	R2-2308651

Toulouse, France, August 21st- 25th 2023

Agenda item:	7.8.2

Source:	China Telecom

Title:	Discussion on Measurement Reporting for NR UAV

Document for:	Discussion and Decision

- Introduction

The WID on NR Support for UAV [1] was approved in Rel-18. One of the objectives is to specify the enhancements on measurement reports of UAV. In the previous meetings, the following agreements were achieved [2]:

Agreements from RAN2#122

- Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

- As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy behaviour applies.

- For UE behavior on L1 and L3 measurement, it is left to UE implementation whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value

- New event types will be introduced on the combination of event Ax and event Hx, at least for event A4 + event H1/H2. FFS for other event Ax + event H1/H2. FFS on details, e.g. whether to include one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event, how to configure height-dependent numberOfTriggeringCells, etc.    This will be applied to all height dependent MR parameters.

- Whether UE height is included when UAV specific MR is triggered is configurable by the network.

- We will use LTE UEheight.

Agreements from RAN2#121bis-e:

- Height-dependent more-than-one configurations is supported on parameter/field level (i.e. different fields/values within the same MO) where different values (or value ranges) of the parameter/field applies to different height or height range.

- For MO configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: SSB- ToMeasure. Details on how to specify is FFS.    FFS on UE behavior on L1 and L3 measurement.

- For MR configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: Event A4 threshold and numberoftriggeringcells.  Details on how to specify is FFS (i.e. maybe it can be achieved by combination of events).

- When height-dependent more-than-one configurations are provided, UE applies the new value once it moves to new height (or height range) similar to the case of RRC reconfiguration. Need Codes, field descriptions, etc. as in legacy specifications apply

- If a height-specific value is not explicitly configured for certain height, whether to keep using the value that was used or consider the parameter as released (i.e. parameter/value not applicable at this height) should be looked into case by case, and can be clarified by need code, field description, or procedural text as needed.   FFS details

The intention of this paper is to further discuss the remaining issues and related details of measurement reporting enhancement.

- Discussion

2.1	On Height-Dependent SSB-ToMeasure

- The number of height ranges

In RAN2#122 meeting, RAN2 agreed to add the height-based list of SSB-ToMeasure with corresponding height regions and hysteresis in MeasObjectNR. Furthermore, the number of height ranges is still FFS. SSB-ToMeasure is used to indicate the set of SS blocks to be measured within the SMTC measurement duration. In RAN2#121bis meeting, it was agreed to use height-dependent SSB-ToMeasure, with the aim of saving UE power consumption and reducing the measurement on beams as well as the size of the measurement report. When UAV UEs fly above a height threshold, it is assumed more cells will be detected. In such a radio environment, UAV UE configured with height-dependent SSB-ToMeasure values can exclude the downtilted beam in SSB measurement.

In our view, two height ranges seem enough to fulfill the above requirements (One is above the height threshold, and another is below the height threshold). Configuring too many height ranges may lead to unnecessary switching of SSB-ToMeasure values and frequent UE measurements, which is against the purpose of introducing height-dependent SSB-ToMeasure in RAN2#121bis..  For future-proof and greater flexibility in network optimization, we are also fine with the number of height ranges extended to 4 or 8. Furthermore, the number of height ranges should be also configured by the network in advance as the radio environment varies from cell to cell.

Proposal 1: The number of height ranges could be configured by the network with 2 or 4 or 8. and height threshold(s) can be configurable by the network.

- How to specify height-dependent SSB-ToMeasure

With regards to how to specify height-dependent SSB-ToMeasure, we believe that the ToAddModList/ToRemvoeList structure could be efficiently used to support height-dependent SSB-ToMeasure. Through this approach, the height-specific SSB-ToMeasure links with the related height region by one entry. In addition, the ToAddModList/ToRemvoeList structure is also beneficial to support future extensions.

Proposal 2:  agree to configure the height-based list of SSB-ToMeasure with ToAddModList/ToRemoveList structure.

- Height regions overlapping

To avoid confusion on the UE side, it is necessary to ensure no overlapping height regions. According to the agreements in previous RAN2 meetings, the network can configure multiple SSB-ToMeasure values for height regions. UAV determines SSB-ToMeasure configuration based on its height. In practical deployment, this issue can be implemented by the network configuring proper height threshold(s)

Proposal 3: No further enhancement will be introduced in UAV WID to avoid overlapping height regions.

2.2	On Height-Dependent Measurement Report Configuration

- Whether to configure different numberoftriggeringcells values corresponding to different height ranges

As the UAV UE flight altitude increases, usually more cells are detected. The purpose of numberOfTriggeringCells is to reduce the effects of interference by reporting to the network a list of cells that may cause significant interference. Similar to supporting multiple height ranges in the combined measurement event, to configure configuring different numberoftriggeringcells values to correspond to different height ranges is beneficial for measurements reporting overhead reduction.

Proposal 4: To configure configuring different numberoftriggeringcells values to correspond to different height ranges is beneficial for measurements reporting overhead reduction.

- Whether to keep the measured cells in cellTriggeredList when UE moves to a new height region

If a height-dependent numberOfTriggeringCells is applied, the new value needs to be applied immediately when the numberOfTriggeringCells value is changed during height change. So, there is still a remaining issue about how the UE handles the old cells in cellTriggeredList when the UE applies a new numberOfTriggeringCells.

The main triggering reason for the numberOfTriggeringCells update is the height range change during UAV mobility. In this case, the old cellTriggeredList measured in the former height range may be partially invalid. However, deleting the old cellTriggeredList directly may increase measurement reporting latency. Therefore, it needs to be a trade-off between measurement reporting latency and measurement results accuracy. From the perspective of the network, it is hard to address this issue. Therefore, this issue could be left to UE implementation.

Proposal 5: Whether to discard or keep the old cells in the cellTriggeredList when the numberOfTriggeringCells is changed in a new height region is left to UE implementation.

- Conclusions

Based on the above analysis, we have the following observation and proposals:

Proposal 1: The number of height ranges and height threshold(s) can be configurable by the network.

Proposal 2: ToAddModList/ToRemoveList structure can be used to configure the height-based list of SSB-ToMeasure.

Proposal 3: The gNB’s configuration for the height-based list of SSB-ToMeasure will avoid overlapping height regions.

Proposal 4: To configure different numberoftriggeringcells values corresponding to different height ranges.

Proposal 5: Whether to discard or keep the old cells in the cellTriggeredList when the numberOfTriggeringCells is changed in a new height region is left to UE implementation.

- References

- RP-213600 WI on NR Support for UAV (NR_UAV), RAN#94e, December 2021.

- R2-230xxxx Report of 3GPP TSG RAN WG2 meeting #121bis-e, Online