3GPP TSG-RAN WG2 Meeting #123                                                                        R2-2308653

Toulouse, France, August 21st- 25th 2023

Agenda item:	7.8.3

Source:	China Telecom

Title:	Discussion on Flight Path Reporting for NR UAV

Document for:	Discussion

- Introduction

One of the objectives in UAV WID is to specify the enhancements on measurement reports. During the previous meetings, the following agreements on measurement reports for UAV were made [2]:

Agreements from RAN2#121bis:

- Flightpath update indication in UAI is configurable by the network

- Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE

- Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307]

- As a baseline, we can consider a simple network control mechanism (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication.  FFS if further differentiation is needed if we decide to have delta signaling

Agreements from RAN2#122:

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

This paper mainly discusses the trigger condition for flight path updating and the remaining issues on flight path reporting.

- Discussion

2.1	Triggering and Indication of Flight Path Updating

We believe it is important to consider a scenario where only one waypoint in the flightpath is greatly changed, while the other waypoints remain unchanged. For instance, the addition or deletion of just one waypoint. If we only consider the changes in delta time or delta distance, it would lead to frequent and unnecessary flight path updates. In order to reduce unnecessary flight path updating, it needs to configure the sum of changes in delta time or delta distance.

Proposal 1: In order to reduce unnecessary flight path updating, it needs to configure the sum of changes in delta time or delta distance.

We believe it is important to consider a scenario where only one waypoint in the flightpath is greatly changed, while the other waypoints remain unchanged. For instance, the addition or deletion of just one waypoint. If we only consider the changes in delta time or delta distance, it would lead to frequent and unnecessary flight path updates.

In the previous RAN2#121bis meeting, RAN2 agreed that a single indication is used for both initial and updated flightpath available. In case the delta signaling was introduced, the update indication also needs the corresponding enhancement. Based on the above proposal, the updated flightpath indication should also include the trigger reason for flightpath update, rather than only indicate the flight path update available.

Proposal 2: The updated flightpath indication should also include the trigger reason for flightpath update.

2.2	Delta Flight Path Reporting

In RAN2 #122 meeting, there was no consensus on whether or not to support delta flight path reporting. The data volume of the flight path information may be very large. To report entire flight path information in case flight path updates may bring negative impact, such as interference towards neighbor nodes, to network and UAV experience. In view of the fact that it is probable that only a part of the flight path is updated/changed in a practical flight path changing scenario. We understand that introducing delta flight path report in NR UAV is benefit to save the radio resource significantly.

Proposal 3: To introduce delta flight path report in Rel-18 NR UAV.

- Conclusions

Based on the discussion in the previous sections we propose the following:

Proposal 1: In order to reduce unnecessary flight path updating, it needs to configure the sum of changes in delta time or delta distance.

Proposal 2: The updated flightpath indication should also include the trigger reason for flightpath update.

Proposal 3: To introduce delta flight path report in Rel-18 NR UAV.

- References

- RP-213600 WI on NR Support for UAV (NR_UAV), RAN#94e, December 2021.

- R2-230xxxx Report of 3GPP TSG RAN WG2 meeting #122, Incheon, Korea