3GPP TSG-RAN WG2 Meeting #123	R2-2308668

Toulouse, France, 21st – 25th August, 2023	Revision of R2-2305899

Agenda Item:	7.5.4.2

Source:	MediaTek Inc.

Title:	Further aspects of PDU discard

Document for:	Discussion and decision

# 1 Introduction

- PDU set discard is modelled using the existing PDCP discard timer for the uplink. The timer is in network control.

- Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling.

- FFS how/whether to minimize additional UL signalling after this indication.

- FFS if the NW indication is a one-shot or also subsequent packets

We reached the following agreements on discard behaviour for PDU sets.

In this document, we go into further detail on the implementation aspects of PSDB based discard. We also discuss PDU Set importance based discard  and how it can be used for data discard procedures to improve capacity.

# 2 Discussion

## 2.1 PSDB based discard

Given that we’re reusing the PDCP discard timer to model PSDB based discard, the first step is to check whether the current range of values of the timer that can be configured by the network are sufficient. From the RRC specification [1], we have the following range of values that can be configured for the discard timer:

discardTimer            ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200,

ms250, ms300, ms500, ms750, ms1500, infinity}

DiscardTimerExt-r16 ::= ENUMERATED {ms0dot5, ms1, ms2, ms4, ms6, ms8, spare2, spare1}

The existing range of discard timer values maps well to the PDB values as defined in the TR 38.838 [2]. Therefore we can continue to use the existing PDCP discard timer range as is for PSDB based discard.

Proposal 1: The current range of the PDCP discard timer is sufficient to cover PSDB based discard for XR.

We had a brief discussion on the transmitter entity notifying the receiver entity of discarded PDUs to avoid window stalls. This is important because when data is discarded, it may result in ‘holes’ in the sequence number space at the receiver. If receiver is unaware of the existence of such ‘holes’, it can result in a window stall at the receiver (reassembly/reordering timers) as it waits for this missing data to arrive. Such a window stall at the receiver is undesirable as it results in unnecessary delays to delay-sensitive XR traffic, and should be avoided. Therefore, when PDU discard takes place at the transmitter, it is important that the receiver is made aware of this operation to avoid unnecessary costly delays to the processing of XR traffic.

Proposal 2: The receiver entity is notified of any PDUs that are discarded by the transmitter entity to avoid window stalls.

Figure 1 PDCP control PDU format for PDCP status report

A straightforward way to do so is to reuse the structure of the PDCP status report. It’s structure is shown in Figure 1 for reference. FMC indicates the count of the first packet that is missing (in our case, dropped by the transmitter) which is then followed by a bitmap of subsequent missing packets. This structure can be used as is, as it is suitable for our purpose, i.e. for the transmitter to notify the receiver of those packets that are dropped by the transmitter.

Proposal 3: The notification from the transmitter entity of discarded packets reuses the structure of the PDCP status report.

## 2.2 PSI based discard

In case of real time streams, it is also equally important to present the latest information to the end user where possible. This is possible when independently decodable frames such as I-frames arrive. If the RAN is aware of an independent frame (using the PDU Set importance indicator of the PDU set), the transmission of this information to the receiver can be expedited. Given that all existing data in the transmission buffer correspond to older video frames that are now irrelevant to the real time video stream, there is no point in transmitting them any longer and can be dropped as illustrated in Figure 2 below. This will help ease congestion in the network while improving the end-user experience of the real time video stream.

Figure 2: Prioritisation of an independent frame along with data discard to ease congestion

Observation 1: In cases where PSDB expiry based PDU discard cannot be used (e.g. differentially encoded information), it is useful to drop all earlier PDU sets in the transmission buffer of the transmitter when a PDU set of high importance arrives, to ease congestion and to ensure that newly arriving independent video frames can be provided to the end-user in a timely manner.

Proposal 4: Arrival of a PDU set with high importance can trigger the discard of all PDU sets that arrived earlier.

At the last meeting, there were discussions on whether:

- An additional PDU set timer is needed before the PSI based discard operation occurs

- A network trigger is needed to indicate that congestion exists in the NW to activate PSI based discard

The case for an additional timer is justifiable, as there may be buffered data that hasn’t exceeded its delay budget and its transmission could still make sense. Obviously, if data periodicity is equal to or larger than its delay budget, there is no reason for such a timer. So the need/configuration of such a timer should be optional as it’s only needed when delay budget exceeds periodicity of data.

Proposal 5: An optional PDCP timer is introduced per PDU set, which is started when the first PDU of a PDU set arrives at PDCP, and the arrival of a PDU set with high importance can trigger the discard of all PDU sets that arrived earlier for which this timer has expired.

On the other hand, the case for a network indication of congestion is weaker. To start with, if there is buffered data in the UE that has exceeded its delay budget, it’s an implicit congestion indication as the NW is unable to schedule UL data transmission in time. In this scenario, once the reasons for the NW’s inability to schedule UL data goes away (i.e. no longer experiencing congestion), PSI based discard ensures that the UE sends the most relevant/important data as early as possible. Furthermore, in case the NW is experiencing congestion, the last thing we should introduce is further signalling (to indicate congestion to all UEs) which could exacerbate the situation. For there reasons, we do not see a need for a NW indication of congestion.

Proposal 6: PSI based discard is active upon configuration, i.e. there is no need for further network indications to activate/deactivate PSI based discard.

# 3 Conclusion

In this contribution we have the following observations and proposals:

On PSDB based discard

Proposal 1: The current range of the PDCP discard timer is sufficient to cover PSDB based discard for XR.

Proposal 2: The receiver entity is notified of any PDUs that are discarded by the transmitter entity to avoid window stalls.

Proposal 3: The notification from the transmitter entity of discarded packets reuses the structure of the PDCP status report.

On PSI based discard

Observation 1: In cases where PSDB expiry based PDU discard cannot be used (e.g. differentially encoded information), it is useful to drop all earlier PDU sets in the transmission buffer of the transmitter when a PDU set of high importance arrives, to ease congestion and to ensure that newly arriving independent video frames can be provided to the end-user in a timely manner.

Proposal 4: Arrival of a PDU set with high importance can trigger the discard of all PDU sets that arrived earlier.

Proposal 5: An optional PDCP timer is introduced per PDU set, which is started when the first PDU of a PDU set arrives at PDCP, and the arrival of a PDU set with high importance can trigger the discard of all PDU sets that arrived earlier for which this timer has expired.

Proposal 6: PSI based discard is active upon configuration, i.e. there is no need for further network indications to activate/deactivate PSI based discard.

# 4 References

- TS 38.331 v17.4.0

- TR 38.838 v17.0.0