3GPP TSG-RAN WG2 Meeting #123	 R2-2308670

Toulouse, France 21 - 25 August 2023

Agenda Item:	7.21.2

Source:	Huawei, China Southern Power Grid, HiSilicon

Title:	RRC aspects for Msg1 repetition

Document for:	Discussion and decision

1. Introduction

In previous RAN2 meetings, how to support Msg1 repetition for Rel-18 further NR coverage enhancements was discussed. There is an ongoing email discussion [1] for CP open issues. In this contribution, based on previous agreements and the email discussion progress, we discuss how to support Msg1 repetitions from RRC perspective.

2. Discussion

2.1 Msg1 repetition for ReconfigurationWithSync CFRA

In RAN2 #122 meeting, RAN2 intends to support CFRA for Msg1 repetition for ReconfigurationWithSync case, as extracted below.

Agreements in RAN2 #122 - RAN2 intends to support CFRA for msg1 repetition for ReconfigurationWithSync case, FFS for other cases.

In the email discussion [1], majority companies support to have one particular repetition number indication from the network, instead of selecting a repetition number from many by the UE. The main reason is that the network has information to determine a repetition number, e.g., by RRM measurement results in the handover preparation step. Therefore, it is unnecessary that the UE evaluates the downlink RSRP again.

Observation 1: The network can indicate one Msg1 repetition number to the UE for ReconfigurationWithSync CFRA case.

In Rel-17 RACH partitioning design, a set of Random Access resources is selected before the SSB selection. Following this design, for Rel-18 Msg1 repetition in CBRA case, a Msg1 repetition number should be determined and a set of Random Access resources should be selected before the SSB selection. It means, for CBRA, the UE does not choose a Msg1 repetition number per SSB. Therefore, for ReconfigurationWithSync CFRA case, we can re-use this logic, i.e., the network indicates one repetition number for all SSBs/CFRA resources, instead of one repetition number per SSB/CFRA resource.

For ReconfigurationWithSync CFRA case, the network indicates one Msg1 repetition number which is applicable to all SSBs/CFRA resources.

2.2 Msg1-based SI request configuration

In this email discussion [1], some companies propose to support Msg1 repetition in Msg1-based SI request. Currently, there are separate Msg1-based SI request RACH resources, which are specific to regular UEs (e.g., si-RequestConfig), specific to RedCap UEs (e.g., si-RequestConfigRedCap), and specific to positioning (e.g., posSI-RequestConfig), respectively. If Msg1 repetition is supported in Msg1-based SI request, RAN2 needs to discuss whether it is applicable to RedCap UEs and positioning, in addition to regular UEs.

In Rel-17 coverage enhancement discussion, RedCap UEs are able to use the Msg3 repetition feature to enhance uplink coverage. In general, in order to further uplink coverage, we tend to believe that the Rel-18 Msg1 repetition feature can be used for RedCap UEs as well, including the Msg1-based SI request. Therefore, Msg1-based SI request with Msg1 repetition should be supported for RedCap UEs.

If Msg1-based SI request with Msg1 repetition is agreed, it is also applicable to RedCap UEs.

Regarding on demand positioning system information request, we see no motivation to exclude this case considering that it is also beneficial for cell edge UEs to use Msg1 repetition in Msg1-based SI request. Thus we think Msg1 repetition can be considered to all kind of Msg1-based SI request.

If Msg1-based SI request with Msg1 repetition is agreed, it is also applicable to on demand positioning system information request.

Another issue related to SI request is: when the UE is in bad coverage area, if Msg1-based SI request without Msg1 repetition and Msg1 repetition for normal RACH are configured, how the UE chooses RACH resources for SI request. Following the legacy procedure as extracted below, the UE will initiate Msg1-based SI request without Msg1 repetition, because the network provides this specific RACH resource and the UE will use it. However, this procedure may fail because the UE cannot send Msg1 repetition even it is in bad coverage area.

### 5.1.2 Random Access Resource selection […] 1> else if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and 1> if the Random Access Resources for SI request have been explicitly provided by RRC: 2> if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available: 3> select an SSB with SS-RSRP above rsrp-ThresholdSSB. 2> else: 3> select any SSB. 2> select a Random Access Preamble corresponding to the selected SSB, from the Random Access Preamble(s) determined according to ra-PreambleStartIndex as specified in TS 38.331 [5]; 2> set the PREAMBLE_INDEX to selected Random Access Preamble.

##### 5.2.2.3.3 Request for on demand system information The UE shall, while SDT procedure is not ongoing: 1> if SIB1 includes si-SchedulingInfo containing si-RequestConfigSUL and criteria to select supplementary uplink as defined in TS 38.321[3], clause 5.1.1 is met: 2> trigger the lower layer to initiate the Random Access procedure on supplementary uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfigSUL corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting; 2> if acknowledgement for SI request is received from lower layers: 3> acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

To solve this issue, there are two options. Opt1 is that we rely on the network implementation to ensure that the network will provide Msg1-based SI request with Msg1 repetition RACH resources when needed for some deployment scenarios. So the UE will use Msg1 repetition for Msg1-based SI request by largely reusing the existing RRC procedure. Regarding the repetition number selection, it can performed in MAC as other cases and RRC only triggers the RACH procedure with the corresponding MSG1 repetition RACH resources.

Opt 2 is that the UE uses RACH resources of Msg1 repetition for normal RACH, and then uses Msg3-based SI request. In Opt2, before the UE determines to use Msg1-based SI request or Msg3-based SI request, the UE shall first check whether Msg1 repetition is needed. For instance, if DL RSRP is lower than a particular threshold, the UE chooses to use Msg1 repetition. If the Msg1-based SI request RACH resources are not applicable to Msg1 repetition, then the UE will choose to initiate Msg3-based SI request. Regarding the thresholds, they can be the same with the normal Msg1 repetition selection thresholds (e.g., rsrp-ThresholdMsg1-RepetitionNum2-r18,  rsrp-ThresholdMsg1-RepetitionNum4-r18 in the TP of the email discussion [1]). We do not see the need to introduce additional thresholds, which only makes the procedure more complex. This option offers more flexibility of network configurations, but will introduce additional complexity on the UE behaviour and a big burden to RRC spec. So we prefer to adopt Opt 1 considering in most cases, network implementation is sufficient to avoid any issue.

Discuss the following two options for determine Msg1-based or Msg3-based SI request:

- Opt1: The UE following the legacy Msg1-based or Msg3 based selection procedure and the network ensures that Msg1-based SI request with Msg1 repetition RACH resources are provided when needed for some deployment scenarios.

- Opt2: The UE first checks whether the Msg1 repetition is needed by comparing the DL RSRP with thresholds. When Msg1 repetition is needed, if there is Msg1 repetition RACH resources for normal RACH and there is no Msg1-based SI request with Msg1 repetition RACH resources, the UE uses Msg3-based SI request.

2.3 Repetition number configuration

In the email discussion [1], regarding how to configure the repetition number, there are three options.

- Option 1: use three reserved bits in featureCombiantion IE for indicating the MSG1 repetition number 2, 4 and 8. - Option 2: use one reserved bit in featureCombiantion IE for indicating MSG1 repeititon as a feature and introduce a new paratmer in featureCombinationPreamble IE for indicating the associated MSG1 repetition number 2, 4 or 8. - Option 3: introduce a new paratmer outside featureCombiantion IE for indicating the associated MSG1 repetition number 2, 4 or 8. No change to featureCombiantion IE.

In Option 1, each Msg1 repetition number is regarded as a separate feature. However, there are only four spare fields in FeatureCombination, and we should not use three of them considering the future extension. In Option 2, a new parameter in featureCombinationPreamble will indicate the UE whether this set of RACH resources can be used for Msg1 repetition feature. And an associated Msg1 repetition number further tells which exact Msg1 repetition number. Considering the future extension, the Option 2 is preferable since it only occupies one spare field. We also provide a RRC TP in Annex.

Use one spare field in FeatureCombination to indicate the Msg1 repetition and use one additional field in the corresponding FeatureCombinationPreambles to indicate the Msg1 repetition number.

2.4 Msg1 repetition and Msg3 repetition co-existence

In Rel-17, Msg3 repetition parameter is provided in BWP-UplinkCommon IE as shown below:

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }                                  OPTIONAL    -- Cond SpCellOnly2

]],

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                                    OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }               OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                                   OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17                  OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                               OPTIONAL   -- Cond Msg3Rep

]]

}

When UE receives a RAR, it will interpret the field in the RAR grant based on the above configured Msg3 repetition parameter based on TS38.214 as shown below:

Table 6.1.4.1-3: MCS index IMCS as a function of 2 LSBs of MCS information field in RAR UL grant

| mcs-Msg3-Repetitions is configured |  |  | mcs-Msg3-Repetitions is not configured |  |
| --- | --- | --- | --- | --- |
| Codepoint | IMCS |  | Codepoint | IMCS |
| 00 | First value of mcs-Msg3-Repetitions |  | 00 | 0 |
| 01 | Second value of mcs-Msg3-Repetitions |  | 01 | 1 |
| 10 | Third value of mcs-Msg3-Repetitions |  | 10 | 2 |
| 11 | Fourth value of mcs-Msg3-Repetitions |  | 11 | 3 |

Table 6.1.2.1-1A: Number of repetition K as a function of 2 MSBs of MCS information field

| numberOfMsg3-RepetitionsList is configured |  |  | numberOfMsg3-RepetitionsList is not configured |  |
| --- | --- | --- | --- | --- |
| Codepoint | K |  | Codepoint | K |
| 00 | First value of numberOfMsg3-RepetitionsList |  | 00 | 1 |
| 01 | Second value of numberOfMsg3-RepetitionsList |  | 01 | 2 |
| 10 | Third value of numberOfMsg3-RepetitionsList |  | 10 | 3 |
| 11 | Fourth value of numberOfMsg3-RepetitionsList |  | 11 | 4 |

For Rel-18, it was agreed that the co-existence between Msg3 repetition and Msg1 repetition is possible. We think the existing IE for Msg3 repetition parameter made for RA with single preamble transmission may not be suitable for RA with Msg1 repetition. Therefore, a separate new parameter for Msg3 repetition (e.g. numberOfMsg3-RepetitionsList-V1800 and mcs-Msg3-Repetitions-V1800) with different values is required when Msg1 repetition is used. We also provide a RRC TP in the Annex.

Introduce separate Msg3 repetition parameter (e.g. numberOfMsg3-RepetitionsList and mcs-Msg3-Repetitions) for Msg1 repetition.

2.5 Additional RACH configuration

In Rel-17 RACH partitioning topic, AdditionalRACH-ConfigList-r17 was introduced to provide more PRACH resources. Though the value of maxAdditionalRACH-r17 is 256, the field description restricts that the maximum number of entries of the list is 16. In Rel-18, each Msg1 repetition number is regarded as a separate feature which may result in various new feature combination requiring for RA resource. RAN2 needs to discuss whether more additional PRACH resources should be provided. If more PRACH resources are needed, then the field description of additionalRACH-ConfigList should be updated, i.e., more than 16 entries can be configured.

BWP-UplinkCommon ::=                SEQUENCE {

----omitted----

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                                    OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }               OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                                   OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17                  OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                               OPTIONAL   -- Cond Msg3Rep

]]

}

AdditionalRACH-ConfigList-r17 ::=       SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                                   OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                               OPTIONAL,  -- Need R

...

}

maxAdditionalRACH-r17                   INTEGER ::= 256     -- Maximum number of additional RACH configurations.

additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

RAN2 to discuss whether the network can configure more than 16 entries for additionalRACH-ConfigList to provide more PRACH resources for increasing features.

3. Conclusion

Based on the discussion in this contribution, we have the following proposals:

- For ReconfigurationWithSync CFRA case, the network indicates one Msg1 repetition number which is applicable to all SSBs/CFRA resources.

- If Msg1-based SI request with Msg1 repetition is agreed, it is also applicable to RedCap UEs.

- If Msg1-based SI request with Msg1 repetition is agreed, it is also applicable to on demand positioning system information request.

Discuss the following two options for determine Msg1-based or Msg3-based SI request:

- Opt1: The UE following the legacy Msg1-based or Msg3 based selection procedure and the network ensures that Msg1-based SI request with Msg1 repetition RACH resources are provided when needed for some deployment scenarios.

- Opt2: The UE first checks whether the Msg1 repetition is needed by comparing the DL RSRP with thresholds. When Msg1 repetition is needed, if there is Msg1 repetition RACH resources for normal RACH and there is no Msg1-based SI request with Msg1 repetition RACH resources, the UE uses Msg3-based SI request.

- Use one spare field in FeatureCombination to indicate the Msg1 repetition and use one additional field in the corresponding FeatureCombinationPreambles to indicate the Msg1 repetition number.

- Introduce separate Msg3 repetition parameter (e.g. numberOfMsg3-RepetitionsList and mcs-Msg3-Repetitions) for Msg1 repetition.

- RAN2 to discuss whether the network can configure more than 16 entries for additionalRACH-ConfigList to provide more PRACH resources for increasing features.

A text proposal for Proposal 3 is provided in annex below.

4. Reference

[1] [POST 122][801][R18 CE enh - CP] CP open issues (Huawei).

5. Annex: TP

#### –	BWP-UplinkCommon

The IE BWP-UplinkCommon is used to configure the common parameters of an uplink BWP. They are "cell specific" and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BWP-UplinkCommon information element

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }                                  OPTIONAL    -- Cond SpCellOnly2

]],

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                                    OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }               OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                                   OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17                  OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                               OPTIONAL   -- Cond Msg3Rep

]],

[[

numberOfMsg3-RepetitionsList-v1800  SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17                  OPTIONAL,  -- Cond Msg1AndMsg3Rep

mcs-Msg3-Repetitions-v1800          SEQUENCE (SIZE (8)) OF INTEGER (0..31)                               OPTIONAL   -- Cond Msg1AndMsg3Rep

]]

}

AdditionalRACH-ConfigList-r17 ::=       SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                                   OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                               OPTIONAL,  -- Need R

...

}

NumberOfMsg3-Repetitions-r17::=         ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16}

-- TAG-BWP-UPLINKCOMMON-STOP

-- ASN1STOP

BWP-UplinkCommon field descriptions

additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

enableRA-PrioritizationForSlicing Indicates whether or not the ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep should override the ra-PrioritizationForAccessIdentity. The field is applicable only when the UE is configured by upper layers with both NSAG and Access Identity 1 or 2. If value TRUE is configured, the UE should only apply the ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep. If value FALSE is configured, the UE should only apply ra-PrioritizationForAccessIdentity. If the field is absent, whether to use ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep or ra-PrioritizationForAccessIdentity is up to UE implementation.

mcs-Msg3-Repetitions Configuration of eight candidate MCS indexes for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. Only the first 4 configured or default MCS indexes are used for PUSCH transmission scheduled by RAR UL grant. This field is only applicable when the UE selects Random Access resources indicating Msg3 repetition in this BWP. If this field is absent when the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon, the UE shall apply the values {0, 1, 2, 3, 4, 5, 6, 7} (see TS 38.214 [19], clause 6.1.4).

msgA-ConfigCommon Configuration of the cell specific PRACH and PUSCH resource parameters for transmission of MsgA in 2-step random access type procedure. The NW can configure msgA-ConfigCommon only for UL BWPs if the linked DL BWPs (same bwp-Id as UL-BWP) are the initial DL BWPs or DL BWPs containing the SSB associated to the initial BL BWP

numberOfMsg3-RepetitionsList The number of repetitions for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. This field is only applicable when the UE selects Random Access resources indicating Msg3 repetition in this BWP. If this field is absent when the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon, the UE shall apply the values {n1, n2, n3, n4} (see TS 38.214 [19], clause 6.1.2.1).

pucch-ConfigCommon Cell specific parameters for the PUCCH of this BWP.

pusch-ConfigCommon Cell specific parameters for the PUSCH of this BWP.

rach-ConfigCommon Configuration of cell specific random access parameters which the UE uses for contention based and contention free random access as well as for contention based beam failure recovery in this BWP. The NW configures SSB-based RA (and hence RACH-ConfigCommon) only for UL BWPs if the linked DL BWPs (same bwp-Id as UL-BWP) are the initial DL BWPs or DL BWPs containing the SSB associated to the initial DL BWP or for RedCap UEs DL BWPs associated with nonCellDefiningSSB or the RedCap-specific initial downlink BWP. The network configures rach-ConfigCommon, whenever it configures contention free random access (for reconfiguration with sync or for beam failure recovery).

rach-ConfigCommonIAB Configuration of cell specific random access parameters for the IAB-MT. The IAB specific IAB RACH configuration is used by IAB-MT, if configured.

rsrp-ThresholdMsg3 Threshold used by the UE for determining whether to select resources indicating Msg3 repetition in this BWP, as specified in TS 38.321 [3]. The field is mandatory if both set(s) of Random Access resources with MSG3 repetition indication and set(s) of Random Access resources without MSG3 repetition indication are configured in the BWP. It is absent otherwise.

useInterlacePUCCH-PUSCH If the field is present, the UE uses uplink frequency domain resource allocation Type 2 for cell-specific PUSCH, e.g., PUSCH scheduled by RAR UL grant (see TS 38.213 [13] clause 8.3 and TS 38.214 [19], clause 6.1.2.2) and uses interlaced PUCCH Format 0 and 1 for cell-specific PUCCH (see TS 38.213 [13], clause 9.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| Msg3Rep | This field is optionally present, Need S, if the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon. It is absent otherwise. |
| Msg1AndMsg3Rep | This field is optionally present, Need S, if the set(s) of Random Access resources with both MSG1 repetition and MSG3 repetition indication are configured in the BWP-UplinkCommon. It is absent otherwise. |
| RA-PrioSliceAI | The field is optionally present in SIB1, Need R, if both parameters ra-PrioritizationForAccessIdentity and the ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep are present in SIB1. It is absent otherwise. |
| SpCellOnly2 | The field is optionally present, Need M, in the BWP-UplinkCommon of an SpCell. It is absent otherwise. |

[Skip unchanged part]

#### –	FeatureCombination

The IE FeatureCombination indicates a feature or a combination of features to be associated with a set of Random Access resources (i.e. an instance of FeatureCombinationPreambles).

FeatureCombination information element

-- ASN1START

-- TAG-FEATURECOMBINATION-START

FeatureCombination-r17 ::= SEQUENCE {

redCap-r17                 ENUMERATED {true}                                    OPTIONAL,  -- Need R

smallData-r17              ENUMERATED {true}                                    OPTIONAL,  -- Need R

nsag-r17                   NSAG-List-r17                                        OPTIONAL,  -- Need R

msg3-Repetitions-r17       ENUMERATED {true}                                    OPTIONAL,  -- Need R

msg1-Repetitions-r18       ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare3                     ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare2                     ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare1                     ENUMERATED {true}                                    OPTIONAL   -- Need R

}

NSAG-List-r17 ::= SEQUENCE (SIZE (1.. maxSliceInfo-r17)) OF NSAG-ID-r17

-- TAG-FEATURECOMBINATION-STOP

-- ASN1STOP

FeatureCombination field descriptions

redCap If present, this field indicates that RedCap is part of this feature combination.

smallData If present, this field indicates that Small Data is part of this feature combination.

nsag If present, this field indicates NSAG(s) that are part of this feature combination.

msg1-Repetitions If present, this field indicates that signalling of msg1 repetition is part of this feature combination. This field is not configured in a set of preambles that is configured with 2-step random-access type.

msg3-Repetitions If present, this field indicates that signalling of msg3 repetition is part of this feature combination. This field is not configured in a set of preambles that is configured with 2-step random-access type.

#### –	FeatureCombinationPreambles

The IE FeatureCombinationPreambles associates a set of preambles with a feature combination. For parameters which can be provided in this IE, the UE applies this field value when performing Random Access using a preamble in this featureCombinationPreambles, otherwise the UE applies the corresponding value as determined by applicable Need Code, e.g. Need S. On a specific BWP, there can be at most one set of preambles associated with a given feature combination per RA Type (i.e. 4-step RACH or 2-step RACH).

FeatureCombinationPreambles information element

-- ASN1START

-- TAG-FEATURECOMBINATIONPREAMBLES-START

FeatureCombinationPreambles-r17 ::=   SEQUENCE {

featureCombination-r17                FeatureCombination-r17,

startPreambleForThisPartition-r17     INTEGER (0..63),

numberOfPreamblesPerSSB-ForThisPartition-r17 INTEGER (1..64),

ssb-SharedRO-MaskIndex-r17            INTEGER (1..15)                                           OPTIONAL, -- Need S

groupBconfigured-r17                  SEQUENCE {

ra-SizeGroupA-r17                     ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB-r17          ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA-r17        INTEGER (1..64)

}                                                                                               OPTIONAL, -- Need R

separateMsgA-PUSCH-Config-r17         MsgA-PUSCH-Config-r16                                     OPTIONAL, -- Cond MsgAConfigCommon

msgA-RSRP-Threshold-r17               RSRP-Range                                                OPTIONAL, -- Need R

rsrp-ThresholdSSB-r17                 RSRP-Range                                                OPTIONAL, -- Need R

deltaPreamble-r17                     INTEGER (-1..6)                                           OPTIONAL, -- Need R

...,

[[

numberOfMsg1-Repetitions-r18          ENUMERATED {n2, n4, n8, spare1}                           OPTIONAL  -- Need R

]]

}

-- TAG-FEATURECOMBINATIONPREAMBLES-STOP

-- ASN1STOPFeatureCombinationPreambles field descriptions

deltaPreamble Power offset between msg3 or msgA-PUSCH and RACH preamble transmission. If configured, this parameter overrides msg3-DeltaPreamble or msgA-DeltaPreamble, Actual value = field value * 2 [dB] (see TS 38.213 [13], clause 7.1). If msgA-DeltaPreamble is configured in separateMsgA-PUSCH-Config-r17, this field is absent.

featureCombination Indicates which combination of features that the preambles indicated by this IE are associated with. The UE ignores a RACH resource defined by this FeatureCombinationPreambles if any feature within the featureCombination is not supported by the UE or if any of the spare fields within the featureCombination is set to true.

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on (see TS 38.321 [3], clause 5.1.2).

msgA-RSRP-Threshold The UE selects 2-step random access type to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). This field is only present if both 2-step and 4-step RA type are configured for the concerned feature combination in the BWP. If configured, this parameter overrides msgA-RSRP-Threshold-r16. If absent, the UE applies msgA-RSRP-Threshold-r16, if configured

numberOfMsg1-Repetitions It indicates the number of Msg1 repetitions to which the FeatureCombinationPreambles is available. This field is only present when the msg1-Repetitions in featureCombination is set to true.

numberOfPreamblesPerSSB-ForThisPartition It determines how many consecutive preambles are associated to the Feature Combination starting from the starting preamble(s) per SSB.

numberOfRA-PreamblesGroupA It determines how many consecutive preambles per SSB are associated to Group A starting from the starting preamble(s). The remaining preambles associated to the Feature Combination are associated to Group B

ra-SizeGroupA Transport Blocks size threshold in bits below which the UE shall use a contention-based RA preamble of group A. (see TS 38.321 [3], clause 5.1.2). If this feature combination preambles are associated to a RACH-ConfigCommon-twostepRA, this field correspond to ra-MsgA-SizeGroupA, otherwise it corresponds to ra-Msg3SizeGroupA.

rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE. If this parameter is included in FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA, it corresponds to msgA-RSRP-ThresholdSSB, as defined in TS 38.321 [3]. If this parameter is included in FeatureCombinationPreambles which is included in RACH-ConfigCommon, it it corresponds to rsrp-ThresholdSSB, as defined in TS 38.321 [3].

separateMsgA-PUSCH-Config If present it specifies how the 2-step RACH preambles identified by this FeatureCombinationPreambles are mapped to a PUSCH slot separate from the one defined in MsgA-ConfigCommon-r16. If the field is absent, the UE should apply the corresponding parameter in the RACH-ConfigCommonTwoStepRA of the BWP which includes the FeatureCombinationPreambles IE.

ssb-SharedRO-MaskIndex Mask index (see TS 38.321 [3]). Indicates a subset of ROs where preambles are allocated for this feature combination. If this field is configured within FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA: - in case of separate ROs are configured for 4-step and 2-step random access, this field indicates a subset of ROs configured within this RACH-ConfigCommonTwoStepRA; - in case shared ROs are used for 4-step and 2-step random access, it indicates the subset of ROs configured within RACH-ConfigCommon, which are the subset of ROs configured for 2-step random access. This field is configured when there is more than one RO per SSB. If the field is absent, all ROs configured in RACH-ConfigCommon or RACH-ConfigCommonTwoStepRA containing this FeatureCombinationPreambles are shared.

startPreambleForThisPartition It defines the first preamble associated with the Feature Combination. If the UE is provided with a number N of SSB block indexes associated with one PRACH occasion, and N<1, the first preamble in each PRACH occasion is the one having the same index as indicated by this field. If N>=1, N blocks of preambles associated with the Feature Combination are defined, each having start index + startPreambleForThisPartition, where n refers to SSB block index (see TS 38.213 [13], clause 8.1).

| Conditional Presence | Explanation |
| --- | --- |
| MsgAConfigCommon | The field is optionally present, Need S, if FeatureCombinationPreambles is included in RACH-ConfigCommonTwoStepRA. Otherwise, it is absent. If the field is absent in FeatureCombinationPreambles included in RACH-ConfigCommonTwoStepRA, the UE applies MsgA-PUSCH-Config included in the corresponding MsgA-ConfigCommon. |

[Skip unchanged part]