3GPP TSG-RAN WG2 Meeting #123	R2-2308672

Toulouse, France, 21st - 25th August, 2023

Agenda Item:	7.5.4.3

Source: 	MediaTek Inc.

Title:  	HARQ ID determination formula for CG

Document for:	Discussion, decision

- Introduction

In last RAN2 meeting minutes [1], we plan to discuss HARQ ID determination formula for CG due to multiple PUSCH occasions. However, RAN1 had finished related discussion and send their agreements [2] to RAN2. We will discuss the formula agreed by RAN1 and provide our view on their agreements.

# 2	Discussion

## 2.1	Multiple CG PUSCH transmission occasions

## 2.1.1	HARQ ID determination

Introducing support for multiple PUSCH transmission occasions per single CG period requires modification to existing CG HARQ ID formula in TS38.321. RAN1 has started discussions on the subject and have reached the following agreements from the last RAN1 meeting and send LS to RAN2 [2]:

--

Agreement

From RAN1 perspective, for determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH:

- The HARQ process ID for the first configured PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable

  - HARQ Process ID = [X*floor( (CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes

  - HARQ Process ID = [X*floor((CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

    - X= the number of configured PUSCHs in the CG period

- The HARQ process ID of the remaining configured and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable.

- Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

--

We want to clarify the second point of the LS, the harq-ProcID-Offset2 should be outside parentheses. For example, if 5 processes are used and the Offset2 is 10, then we end up doing modulo 15 instead of 5. After module operation, we always use processes 0-14 instead of using processes 10-14. It should be always module with nrofHARQ-Processes and Offset2 should be added after module operation, i.e., harq-ProcID-Offset2 should be outside of the parentheses.

Proposal 1: Clarify and correct the HARQ Process ID formula for multiple CG PUSCH transmission occasions in RAN1 LS.

Another point to be clarified is the definition of valid/invalid CGs do not add value to HARQ PID calculation. It introduced more confusion because TDD patterns can be updated using DCI 2-0 (SFI-RNTI) or by any scheduling DCI. It is unclear whether this should be considered valid or invalid. Similarly SSB could be CD or NCD-SSB, which is also unclear to us. Another case is that if it overlaps with MG, should we consider it as valid or invalid. We suggest to remove the invalid and valid description for CG PUSCH and the Note, i.e., HARQ process ID is increased no matter the CG PUSCH is valid or not.

--

Agreement

- The HARQ process ID of the remaining configured and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable.

- Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

--

Proposal 2: Always increment HARQ PID within a set of configured CG PUSCHs in a period regardless of their validity.

Proposal 3: Send reply LS about correct HARQ Process ID formula back to RAN1.

# 3	Conclusion

We have the following proposals:

Proposal 1: Clarify and correct the HARQ Process ID formula for multiple CG PUSCH transmission occasions in RAN1 LS.

Proposal 2: Always increment HARQ PID within a set of configured CG PUSCHs in a period regardless of their validity.

Proposal 3: Send reply LS about correct HARQ Process ID formula back to RAN1.

# 4	References

[1]	R2-2306542, Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia).

[2]	R1-2306233, LS on XR capacity enhancements, RAN1.