3GPP TSG-RAN WG2 Meeting #123	R2-2308675

Toulouse, France, August 21st-25th, 2023

Source: 	vivo

Title:  	Discussion on UE capability for R18 eNPN

Agenda Item:	7.25.3

Document for:	Discussion and Decision

Introduction

In the last meeting, UE capability for supporting R18 eNPN features was discussed and the related agreements are as follows [1].

R2 assumes that No new AS UE capability (signalled) is needed for supporting R18 eNPN features. FFS any 38.306 impact for conditional UE cap dependent on NAS cap.

In this contribution, we will further discuss this topic and give our proposals.

Discussion

Reviewing the WID on further enhancement of private network support for NG-RAN [2], two objectives are relevant to RAN which are excerpted below.

The objective of this work item is to perform normative work for RAN functionalities required to support 5GS functions of the SA2 WID “Enhanced support of Non-Public Networks Phase 2”, with the following features being of RAN relevance: - Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection [RAN3, RAN2] - Support for non-3GPP access for SNPN [RAN3]

For bullet 2, to support non-3GPP for SNPN, no AS UE capability is needed since the access type (3GPP access or non-3GPP access) is determined by NAS layer, which has no AS impact.

For bullet 1, according to TS 24.501 below [3], whether support equivalent SNPNs is indicated to the NW by UE NAS, which is a NAS feature. If UE indicates it supports equivalent SNPNs, the AMF will provide an equivalent SNPNs list to UE NAS and serving RAN.

TS 24.501 #### 9.11.3.1 5GMM capability The purpose of the 5GMM capability information element is to provide the network with information concerning aspects of the UE related to the 5GCN or interworking with the EPS. The contents might affect the manner in which the network handles the operation of the UE. The 5GMM capability information element is coded as shown in figure 9.11.3.1.1 and table 9.11.3.1.1. The 5GMM capability is a type 4 information element with a minimum length of 3 octets and a maximum length of 15 octets. <Omit the irrelevant part> Table 9.11.3.1.1: 5GMM capability information element Equivalent SNPNs indicator (ESI) (octet 7, bit 7). This bit indicates the capability to support equivalent SNPNs. Bit. 7. 0; Equivalent SNPNs not supported. 1; Equivalent SNPNs supported <Omit the irrelevant part>.

From RAN2 perspective, supporting equivalent SNPNs impacts idle mode and connected mode mobility. To support connected mode mobility between SNPNs, the mobility decision is made by the network correctly since RAN node has the equivalent SNPNs for UE, which is provided via AMF, so new radio access capability with signaling to the network is not needed to be introduced. Moreover, such an assumption was made in the last meeting, we propose to confirm the assumption.

Proposal 1: Confirm that no new AS UE capability with signaling is needed for supporting R18 eNPN features.

To support idle mode mobility between equivalent SNPNs, UE NAS shall maintain one list of equivalent SNPNs and provide the list to UE AS for cell selection and reselection. UE AS only needs to perform cell (re)selection criterion evaluation according to the list of equivalent SNPNs indicated by UE NAS. Therefore, AS naturally supports equivalent SNPNs as long as the NAS supports equivalent SNPNs. Specifically, based on the description of the equivalent SNPN in TS 24.501 below, it is specified that SNPNs in the list of equivalent SNPNs shall be regarded by the UE to each other for SNPN selection, cell selection and cell re-selection. It can be found that it is mandatory for AS to support cell selection and cell re-selection related to the equivalent SNPN if NAS supports it. As NAS specification has clearly defined the capability for equivalent SNPN, AS capability without signaling to the network is not needed. In other words, no RAN specification modification is needed regarding the AS UE capability (i.e., in TS 38.306).

TS 24.501 - 5.3.14A List of equivalent SNPNs The UE may support equivalent SNPNs. If the UE supports equivalent SNPNs, the ME shall store up to one list of equivalent SNPNs: - per entry of "list of subscriber data"; or - per the PLMN subscription, if the UE supports access to an SNPN using credentials from a credentials holder. SNPNs in the list of equivalent SNPNs associated with the selected entry of "list of subscriber data" or the selected PLMN subscription shall be regarded by the UE as equivalent to each other for SNPN selection, cell selection and cell re-selection.

Proposal 2: No new AS UE capability without signaling is needed for supporting R18 eNPN features. No specification impact in TS 38.306.

Conclusions

Based on the analyses given above, we have the following proposals:

Proposal 1: Confirm that no new AS UE capability with signaling is needed for supporting R18 eNPN features.

Proposal 2: No new AS UE capability without signaling is needed for supporting R18 eNPN features. No specification impact in TS 38.306.

Reference

- RAN2#122 Chairman_notes

- RP-230788 New WID on Further Enhancement of Private Network Support for NG-RAN

- TS 24.501 Non-Access-Stratum (NAS) protocol for 5G system (5GS)