3GPP TSG-RAN WG2 Meeting #123                                  R2-2308692

Toulouse, France, 21st August – 25th August, 2023

Agenda item:		7.8.3 (Flight path reporting)

Source:				Samsung

Title:					Discussion on flight path reporting

Document for:		Discussion & Decision

1	Introduction

The following agreements are made regarding flight path reporting:

At RAN2#119bis-e,

Agreements:

- The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE.  FFS on flight path details (waypoints and what is time information).

- Allow the flight path to be updated.  FFS on the details.

- FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting

At RAN2#120,

Agreements:

- A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355.

- A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline.  FFS on granularity

- No requirements are placed on spatial distribution of waypoints

- A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message.   Flight path reporting uses at the UE Information request/response procedure as baseline.

- UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report.

- UAI message can also be used to indicate the UE has flight path availability.

At RAN2#121,

Agreements:

- The granularity of flightpath timestamp is 1s.

- Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE

At RAN2#121bis-e,

Agreements:

- Flightpath update indication in UAI is configurable by the network

- Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE

- Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307]

- As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication.  FFS if further differentiation is needed if we decide to have delta signaling

At RAN2#122,

Agreements:

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

This contribution provides our views on some remaining issues on flight path reporting.

2	Discussion

2.1 Flight path triggering

RAN2 agreed that the flightpath update indication in UAI is configurable by the network. Also, at the last meeting RAN2#122, it was agreed that the NW can configure the UE to trigger a flightpath update notification based on a configured delta time or distance configuration.

Regarding the delta time/distance configuration, we understand that the time and/or distance threshold can be configured by NW via RRCConfiguration and the time/distance threshold configuration should not be mandatory (i.e., should be optional).

Proposal 1: RAN2 to confirm that the gNB can configure the delta time and/or distance threshold via RRCReconfiguration and the configuration should be optional.

We understand that when the delta time and/or distance threshold is configured by NW, the UE can trigger flight path update indication based on the time/distance threshold. However, the specific procedure for that still seems unclear. In our view, if the gNB configures the delta distance threshold, the UE can indicate flight path update when location information of any waypoint within the last report changes more than the distance threshold (e.g., x meters). For the time threshold, if the gNB configures the delta time threshold, the UE can indicate flight path update when time information of any waypoint within the last report changes more than the time threshold (e.g., y secs). Also, when the gNB configures both the distance and time thresholds, the UE can indicate flight path update if either the distance or time condition above is satisfied. Based on the discussion above, we would like to propose the following.

Proposal 2: When the delta time and/or distance threshold are configured by the gNB, the UE can indicate flight path update as below.

- If the delta distance is configured by the gNB, the UE can indicate flight path update when location information of any waypoint within the last report changes more than the distance threshold (e.g., x meters).

- If the delta time is configured by the gNB, the UE can indicate flight path update when time information of any waypoint within the last report changes more than the time threshold (e.g., y secs).

We also need to consider the case that both the distance and time threshold are not configured. In that case, we think the UE can indicate flight path update when there is any change in the location or time information of any waypoint within the last report.

Proposal 3: When both the delta time and distance threshold are not configured by the gNB, the UE can indicate flight path update when there is any change in the location or time information of any waypoint within the last report.

Meanwhile, when both delta time and distance thresholds are not configured, we think at least the prohibit timer should be configured to avoid too frequent UAI message triggered by flight path update. On the other hand, when either the time or distance threshold is configured, the UE should be allowed to indicate flight path update ASAP when either the time or distance condition is met. Thus, for that case, the use of prohibit timer does not make sense.

Proposal 4: If both delta time and distance thresholds are not configured, at least prohibit timer should be configured to avoid too frequent UAI transmission. Otherwise, prohibit timer can not be configured.

2.2 Fligth path available indication

RAN2 agreed to support availability of flight path from aerial UE to gNB through RRC (e.g. xxxComplete message) and flight path reporting through the UE Information request/response procedure in NR, with LTE as baseline. Meanwhile, it is also agreed that UAI message can be used to indicate the UE has flight path availability.

We understand that the UE can use RRCxxxComplete message and UAI message together to report the initial flight path availability to gNB. For example, when the UE has flight path available, it reports the initial availability via RRCSetupComplete message as in LTE. However, if the network does not retrieve flight path information for a certain time period, the UE may report it again using UAI message. To avoid the duplicated flight path availability, we think that the UE includes the initial flight path availability in UAI message only if the UE does not include the flight path availability in RRCxxxComplete message beforehand. Thus, we would like to propose the following:

Proposal 5: The UE sends UAI message including flight path availability to the network, if the UE did not transmit flight path availability before in RRCxxxComplete and UE has flight path information available.

Another open issue is on how to implement the “flightpath available indication” in the UEAssistanceInformation message. In the offline e-mail discussion, the following options are captured from the companies’ contributions.

- Option 1: Single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication) [5,8]

- Option 2: Different indications are used to provide initial or future reporting of the flightpath information, (i.e.one flag is used for initial, and one flag is used to indicate the flightpath update available indication)

- Option 3: Different indications are used to report when flight path info is available and the reason that cause the reporting, i.e. one flag for flight path available, one flag for update cause (e.g. initial, available of new flightpath) [15, 11]

- Option 4: UE can also report when flightpath is unavailable. FFS if this is done via same or different IE as the flight path related information discussed in previous options [7]

At the previous meeting RAN2#121bis-e, it was agreed that single indication is used for both initial and updated flightpath available (i.e., Option1 above) as a baseline with FFS on if further differentiation is needed if we decide to have delta signaling.

From our understanding, a single indication (i.e., same flag) can be used for initial and updated flight path indication regardless of whether delta signalling is introduced or not. If there is any previously reported flight path, the network can regard the flag as update flag. Otherwise, the flag can be regarded as intial flag. Thus, it is sufficient to use single indication for both initial and updated flight path availability.

Proposal 6: RAN2 to agree that a single indication can be used for both initial and updated flightpath available.

2.3 Delta support of flight path reporting

While it is understood that delta reporting of flight path can reduce signalling overhead, we have doubt on whether the delta reporting can be easily implemented with minimum spec. impact.

At the previous meetings, some companies suggest to simply reuse the concept of add/mod/remove lists in the current RRC to support delta reporting of flight path. However, in the current RRC, the delta signalling is supported only in DL RRC message from the network. Since the flight path information is reported by UE via UL RRC message, we need to discuss first whether delta signalling can be allowed also for UL information. In our view, the UL delta reporting seems more like adding a new feature in RRC rather than resuing the existing procedure becase it adds a new requirement to the network to store the information received from each UE and update the information based on the following delta signalling.

Meanwhile, as a baseline, now RAN2 is considering a network control mechanism that controls triggering the flight path update indication. From our understanding, if we allow the network to configure some conditions for flight path update, the concern on signalling overhead of flight path update can be addressed to some extent without having delta reporting.

Based on the observations above, we think the the delta signalling of flight path information is not essential and do not support it considering the expected impact on network implementation.

Proposal 7: RAN2 do not support delta flight path reporting.

2.4 Flight path information

RAN2 agreed to adopt location list of waypoints and optional timestamp as the basic content of flight path report. It is also agreed that the waypoint is described via the existing parameter type LocationCoordinates defined in TS 37.355 [2] and the timestamp provides the UTC time with FFS on granularity.

Regarding time information, RAN2 ageed the following at the previous meeting:

- The granularity of flightpath timestamp is 1s.

- Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE

Now one remaining issue is how to implement the location information in flightpath. For the location information, we think that the existing CommonLocationInfo-r16 IE can be reused for flight path reporting. In that case, at least locationCoordinate-r16 in the IE should be included for location information as in LTE. On the other hand, further discussion seems to be needed what other information in CommonLocationInfo-r16 IE can be useful for flight path information. Note that the CommonLocationInfo-r16 IE is defined as below [1].

CommonLocationInfo-r16 ::= SEQUENCE {

gnss-TOD-msec-r16          OCTET STRING     OPTIONAL,

locationTimestamp-r16      OCTET STRING     OPTIONAL,

locationCoordinate-r16     OCTET STRING     OPTIONAL,

locationError-r16          OCTET STRING     OPTIONAL,

locationSource-r16         OCTET STRING     OPTIONAL,

velocityEstimate-r16       OCTET STRING     OPTIONAL

}

In our view, the velocity information (i.e., velocityEstimate-r16) can be also valuable. With the timestamp information, the network can know when the UE will arrive at the waypoint. However, for mobility support, it may be more crucial to have information on how fast the UE will pass through the point. If the nework is aware of UE’s velocity at a specific location, it could use this information to determine the HO timing and to configure other timing-related parameters (e.g., TAT, TTT for event report …) properly for mobility optimization.

Proposal 8: RAN2 to agree to reuse the existing IE CommonLocationInfo as location information (e.g. locationCoordinate-r16) in flightpath.

Proposal 9: RAN2 to discuss what other information in IE CommonLocationInfo (e.g, velocityEstimates-r16) can be included in flightpath.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: RAN2 to confirm that the gNB can configure the delta time and/or distance threshold via RRCReconfiguration and the configuration should be optional.

Proposal 2: When the delta time and/or distance threshold are configured by the gNB, the UE can indicate flight path update as below.

- If the delta distance is configured by the gNB, the UE can indicate flight path update when location information of any waypoint within the last report changes more than the distance threshold (e.g., x meters).

- If the delta time is configured by the gNB, the UE can indicate flight path update when time information of any waypoint within the last report changes more than the time threshold (e.g., y secs).

Proposal 3: When both the delta time and distance threshold are not configured by the gNB, the UE can indicate flight path update when there is any change in the location or time information of any waypoint within the last report.

Proposal 4: If both delta time and distance thresholds are not configured, at least prohibit timer should be configured to avoid too frequent UAI transmission. Otherwise, prohibit timer can not be configured.

Proposal 5: The UE sends UAI message including flight path availability to the network, if the UE did not transmit flight path availability before in RRCxxxComplete and UE has flight path information available.

Proposal 6: RAN2 to agree that a single indication can be used for both initial and updated flightpath available.

Proposal 7: RAN2 do not support delta flight path reporting.

Proposal 8: RAN2 to agree to reuse the existing IE CommonLocationInfo as location information (e.g. locationCoordinate-r16) in flightpath.

Proposal 9: RAN2 to discuss what other information in IE CommonLocationInfo (e.g, velocityEstimates-r16) can be included in flightpath.

4	Reference

- 3GPP TS 38.331 v17.3.0, ‘Radio Resource Control (RRC) protocol specification’

- 3GPP TS 37.355 v17.3.0, ‘LTE Positioning Protocol (LPP)’