3GPP TSG-RAN WG2 Meeting #123			     R2-2308697

Toulouse, France, 21– 25 August, 2023

Agenda Item:	7.3.5 (Netw_Energy_NR-core)

Source: 	LG Electronics Inc.

Title:         	CHO enhancement for different NES use cases

Document for: 	Discussion

Introduction

The following were agreed in the previous meeting RAN2#122.

Agreements

1.	We will have a CHO solution that considers NES mode of at least source cell.

2.	We can have a specific NES CHO execution condition based on source cell NES mode.   FFS how the UE determines is in NES mode.   FFS on how this is achieved in RRC

3.	We will not introduce new L1 signalling for the purpose of CHO

4.	Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario.   We will study the time based mechanism

In this contribution, we discuss CHO enhancement for different NES use cases.

Discussion

CHO enhancement for different NES use cases

We discuss the potential CHO enhancements for the he following two NES techniques:

- Cell DTX/DRX use case

- Cell-off use case

We first discuss Cell DTX/DRX use case. When network activates Cell DTX/DRX mode, it explicitly or implicitly indicates the activation to UEs. There are two options to indicate the Cell DTX/DRX activation/deactivation

- Case 1: Activated/deactivated implicitly, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released

- Case 2: Activated/deactivated by dynamic L1 or L2 signalling (in this option please state which one do you prefer)

For case1, Cell DTX/DRX configuration is configured by RRC signalling. Once configured by RRC, Cell DTX/DRX is activated immediately. Network can configure new CHO conditions specific (or in relation) to the activation of Cell DTX/DRX when configuring the Cell DTX. If network releases Cell DTX/DRX, it should then release the CHO conditions. In this manner, the CHO conditions work only while the Cell DTX/DRX is activated, and no CHO enhancement is needed.

For case2, RAN1 agreed to introduce L1 group signalling for Cell DTX/DRX activation/deactivation. In this case, Cell DTX/DRX configuration is configured by RRC signalling, and UE applies the L1 signalling based on the configured cell DTX/DRX configuration. Since the activation and deactivation is determined based on L1 signalling, it is inefficient to rely on network reconfiguration of CHO conditions. If network has configured CHO condition specific (or in relation) to the Cell DTX/DRX activation, UE should execute CHO related to the CHO condition only if the CHO condition is met and the UE determines that the Cell DRX/DRX is in activated state (i.e., the L1 activation indication has been received). To support this restrictive CHO execution, some enhancement is needed.

In our view, the simplest way to support such restrictive CHO execution is to introduce a new flag for conditional event for CHO. If the flag is configured for a certain event, the event can be fulfilled only if Cell DTX/DRX state is in an activated state.

The above restrictive CHO execution based on source cell Cell DRX/DRX state can be applied to both the case1 and case2 in principle. Even if no CHO enhancement is needed for case1, it is consistent to apply the CHO restriction to both cases.

Proposal 1.	For a CHO execution condition configured for NES, UE consider the condition to be fulfilled if event of the condition is met and NES state is active

Then we discuss cell-off use case. Before turning off a cell, network wants all UEs in the cell to move out of the cell. Note that cell-off is not an urgent decision. Network can take time before it turns off the cell, therefore network can configure existing CondAx events to trigger CHO and possibly decrease its SSB transmission power gradually to make the event get fulfilled. Furthermore, network normally decides to turn off the cell if cell utilization is very low. Considering the non-urgency of the cell-off, network can take sufficient time to trigger handover to all UEs before the actual cell-off. This implies that any new indication to immediately trigger CHO of UEs is not essential.

In case network can decide the precise time to turn off the cell, CondEventT1 may be used to trigger CHO. If network can rely on CondEventT1, it does not have to keep adjusting its SSB transmission power if network configures the CondEventT1 time window to include the expected cell off time and CondAx associated with the CondEventT1to have aggressive threshold (very high serving cell threshold/very low neighbour cell threshold).

CondEventT1 requires UE to know UTC time. If UE is equipped with GNSS and keeps GNSS measurements, UTC time is available. But, if not, UTC time is not available. If network broadcasts SIB9, UE supporting SIB9 can acquire UTC, but network needs to know whether the UE is supporting UTC acquisition via SIB9. Given the UTC availability requirement, CondEventT1 cannot be assumed as mandatory solution for cell-off use case. But, it can be used as optional solution for UEs capable of acquiring UTC.

Proposal 2.	CondEvent T1 is supported for UE capable of GNSS or SIB9 to support cell-off use case.

Mobility Failure Handling

Network may configure appropriate CHO configuration based on the UE’s measurement. Note that, it cannot be guaranteed that there is always one or more target candidate satisfying the CHO execution condition upon activating NES state. In such case, UE cannot execute CHO, resulting in connection failure. It can also take more time to change Pcell compared to the legacy HO.

Given that the CHO enhancement is introduced for fast Pcell change according to NES state, RAN2 can consider fast recovery process for the mobility (CHO) failure cases as follows:

- Option 1. UE-based Solution:

  - UE performs RRC Reestablishment procedure immediately to select a suitable target cell; or

  - UE performs mobility to a target candidate among the target candidates

- Option 2. NW-based Solution:

  - UE sends an indication when there is no suitable cell for CHO execution. Network sends a HO command with target cell information

Considering the assurance of seamless mobility, Option 2 is a more efficient approach. By allowing the network to have control over the mobility decision in this failure case, it can enable the network to assess the suitable cells and initiate the handover process. Alternatively, network can reconsider NES state (de)activation. Therefore, RAN2 can consider Option 2 for mobility failure handling.

Proposal 3.	The UE should send an indication indicating that there is no suitable cell                             for execution conditional mobility.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Proposal 1.  	For a CHO execution condition configured for NES, UE consider the condition to be fulfilled if event of the condition is met and NES state is active

Proposal 2.	CondEvent T1 is supported for UE capable of GNSS or SIB9 to support cell-off use case.

Proposal 3.	The UE should send an indication indicating that there is no suitable cell                             for execution conditional mobility.