3GPP TSG-RAN WG2 Meeting #123	R2-2308704

Toulouse, France, 21 August – 25 August 2023               (revision of R2-2306216)

Agenda item:	7.8.3

Source: 	Huawei, HiSilicon

Title: 	Further discussion on flight path reporting

Document for:	Discussion and Decision

- Introduction

In RAN2 meeting #122, the following agreements have been reached regarding flight path report for NR UAVs [1]:

Agreements

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

This contribution is a revision of R2- 2303809. Compared to the previous version we analysed a few more technical aspects due to be resolved in RAN2 (i.e. FFSs).

- Discussion

As agreed in RAN2 #122, flight path update indication triggering can be restricted by the gNB. The wording captured in the chairman notes is that “The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamps are configured to be reported) or distance configuration”. It seems that the gNB can configure either delta time or delta distance.

We though that the intention of the agreement of the last meeting was time “and/or” distance, because the gNB may need to configure the delta time and the delta distance simultaneously. The network doesn’t want a report in case for example the time didn’t change but the distance changed by less than e.g. 20 meters. Whether to configure the delta time and/or the delta distance can be left to gNB choice.

On the other hand, for the UE implementation to have time AND distance is not really more complex compared to time OR distance (the software to implement this is simple). Thus, the gNB can configure the delta time “and/or” the delta distance for the flight path update indication trigger to the UE rather than the delta time “or” the delta distance.

Proposal 1: RAN2 should clarify that the gNB can configure the delta time AND/OR the delta distance for the flight path update indication trigger to the UE, rather than the delta time OR the delta distance.

We think delta time and/or distance can control the frequency of the UE’s flight path update indication very well. Sometimes, the different gNBs may be interested in different waypoints. For example, we assume gNB1 is interested in waypoints 1, 2, 3 (these waypoints are in the coverage of gNB1) and gNB2 is interested in waypoints 5, 6, 7 (these waypoints are in the coverage of gNB2). It is obvious that gNB1 does not care about the changing of others except for waypoints 1, 2, and 3. Another example is that the gNB may need to configure different delta time and/or distance for the centre of the cell and the edge of the cell when the UAV crosses the cell. Thus, the gNB needs to configure the delta time and/or the delta distance only for certain waypoints for the flight path update indication trigger to the UE.

Besides, the gNB may be more interested in certain waypoints and does not want to put any limitations on these waypoints, i.e., these waypoints need to be reported every time, even for the smallest change. In this case, the gNB can set delta time and/or distance to zero as the triggering condition for flight path update indication.

Sometimes, the gNB may be interested in the entire flight path (e.g., a loop flight path where all waypoints are covered by one gNB). Thus, the gNB needs to configure the delta time and/or the delta distance for the entire flight path for the flight path update indication trigger to the UE.

One more issue that needs to be solved if the gNB configures the delta time and/or the delta distance for certain waypoints is the UE behaviour for the rest of the waypoints. These waypoints are not configured by the gNB, which means the gNB is not interested in these waypoints. Thus, the UE should not send the flight path update indication to the gNB even if these waypoints have changed.

Proposal 2: The gNB can configure the delta time and/or the delta distance for certain waypoints or the entire flight path for the flight path update indication trigger to the UE.

Proposal 3: The gNB can set the delta time and/or distance to zero if the gNB wants the UE to report the flight path update indication once the flight path changes.

Proposal 4: If one waypoint has changed but the gNB does not configure the delta time and/or delta distance for this waypoint, then the UE should not send the flight path indication to the gNB.

RAN2 has agreed that the source gNB forwards the flight path to the target gNB during the handover procedure, which can reduce the time that the target gNB obtains the flight path. However, the source gNB may not have (collected) the new flight path or anyway have an old (out of date) flight path, but it has (received) the flight path (update) indication. This is because the source gNB may not be interested in the flight path (update) when it receives the flight path indication. At this time, the source gNB needs to send the old flight path to the target gNB according to the current RAN2 agreement. Then, the target gNB may make the wrong decision based on this old (out of date) flight path. Thus, if the source gNB has a flight path (update) indication and the old (out of date) flight path, the source gNB should forward the flight path (update) indication to the target gNB during the handover procedure, so that the target gNB is aware of this and can fetch the new flight path from the UE.

Proposal 5: If the source gNB has a flight path (update) indication and the old flight path, the source gNB should forward the flight path (update) indication to the target gNB during the handover procedure.

- Conclusion

Proposal 1: RAN2 should clarify that the gNB can configure the delta time AND/OR the delta distance for the flight path update indication trigger to the UE rather than the delta time OR the delta distance.

Proposal 2: The gNB can configure the delta time and/or the delta distance for certain waypoints or the entire flight path for the flight path update indication trigger to the UE.

Proposal 3: The gNB can set the delta time and/or distance to zero if the gNB wants the UE to report the flight path update indication once the flight path changes.

Proposal 4: If one waypoint has changed but the gNB does not configure the delta time and/or delta distance for this waypoint, then the UE should not send the flight path indication to the gNB.

Proposal 5: If the source gNB has a flight path (update) indication and the old flight path, the source gNB should forward the flight path (update) indication to the target gNB during the handover procedure.

- Reference

[1] R2-2306543 Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning