3GPP TSG-RAN WG2 Meeting #123	                                               R2-2308706

Toulouse, France, August 21-25, 2023

Agenda item:		7.7.3

Title:			Discussion on Network Verified UE Location

Source:		TCL

Document for:		Discussion

# 1. Introduction

In Release 17, a work item was carried out to define solutions enabling New Radio and NG-RAN to support Non-Terrestrial Networks (NTN). As part of Release 18, a new work item [1] is proposed to define enhancements aspects of NR NTN in terms of coverage, service continuity and UE location verification by the network to enable selection of an appropriate core network for UE attachment and/or UE service access in accordance with regional regulatory requirements applicable to specific UE location. Regarding network verified UE location, the work item objective was to

- Mandate the network operator to cross check the UE location reported by the UE accessing 3GPP network via satellite.

- Prioritize the specification of necessary enhancements to multi-RTT or DL-TDOA positioning methods to support the network verified UE location in NTN assuming a single satellite in view of UE at time.

- Reuse of the RAT only dependent positioning framework based on RAN1 conclusions (i.e., to avoid malicious UE from faking PLMN selection).

- Verify UE location with a target accuracy similar to terrestrial network macro cell size (i.e., 5-10 km granularity) taking into account take into account the mirror-image ambiguity issue.

During the RAN1#110 meeting, it was agreed that time-based positioning methods such as multi-RTT and DL/UL-TDOA positioning methods to be considered as starting point for the study on network verified UE location. And during RAN2#120,121 and meetings the following agreement have been made:

Agreements

- From RAN2 point of view, assuming the NW may allow the UEs access to services before verifying the UE reported location, the latency of the NW verification can be handled by the NW.).

- RAN2 agrees the re-use of the LCS framework of the LMF for the network verification of UE reported location information in NTN.

- RAN2 will work on the details of radio protocol aspects of the verification procedure based on the solution investigated by RAN1.

- RAN2 will work on a solution that ensures that location verification can be completed within a period of approximately 1 minute maximum and 30 seconds preferably.

- For network verified UE location, the verification procedure can only be triggered by the CN.

- Network initiated verification procedure can be triggered by the NW when the UE is in RRC Connected. FFS whether the NTN UE can perform/report measurements also when in Inactive state.

- RAN2 will not specify an AS mechanism to prevent UEs not supporting the required RAT dependent positioning methods to access the network.

- RAN2 assumes that in general the mirror point issue can be resolved by properly configuring neighbor cell measurement to UE, for example, measurement of two neighbor cells in the opposite side of a satellite beam. FFS if there are any cases that require anything in the spec

- NTN UE doesn’t support positioning measurement and report in RRC INACTIVE

- In order to resolve the mirror point ambiguity issue, the network relies on the legacy signaling and procedure to configure NTN UE to measure and report neighbor cells or reference signals/beams. No spec changes to radio interface are needed from RAN2 perspective. Unclear if changes are needed to other interfaces, NRPPa protocol (RAN2 will no longer discuss this

In this contribution, we discuss the feasibility of time-based positioning methods for UE location verification taking into account the timing measurement error, the mirror-image ambiguity and the above RAN2 progress.

# 2. Discussion

Time-based positioning methods make use of the measured positioning signal timing difference or the time difference of arrival/departure of downlink/uplink signals received from/to different Transmission-Reception Points (TRPs) (i.e. a reference gNB and neighboring gNBs) at the same time. The resulting measurements are used along with other configuration information are used to locate the UE in relation to the neighboring TRPs. In case of single satellite in view [as per WID requirement], Time-based positioning methods require multiple satellites in view for collecting the positioning signal timing difference measurement for UE location verification due to the fact that coverage form multiples at the same time is hard to achieve in NTN. To address the first issue related to the requirement of multiple satellites in view for measuring the time difference a geometry of a virtual satellite at different time instance can be applied in NTN as given in (Figure 1).

Figure 2: Timing difference Measurement Error for Multi/RRT DL-TDOA Positioning Methods

However, even with multiple satellites in view discussed above still time-based positioning methods cannot address NTN UE location verification requirements as per WID requirement, because of many issues such as: 1) the timing difference measurement estimation error 2) UE Image-mirror ambiguity issue and 3) Lack of signaling enhancement for addressing these issues. The following section of this report is aiming a discussing these issues in details.

## 2.1 Timing Difference Estimation Error Issue

Time-based positioning requires very tight synchronization between UE and satellite, which is hard to achieve due to the satellite, and possibly the UE movements, the synchronization error and/or the clock drift between the UE and satellite. This could lead to a positioning error exceeding the required 5-10 km as shown in Figure 1 and Table 1, [2, 3, 4, and 5]). To address this issue, timing error correction information such as UE MAC Layer Time advance and/or UE L1 time Rx-Tx time-difference measurement error of the positioning measurement signal can be exchanged between the UE and satellite/gNB node to help correcting the error on the estimation of timing difference and reduce the positioning error.

Table 1 UE position uncertainty area size under time difference error multi-RRT

|  | Resulting UE position uncertainty area (km) |  |  |
| --- | --- | --- | --- |
|  | RTT error = 300ns | RTT error = 100ns | RTT error = 50ns |
| Scenario 1 | 224.7 | 74.9 | 37.4 |
| Scenario 2 | 38.2 | 12.7 | 6.3 |
| Scenario 3 | 24.5 | 8.1 | 4.0 |

Observation 1: Time based positioning methods such as multi-RTT and DL/UL-TDOA cannot to guarantee the targeted accuracy of 5-10km under the consideration of the timing measurement error due to synchronization error and/or the clock drift between the UE and satellite.

Proposal 1: RAN2 discuss exchanging between a UE and a satellite/gNB a timing-error correction information such as UE time advance MAC Layer time advance and/or L1 time Rx-Tx time-difference measurement error to help addressing timing-error issue.

## 2.2 UE Image-mirror Ambiguity Issue

Time based positioning methods cannot resolve the ambiguity issue for a UE in a symmetric cells or beams with the respect to a satellite orbit plane. More specifically, it is very hard to locate the UE falling within the beams or cells that underneath the satellite’s orbital path (orbital plane) due to the mirror image ambiguity issue (Figure 2). Because of this issues it is hard to achieve a positioning accuracy performance with a granularity of 5-10 km as per the work item requirements [See Figure 2 and Table 2 [6, 7]. To address the mirror image ambiguity issues, a satellite orbital trajectories information (e.g., ephemeris parameters) and/or the angular information (e.g., the azimuth and/or elevation, the angle of arrival (AoA) or the direction of arrival (DoA), angle of departure (AoD)) of the uplink/downlink positioning signal can provided to network to help resolving the mirror image ambiguity issue.

Observation 2: Under the consideration of image ambiguity issue, it is very hard for time based positioning methods such as multi-RTT and DL/UL-TDOA achieve a positioning accuracy performance with a granularity of 5-10 km as per the work item requirements.

Figure 2. UE under a beams/cells underneath satellite’s orbital path suffering from mirror image ambiguity

Table 2 Positioning error performance for multi-RTT under consideration of ambiguity issue

|  | CDF=50% | CDF=67% | CDF=80% | CDF=90% | CDF=95% |
| --- | --- | --- | --- | --- | --- |
| δ=1s | 21.84 km | 29.56 km | 37.97 km | 47.00 km | 54.78 km |
| δ=5s | 19.50 km | 27.08 km | 35.56 km | 45.05 km | 53.35 km |
| δ=8s | 6.66 km | 12.32 km | 14.94 km | 20.64 km | 24.87 km |

Regarding mirror image ambiguity issue, it was also agreed during last RAN2#121 meeting, to assume that the issue can be resolved by configuring neighbour cell measurement to UE, for example, measurement of two neighbour cells in the opposite side of a satellite beam [8]. In our view, configuration of neighbour cell measurement to UE could not able to address the ambiguity issue for some cases. For example for the case of moving cell because the cell change can happen frequently it is hard to determine the neighbours’ cells for a given UE in such situation. In addition, such a situation requires that the satellite beam where UE is located to be determined or known by the network for a given UE. Therefore, resolving the ambiguity issue may require combining reporting neighbor cells or reference signals/beams with other measurements such as ephemeris parameters and or angular information of the positioning reference signal.

Figure 3: Satellite beams in which UE can be located at a given time measurement instance

During RAN1#112 and RAN2#113 it was agreed meetings, to consider either options one or 2 form the following options for addressing this issue. Whereas, in RAN1#122 it was agreed that in order to resolve the mirror point ambiguity issue, the network relies on the legacy signalling and procedure to configure NTN UE to measure and report neighbour cells or reference signals/beams with no spec changes to radio interface. In our view, the spec changes in RAN2 is mostly associated with which option will be finally agreed in RAN1, therefore we propose to leave the discussion on spec changes as FFS until RAN1 conclusion.

Agreement

Study the following options to resolve the mirror positions ambiguity for multi-RTT positioning

1- Option 1: gNB or LMF implementation to solve the mirror error issue.

FFS: whether there is spec impact

- Option 2: Reuse existing ECID method (e.g. combine UE neighbor measurements to solve the ambiguity between mirror positions), with potential enhancements

- Option 3: NR NTN UE should report the Doppler calculated on the service link

- Option 4: a VSAT UE should report its beam pointing in respect to satellite beam line of sight

- Option 5: Reporting of cell coverage information (e.g. cell footprint and reference point, or antenna pattern) to the LMF

- Option 6: Support and potentially enhance the optional Rel-17 UL-AoA measurements defined for multi-RTT positioningOther solutions are not precluded

Observation 3: Configuration of neighbour cell measurement to UE may not address the ambiguity issue for some cases e.g., the case of moving cell because in such a case the cell change can happen frequently for a given UE. Additionally, such a configuration requires that the satellite beam where UE is located to be determined or known by the network for a given UE.

Based on the above discussion, for addressing UE mirror image ambiguity issue, we propose

Proposal 2: For addressing UE mirror image ambiguity issue, a combined reporting of a neighbour cells or reference signals/beams and other measurements such as ephemeris parameters and/or the angular information of the positioning reference signal maybe required.

Proposal 3: Regarding spec changes for addressing UE mirror image ambiguity issue, wait conclusion and input from RAN1.

# 3. Conclusion

In this contribution, we have discussed the feasibility of time-based positioning methods for UE location verification taking into account the timing measurement error, the mirror-image ambiguity and the latest RAN2 progress. Based on the discussion, we observe that:

Observation 1: Time based positioning methods such as multi-RTT and DL/UL-TDOA cannot to guarantee the targeted accuracy of 5-10km under the consideration of the timing measurement error due to synchronization error and/or the clock drift between the UE and satellite.

Observation 2: Under the consideration of mirror image ambiguity issue, it is very hard for time-based positioning methods such as multi-RTT and DL/UL-TDOA to achieve the positioning accuracy performance with a granularity of 5-10 km as per the work item requirements.

Observation 3: For the ambiguity issue, configuration of neighbour cell measurement to UE may not address the ambiguity for some cases e.g., the case of moving cell because in such a case the cell change can happen frequently and it is hard to determine the neighbours’ cells for a given UE in such situation. Additionally, such a configuration requires that the satellite beam where UE is located to be determined or known by the network for a given UE.

Based on these observations, we proposed the following proposals:

Proposal 1: RAN2 discuss exchanging between a UE and a satellite/gNB a timing-error correction information such as UE time advance MAC Layer time advance and/or L1 time Rx-Tx time-difference measurement error to help addressing timing-error issue.

Proposal 2: For addressing UE mirror image ambiguity issue, a combined reporting of a neighbour cells or reference signals/beams and other measurements such as ephemeris parameters and/or the angular information of the positioning reference signal maybe required.

Proposal 3: Regarding spec changes for addressing UE mirror image ambiguity issue, wait conclusion and input from RAN1.

# References

[1] RP-223534, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, Thales, RAN#98-e

[2] R1-2208389, Discussion on network verified UE location in NR NTN, Thales, RAN1#110-bis-e

[3] R1-2208835, Discussion on network verified UE location for NR NTN, OPPO, RAN1#110-bis-e

[4] R1-2208663, Discussions on network verified UE location for NR NTN, VIVO, RAN1#110-bis-e

[5] R1-2209265, Discussion on the network verified location, Xiaomi, RAN1#110-bis-e

[6] R1-2209751, Network verified UE location for NR NTN, Samsung, and RAN1 #110-bis-e

[7] R1-2208694, Discussion on network verified UE location for NR NTN, ZTE, RAN1#110-bis-e

[8] R2-230xxxx, Report of 3GPP TSG RAN WG2 meeting #121, ETSI MCC

[9] R1-2300001, Final Report of 3GPP TSG RAN WG1 #111, MCC.