3GPP TSG-RAN WG2 Meeting #123                                    R2-2308707

Toulouse, France, August 21-25, 2023

Agenda item:		7.8.2

Source:				Samsung

Title:					Remaining issues on measurement reporting enhancements in NR

UAV

Document for:		Discussion & Decision

1	Introduction

This contribution focuses on resolving leftover issues on enhancement to measurement reports in NR UAV.

2	Discussion

2.1	Content of the measurement report for event H1/2

When event H1 or H2 triggers, RAN2 agreed that network can configure inclusion of location information and/or RSRP/RSRQ measurement results but details are still FFS.

Agreements:

- When event H1 or H2 triggers, the content of the measurement report is configurable by the network (i.e. it can contain UAV UEs height, location information and/or RSRP/RSRQ measurement results). FFS whether UAV UE’s height is mandatorily reported and which parameter/IE is used for height reporting.

We understand that its main motivation is to reduce the size of measurement report. Hence, it is necessary to understand how existing UE includes location information and/or RSRP/RSRQ measurement results in the measurement report.

In the current specification, it is already configurable per report configuration whether detailed location information (e.g. the content of commonLocationInfo) is included in the measurement report as per below [1].

1> if the includeCommonLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of commonLocationInfo of the locationInfo as follows: 2> include the locationTimestamp; 2> include the locationCoordinate, if available; 2> include the velocityEstimate, if available; 2> include the locationError, if available; 2> include the locationSource, if available; 2> if available, include the gnss-TOD-msec,

Observation 1: It is already possible for network to control whether UE's location information is included in the measurement report per report configuration.

Thus, we think that UAV UE sets the content of commonLocationInfo in the measurement report as in legacy when event H1 or event H2 triggers.

Proposal 1: When event H1 or event H2 triggers, location information (e.g. the IE CommonLocationInfo) is included in the measurement report as in legacy.

When measurement reporting procedure is triggered due to any kind of reporting criterion, current UE shall include measurements for each serving cell configured with ServingCellMO (e.g. measResultServingMOList) in the measurement report (see relevant procedure text in clause 5.5.5.1 in TS 38.331 [1] copied below).

1> for each serving cell configured with servingCellMO: 2> if the reportConfig associated with the measId that triggered the measurement reporting includes rsType: 3> if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available: 4> set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report; 2> else: 3> if SSB based serving cell measurements are available: 4> set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB; 3> else if CSI-RS based serving cell measurements are available: 4> set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

Observation 2: When measurement reporting is triggered for any kind of reporting criterion, UE shall include measurements for each serving cell configured with servingCellMO (e.g. measResultServingMOList) in the measurement report.

Since servingCellMO is indicated per each serving cell within the serving cell configuration, we need to discuss whether previous RAN2 agreement intends to allow UAV UE not to include measResultServingMOList in the measurement report when event H1 or H2 triggers. If it is the case, further discussion seems to be required what extent of measurements for each serving cell configured with servingCellMO can be absent i.e. whether SpCell's measurement can be also absent or network indicate explicitly which measured results in serving cell measurement objects can be absent, etc.

Proposal 2: RAN2 to discuss whether network can control inclusion of all or subset of measResultServMOs in the measurement report when event H1 or event H2 triggers. FFS on network configuration details.

2.2	Height-dependent more-than-one configurations in MO

It is still open which MO configuration parameters other than SSB-ToMeasure can be configured with height-dependent more-than-one configurations/values.

Agreements

- For MO configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: SSB-ToMeasure. Details on how to specify is FFS.    FFS on UE behavior on L1 and L3 measurement.

As there have been multiple round of discussion on height-dependent parameters, further optimization should be avoided unless there is a strong need/justification. In this sense, we think that SSB-ToMeasure is enough since it generally allows operators to support efficient beam operation/management (e.g. beam tiling) by configuring the pattern of SSBs per each tailored height range.

Proposal 3: For MO configuration parameters, RAN2 to confirm that only SSB-ToMeasure can be with height-dependent more-than-one configurations/values.

In RAN2#122 meeting, RAN2 agreed to add height-based list of SSB-ToMeasure in MO as follows:

Agreements

- Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

- As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy behaviour applies.

- For UE behavior on L1 and L3 measurement, it is left to UE implementation whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value

Given that the maximum supported altitude in NR UAV is 300m, it is sufficient to configure up to 4 different ranges for height-based SSB-ToMeasure in MO.

Proposal 4: Maximum number of height ranges in height-based list of SSB-ToMeasure in MeasObjectNR is 4.

2.3	Height-dependent more-than-one configurations in MR

Similar to MO configuration parameters, we need to discuss which additional MR configuration parameters can be configured with height-dependent more-than-one configurations/values.

Agreements

- For MR configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: Event A4 threshold and numberoftriggeringcells.  Details on how to specify is FFS (i.e. maybe it can be achieved by combination of events).

Note that numberoftriggeringcells can be set for the events concerning neighbour cells i.e. eventA3, eventA4, and eventA5. Hence, it does not make much sense to restrict network flexibility of configuring height-dependent event A3 (or event A5) threshold on top of height-dependent numberoftriggeringcells.

Proposal 5: For MR configuration parameters, RAN2 to agree that only Event A3/A4/A5 threshold and numberoftriggeringcells can be with height-dependent more-than-one configurations/values.

It is possible for network not to configure a height specific field (e.g. Event A4 threshold or numberoftriggeringcells) for a specific height region. Alike height-specific SSB-ToMeasure, we think that the legacy behaviour should be applied here i.e. applies legacy field if configured.

Proposal 6: As a basic principle, if no height-specific field is configured for a specific height region in MR configuration, the legacy behaviour applies.

2.4	New event types for combined events

RAN2 agreed to introduce new event types to support combination of eventAx and event Hx, as per below.

Agreements

- New event types will be introduced on the combination of event Ax and event Hx, at least for event A4 + event H1/H2. FFS for other event Ax + event H1/H2. FFS on details, e.g. whether to include one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event, how to configure height-dependent numberOfTriggeringCells, etc.    This will be applied to all height dependent MR parameters.

For the first FFS part, we understand that the main intent of supporting such event combination is to detect airborne interference characteristics in accordance with height range. Thus, it is beneficial to support both event A3 + event H1/2 and event A5 + event H1/2 via new event types.

Proposal 7: Event A3 + event H1/2 and event A5 + event H1/2 are also supported by introducing new event types.

Our understanding is that one height range can be expressed by one height region 1) with upper bound, 2) with lower bound or 3) with upper and lower bound. It means that both Alt.1 and Alt.2 can be specified depending on network configuration with height range

- Alt.1: CombinedEventX (A4+H1): Neighbour becomes better than absolute threshold AND the Aerial UE height is above a threshold

- Alt. 2: CombinedEventX (A4+H1+H2): Neighbour becomes better than absolute threshold AND the Aerial UE height is above a threshold 1 AND the Aerial UE height is below a threshold 2

Proposal 8: Introduce a height range in new event types i.e. it can be height region 1) with upper bound, 2) with lower bound, or 3) with upper and lower bound.

In general, we should aim for a common framework for height-dependent more-than-one configurations and new event types for combined events. Since we agreed to add height-based list of SSB-ToMeasure in MO i.e. each entry in the list is associated with height range and hysteresis. In this sense, one may wonder whether more than one height range can be configured in new event type for combined events i.e. different event A4 thresholds associated with different height ranges.

Proposal 9: RAN2 to discuss whether more than one height range can be configured in new event type for combined events.

2.5	User consent for location reporting

RAN2 could not reach a consensus whether user conset is needed for location reporting in NR UAV as per below agreement.

Agreements

- Use LTE principle as a baseline, introduce similar event H1 (aerial UE height become higher than threshold) and H2 (aerial UE height become lower than threshold.  FFS if further NR enhancements are needed.  FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/benefit of the scaling and how)

FFS how to limit excessive measurements and measurement reporting

FFS if user consent is needed for location reporting in CONNECTED

FFS study the vertical movement and associated mobility for UAV UEs

Since there was lengthy discussion on the necessity of user consent during Rel-17 NTN, one may consider that RAN2 should indicate to SA3 eariler whether there are potential issues in NR UAV. But we understand that the conclusion in Rel-17 NTN was that NW implementation (e.g. proprietary mechanims or provisonal means) can handle potential issues of user consent appropriately, and standard solutions will be studied in Rel-18 SI. In our understanding, the conclusion of Rel-17 NTN can be applied to NR UAV, if requested by other WGs (e.g. SA2 and SA3). In addition, location reporting will not be much different between LTE UAV and NR UAV. As there is no requirement on user consent for aerial UE's location reporting in LTE, it seems strange to have it solely in NR. Given the limited TU for this WI, we suggest to follow LTE design unless there is a request from other WGs e.g. SA2 and SA3.

Proposal 10: RAN2 not to study user consent for UAV use case in Rel-18, unless there is a request from other WGs (e.g. SA2 and SA3).

3	Conclusion

Content of the measurement report for event H1/2:

Proposal 1: When event H1 or event H2 triggers, location information (e.g. the IE CommonLocationInfo) is included in the measurement report as in legacy.

Proposal 2: RAN2 to discuss whether network can control inclusion of all or subset of measResultServMOs in the measurement report when event H1 or event H2 triggers. FFS on network configuration details.

Height-dependent more-than-one configurations in MO:

Proposal 3: For MO configuration parameters, RAN2 to confirm that only SSB-ToMeasure can be with height-dependent more-than-one configurations/values.

Proposal 4: Maximum number of height ranges in height-based list of SSB-ToMeasure in MeasObjectNR is 4.

Height-dependent more-than-one configurations in MR:

Proposal 5: For MR configuration parameters, RAN2 to agree that only Event A3/A4/A5 threshold and numberoftriggeringcells can be with height-dependent more-than-one configurations/values.

Proposal 6: As a basic principle, if no height-specific field is configured for a specific height region in MR configuration, the legacy behaviour applies.

New event types for combined events:

Proposal 7: Event A3 + event H1/2 and event A5 + event H1/2 are also supported by introducing new event types.

Proposal 8: Introduce a height range in new event types i.e. it can be height region 1) with upper bound, 2) with lower bound, or 3) with upper and lower bound.

Proposal 9: RAN2 to discuss whether more than one height range can be configured in new event type for combined events.

User consent for location reporting:

Proposal 10: RAN2 not to study user consent for UAV use case in Rel-18, unless there is a request from other WGs (e.g. SA2 and SA3).

4	Reference

[1] 3GPP TS 38.331 v17.4.0 Radio Resource Control (RRC) protocol specification