3GPP TSG-RAN WG2 Meeting #123	R2-2308715

# Toulouse, France, August 21-25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1654 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MAC corrections for Sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In 5.22.1.1, there is a misplaced ‘if configured’ in resource selection procedure. - In NOTE 3A in 5.22.1.3.1: “When determining Sidelink transmission information” is missing for determining priority of Sidelink Inter-UE Coordination Request MAC CE and the usage of the priority to which the NOTE is referring is unclear. - In NOTE 3A in 5.22.1.3.1: The parameter sl-PriorityRequest should be used when determining the priority value of Sidelink Inter-UE Coordination Request MAC CE, and priority value of the MAC CE is the same as that of a TB to be transmitted by the UE, if sl-PriorityRequest is not configured. However, current note has the UE determine the priority value of the request MAC CE based on configuration of sl-PriorityCoordInfoCondition-r17 and should be fixed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Fixed the wording of the sentence so that the meaining of configured SL active time is clear. - Clarify that the priority for Sidelink Inter-UE Coordination Request MAC CE is for determining Sidelink transmission information. - Changed the referenced parameter to sl-PriorityRequest when determining how to set the priority for Sidelink Inter-UE Coordination Request MAC CE. Impact analysis Impacted functionality: Sidelink This CR is mandatory for sidelink functionality to avoid issue in resource selection and to avoid wrong setting of the priority value for Sidelink Inter-UE Coordination Request MAC CE. Inter-operability: - If the UE is implemented according to this CR but the network is not, there will be different understanding of priority value between the UE and the network for Sidelink Inter-UE Coordination Request MAC CE if sl-PriorityRequest is configured. - If the network is implemented according to this CR but the UE is not, there will be different understanding of priority value between the UE and the network for Sidelink Inter-UE Coordination Request MAC CE if sl-PriorityRequest is configured. - If one UE is implemented according to this CR while the other UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. The sentence in resource selection cannot be interpreted correctly. 2. The usage of the priority of Sidelink Inter-UE Coordination Request MAC CE to which the NOTE is referring is unclear. 3. The UE does not have correct priorty value set for Sidelink Inter-UE Coordination Request MAC CE if sl-PriorityRequest is configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.1, 5.22.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### First change

#### 5.22.1.1	SL Grant reception and SCI transmission

[…]

3>	if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is not configured by RRC:

4>	if transmission based on random selection is configured by upper layers:

5>	randomly select the time and frequency resources for one transmission opportunity from the resources pool which occur within the SL DRX Active time if configured as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX Active time above, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and the latency requirement of the triggered SL CSI reporting.

4>	else:

5>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] which occur within the SL DRX Active time if configured as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX Active time above, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL-CSI reporting.

[…]

##### Second change and third change

##### 5.22.1.3.1	Sidelink HARQ Entity

[…]

NOTE 3A:	When determining Sidelink transmission information, the priority of the Sidelink Inter-UE Coordination Information MAC CE is the value configured in RRC parameters sl-PriorityCoordInfoCondition when triggered by a condition, or sl-PriorityCoordInfoExplicit when triggered by an explicit request. When determining Sidelink transmission information, the priority of the Sidelink Inter-UE Coordination Request MAC CE is the value configured in RRC parameter sl-PriorityRequest. When determining Sidelink transmission information, the priority of the Sidelink Inter-UE Coordination Information MAC CE is the value indicated in Priority field in the Sidelink Inter-UE Coordination Request MAC CE provided by the UE when triggered by an explicit request, if sl-PriorityCoordInfoExplicit-r17 is not configured. When determining Sidelink transmission information for performing sensing and candidate resource selections in PHY, the priority value of the Sidelink Inter-UE Coordination Information MAC CE triggered under a condition is up to UE implementation, if sl-PriorityCoordInfoCondition-r17 is not configured. When determining Sidelink transmission information for performing sensing and candidate resource selections in PHY, the priority value of Sidelink Inter-UE Coordination Request MAC CE is the same as that of a TB to be transmitted by the UE, if sl-PriorityRequest is not configured.

[…]

##### End of change