3GPP TSG-RAN WG2 Meeting #123	R2-2308718

Toulouse, France, 21st August – 25th August 2023

Agenda Item:		7.7.4.1.2

Source:				ASUSTeK

Title:		Discussion on description of movingReferenceLocation

Document for: 	Discussion and Decision

- Introduction

In previous RAN2 meetings, it has been discussed how to handle cell reference location for earth-moving cell, e.g. for the purpose of neighbour cell measurements. Several agreements has been made for NR NTN and IoT NTN separately. In this contribution, we discuss the field description of earth-moving cell reference location in current running RRC CR [2].

- Discussion

For Rel-18 NR NTN, the following agreements are made in previous meetings:

- In RAN2 #121:

  - A serving cell reference location and a distance threshold/radius will be broadcast for earth-moving cell.

  - For cell selection/reselection, location-based measurement initiation is supported in earth-moving cell.

- In RAN2 #121bis:

  - RAN2 understands that for earth-moving cell reselection, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time;

  - For earth-moving cell, new IE is introduced to indicate the reference location of serving cell;

  - For cell (re)selection in earth-moving system, a distance threshold is introduced for location-based measurement initiation, which reuses distanceThresh in SIB19.

- In RAN2 #122:

  - Re-use epochTime-r17 in ntn-Config IE to provide the time reference for an Earth moving cell reference location.

Based on the agreements in NTN, for an earth-moving cell, the UE could receive a cell reference location and a distance threshold (i.e. distanceThresh) via SIB19. And the UE could perform measurements for cell re-selection based on the cell reference location and distance threshold. The cell reference location is the earth-moving cell’s reference location at epoch time (i.e. epochTime). The UE could derive the trajectory of the earth-moving cell based on serving satellite ephemeris and the epoch time.

The current running CR for TS 38.331 [2] is quoted as below. Based on the running RRC CR, the UE receives movingReferenceLocation from SIB19 as the new IE to indicate the reference location of the serving earth-moving cell. The movingReferenceLocation is used in location-based measurement initiation as implemented in running CR for TS 38.304 [1]. And the time reference of movingReferenceLocation is indicated by epochTime of the serving cell. However, the following agreement in RAN2 #121bis is not reflected in the running CRs: the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime.

Observation 1: The agreement that the UE could derive trajectory of reference location for earth-moving cell based on assistant information is not captured in current running CR of TS 38.304 [1] or TS 38.331 [2].

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]],

[[

movingReferenceLocation-r18              FFS                                             OPTIONAL        -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

SIB19 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m.

movingReferenceLocation Reference location of the serving cell of an NTN Earth moving system at a time reference. It is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. The time reference of this field is indicated by epochTime in ntn-Config of the serving cell.

ntn-Config Provides parameters needed for the UE to access NR via NTN access such as Ephemeris data, common TA parameters, k_offset, validity duration for UL sync information and epoch.

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList and all elements of ntn-NeighCellConfigListExt. If ntn-Config is absent for an entry in ntn-NeighCellConfigListExt, the ntn-Config provided in the entry at the same position in ntn-NeighCellConfigList applies. Network provides ntn-Config for the first entry of ntn-NeighCellConfigList. If the ntn-Config is absent for any other entry in ntn-NeighCellConfigList, the ntn-Config provided in the previous entry in ntn-NeighCellConfigList applies.

referenceLocation Reference location of the serving cell provided via NTN quasi-Earth fixed system and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20].

t-Service Indicates the time information on when a cell provided via NTN quasi-Earth fixed system is going to stop serving the area it is currently covering. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field.

On the other hand, for Rel-18 IoT NTN, the following agreements are made in previous meetings:

- In RAN2 #121:

  - Location-based connected mode measurement initiation is supported in earth-moving cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for earth-moving cell.

- In RAN2 #121bis:

  - For moving cell, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (like in NR-NTN)

  - For eMTC NTN, for moving cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN).

- In RAN2 #122:

  - R18 location and time based trigger for measurements (for connected mode and for idle) apply to both NB-IoT and eMTC.

Based on the agreements in IoT NTN, the UE could receive a cell reference location and a distance threshold via SIB31. And the UE could perform measurements in both RRC connected state and idle state based on the cell reference location and distance threshold. As in NR NTN, the cell reference location for an earth-moving cell is the cell’s reference location at epoch time (i.e. epochTime) and the UE could derive the trajectory of the cell based on serving satellite ephemeris and the epoch time.

In the running CR for TS 36.331 [3], the field description for the distance threshold (i.e. distanceThresh) and cell reference location (i.e. referenceLocation) is quoted as below. Based on the running RRC CR, the cell reference location is used in location-based measurement initiation in RRC_IDLE and RRC_CONNECTED. If the cell reference location is configured for earth-moving cell, it corresponds to epochTime, and the UE derives the real-time reference location based on the serving satellite ephemeris.

SystemInformationBlockType31 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED. Each step represents 50m.

referenceLocation Reference location of the NTN quasi-earth fixed cell or earth moving cell, used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED. If configured by an earth moving cell, the broadcast reference location corresponds to the epoch time, and the UE derives the real-time reference location based on the serving satellite ephemeris.

Observation 2: The same agreement was made in IoT NTN and has been captured in the current running CR of TS 36.331 [3].

Since the similar behavior is applied in both NR NTN and IoT NTN, the description in TS 36.331 could be simply added to TS 38.331. The text proposal is provided as below.

SIB19 field descriptions

movingReferenceLocation Reference location of the serving cell of an NTN Earth moving system at a time reference. It is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. The time reference of this field is indicated by epochTime in ntn-Config of the serving cell. The UE derives the real-time reference location based on the serving satellite ephemeris.

Proposal 1: Capture the agreement by adding “The UE derives the real-time reference location based on the serving satellite ephemeris” in the field description of movingReferenceLocation in NR running RRC CR.

- Conclusion

In this contribution, we discuss the earth-moving cell reference location in running CRs and have following observations and proposal:

Observation 1: The agreement that the UE could derive trajectory of reference location for earth-moving cell based on assistant information is not captured in current running CR of TS 38.304 [1] or TS 38.331 [2].

Observation 2: The same agreement was made in IoT NTN and has been captured in the current running CR of TS 36.331 [3].

Proposal 1: Capture the agreement by adding “The UE derives the real-time reference location based on the serving satellite ephemeris” in the field description of movingReferenceLocation in NR running RRC CR.

- Reference

- R2-2306961, “Stage 3 running 38.304 CR for NTN.”

- R2-2306964, “Stage 3 Running RRC CR for NR NTN Rel-18.”

- R2-2306954, “Running CR - Introduction of IoT NTN enhancements.”