3GPP TSG-RAN WG2 Meeting #123                 	R2-2308740

Toulouse, FR, 21th – 25th Aug, 2023

Agenda Item:	7.3.5

Source: 	vivo

Title:         	Conditional handover enhancement for network energy saving

Document for: 	Discussion and Decision

- Introduction

One objective of the Rel-18 network energy saving WID [1] is provided below:

- Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2]

For this objective, RAN2 has made the below agreements so far.

Agreements: - Study whether CHO enhancements are needed for the purpose of turning off the cell - Continue discussing CHO in the context of different NES techniques. - RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”. FFS further details - For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode” - As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration. FFS what trigger is used for execution of CHO - We will have a CHO solution that considers NES mode of at least source cell. - We can have a specific NES CHO execution condition based on source cell NES mode. FFS how the UE determines is in NES mode. FFS on how this is achieved in RRC - We will not introduce new L1 signalling for the purpose of CHO - Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario. We will study the time based mechanism

This paper will discuss the above FFSs and the remaining open issues.

- Discussion

2.1 Scenario 1: Source cell is in NES mode

How does the UE determine the source cell is in NES mode?

According to the agreements, the UE will use NES specific CHO execution condition based on source cell NES mode. The UE may determine the source cell is in NES mode via the following ways:

- Option 1: based on cell DTX/DRX activation command or the cell DTX/DRX non-active period.

- Option 2: based on configured T1 event.

- Option 3: based on explicit signalling sent by the source cell.

From our understanding, the UE can work in cell DTX/DRX mode, otherwise, the network should turn off this cell instead of activating cell DTX/DRX. It is true that the UE may have performance loss when the cell activates cell DTX/DRX mode, however, considering the neighbouring cell could also activate cell DTX/DRX, ping-pong handover would occur which is also unexpected. For option 2, it is difficult for the network to configure a proper T1 event as the network may decide to turn off the source cell at any time based on the traffic fluctuation. Someone may argue that the network may observe the low network traffic for a long period of time, and then decide to configure T1 event. However, if this is true, it is most likely that the network will handover all the UEs to the neighbouring cells immediately, not configuring T1 event. So, we prefer to use option 3 as it provides more flexibility of the network to determine the sleeping time. Thus, we have the following proposal:

Proposal 1: The UE determines the source cell is in NES mode based on the explicit signalling sent by the network.

How is this achieved in RRC?

UE can use specific NES CHO execution condition based on source cell NES mode. This can be achieved by configuring A4 event or A3/5 offsets in CHO configuration. When receiving the A4 event or A3/5 offsets in CHO configuration, the UE stores it and only use A4 event or A3/5 offsets as the CHO execution condition when the UE determines the source cell is in NES mode. In some scenario, the coverage of target cell is overlaid by the source cell, so the UE does not need to evaluate the signal quality of the target cell. So, A4 event or A3/5 offsets  can be optionally configured. Specifically, if A4 event or A3/5 offsets are not configured, the explicit signaling is used to trigger CHO execution. If A4 event or A3/5 offsets are configured, the explicit signaling is used to trigger CHO evaluation based on A4 event or A3/5 events with A3/5 offsets. Thus, the below proposals are given:

Proposal 2:  an A4 event or an offset for A3/5 event can be configured, which are to be used by the UE when it determines the source cell is in NES mode.

Proposal 3: If no NES specific CHO execution condition (i.e., A3/4/5) is configured, the explicit signaling is used to trigger CHO execution, otherwise, trigger CHO evaluation based on A4 event or A3/5 event with an offset.

The signalling of the explicit trigger for CHO:

In the last RAN2 meeting, L1 signalling is precluded for CHO enhancement. The following options for sending the explicit trigger can be considered:

- Option 1: the explicit trigger is sent via dedicated MAC CE.

- Option 2: the explicit trigger is included in short message which is sent on common POs.

From our understanding, both options can work. The benefit of option 1 is low latency, while option 2 can reduce some signaling overhead. Thus, we have the following proposal:

Proposal 4: For the explicit signalling for CHO, RAN2 to consider the below options:

- Dedicated MAC CE;

- Short message sent on common PO.

2.2 Scenario 2: Target cell is in NES mode

In this scenario, when CHO is configured at the UE, and if multiple candidate target cells satisfy the execution condition, currently it is up to UE implementation to select one of them to perform handover. If network energy saving technique(s) is implemented, it is possible that the satisfying candidate target cell may plan to enter NES mode. In this case, if the UE selects the target cells via UE implementation, the UE may handover to the NES cell. From the UE’s perspective, its performance may be impacted if it is switched to this NES cell. From the network’s perspective, the network may expect the UEs to select the NES cell only when there is no available normal cell.

Figure 1: RRC re-establishment to the NES cells

In the last RAN2 meeting, some companies think the network should not configure NES cells as candidate cells in CHO. However, we think this means network energy saving and UE fast mobility cannot be achieved at the same time. The Figure 1 shows an example. If the cell 1 activates Cell DTX/DRX, it is possible that cell 1 is the best target cell of one UE at the same time. However, if the network does not configure cell 1 to this UE because this cell enters NES mode, then the UE will suffer from RLF and perform RRC re-establishment. Since the UE can still access to the cell 1 which is in cell DTX / DRX operation, the handover results are the same but longer delay occurs at the UE if the cell 1 is not configured as an candidate cell in the CHO configuration.

So, the NES cells can be candidate cells of the UE and the UE should be able to know the NES information of the candidate cells for better selection. There are two candidate options for UE to select target cells:

- Option 1: the NES mode/priority/NES capability of candidate cells are configured by the network, and it is up to the UE which one to select;

- Option 2: The NES mode/priority/NES capability of candidate cells are configured by the network, and the UE selects the target cell based on some specified rules.

From our understanding, option 1 can provide more flexibility for the UE and has minimum spec impact, and hence preferred. Thus, we have the following proposal:

Proposal 5: The NES mode/priority/NES capability of candidate cells are configured by the network, and it is up to the UE which one to select based on the NES mode and the signal quality of the candidate cells.

- Conclusion

In this paper, the following observation and proposals are given:

For scenario 1 of CHO enhancement:

Proposal 1: The UE determines the source cell is in NES mode based on the explicit signalling sent by the network.

Proposal 2:  an A4 event or an offset for A3/5 event can be configured, which are to be used by the UE when it determines the source cell is in NES mode.

Proposal 3: If no NES specific CHO execution condition (i.e., A3/4/5) is configured, the explicit signaling is used to trigger CHO execution, otherwise, trigger CHO evaluation based on A4 event or A3/5 event with an offset.

Proposal 4: For the explicit signalling for CHO, RAN2 to consider the below options:

- Dedicated MAC CE;

- Short message sent on common PO.

For scenario 2 of CHO enhancement:

Proposal 5: The NES mode/priority/NES capability of candidate cells are configured by the network, and it is up to the UE which one to select based on the NES mode and the signal quality of the candidate cells.

- References

[1] RP-223540, Network energy savings for NR, Huawei.