3GPP TSG-RAN WG2 Meeting #123	R2-2308749

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.9.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On remaining issues on multipath SL relay

WID/SID:	NR_SL_relay_enh-Core – Release 18

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#122, RAN2 discussed the multi-path sidelink relay issues based on the AT email discussion summarized in [R2-2306672]. This contribution presents our views on the FFS parts and remaining aspects.

The agreements from the last meeting are captured as below:

Agreements:

For Scenario-1/2, PDCP duplication of DRB is controlled by legacy Duplication Activation/Deactivation MAC CE and Duplication RLC Activation/Deactivation MAC CE delivered via direct path.

For Scenario-1/2, RRC sets the initial state of PDCP duplication for split SRB/DRB as in legacy.

For Scenario-1/2, when reporting direct-path failure via indirect-path, use MCGFailureInformation message. FFS on whether additional IE needs to be introduced.

For Scenario-1/2, if MCGFailureInformation is agreed for direct path failure recovery in P5, reuse T316 timer for the direct path failure recovery.

For Scenario-1/2, confirm the WA that: for a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/INACTIVE. No spec impact is foreseen.

For Scenario-1/2, not pursue remote UE notifying network upon reception of notification message indicating relay UE handover. FFS whether rely on network to release configuration of relay UE at remote UE before relay UE handover, or rely on remote UE to suspend the indirect path upon reception of notification message indicating relay UE handover.

Agreement:

For Scenario-1/2, optionally configure UL data split threshold for split DRB. Usage of the threshold follows legacy behavior.

Agreement:

For Scenario-1/2, no specification effort to handle the case when the relay UE moves to RRC_IDLE following expiry of dataInactivityTimer, i.e., not pursue relay UE notifying remote UE, and remote UE notifying network.

Agreements:

For Scenario-1, use T304-like timer for direct path addition and change. FFS on expiry behavior.

For Scenario-1, use T420-like timer for indirect path addition and change. FFS on stop condition and expiry behavior.

FFS if these two timers are new or reuse the existing timers.

WA: For scenario 2, remote-UE reports the RRC_CONNECTED relay-UE C-RNTI and serving cell ID (e.g., NCGI) for indirect path addition.

LS to SA3 to check if there is any security concern with the WA.

WA: For Scenario-1/2, MP remote UE is configured with a single cell group, i.e., MCG, for the direct path, and SL configuration, for the indirect path.

For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path.  This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure.

Agreements:

For scenario 1, SRB1 and SRB2 are not decoupled in terms of support of non-split SRB on indirect path; i.e., if SRB1 can be supported on indirect path, so can SRB2.

Agreement:

For Scenario-1, mode-1 scheduling for remote UE is supported at least for intra-DU case, with the SR/BSR and grant sent on the direct path; whether it is supported for inter-DU case is up to R3, but R2 do not intend to make specification changes to support this case, and for specification purposes RAN2 intend to model it as a single MAC entity at the UE. LS to R3 to notify this conclusion, with “take into account” action.

# 2	Discussions

## 2.1	RLF report

RAN2 discussed primary path for split SRB1/2 and agreed the followings, which requires further discussion on how to manage the primary path when direct path fails.

For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path.  This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure.

In RAN2#121, RAN2 agreed that, at least for split SRB1, if SRB1 is available on the indirect path not suspended, the UE reports the Uu-RLF via the indirect path. The only question is whether to switch the primary path to the indirect path or not for the agreed operation.

In our view, there is no technical issue for switching the primary path to the indirect path because the direct path is already failed and will be suspended. It would only be complicated if the failure report is transmitted over the non-primary indirect path even when duplication is not activated because it should be regarded as an exceptional case, i.e., to use the non-primary path even when duplication is not activated for split SRB1.

In DC, furthermore, the UE changes the primary path to SCG for failure report when detecting a primary path failure on MCG and we see no motivation to deviate from the DC in this case.

For scenario 2, it might be considered safer to perform an RRC connection re-establishment instead of changing the primary path to an indirect path and sending a failure report over the indirect path. However, if duplication is being used for split SRB1, then regardless of the situation, RRC messages are sent through the indirect path. Therefore, there shouldn’t be a critical disadvantage in changing the primary path to send a failure report and having a common mechanism for scenario 1 and 2 might be preferred. However, it can be considered not safe to use the indirect path for failure report when the direct path is not working given that RAN2 didn’t allow non-split indirect SRB for scenario 2.

Proposal 1: For scenario 1, the UE switches the primary path from the direct path to the indirect path upon detection of failure on the direct path. FFS for scenario 2.

It was agreed in RAN2#122 that MCGFailureInformation message is used for direct path failure report, and it is FFS whether additional IE needs to be introduced. Originally, MCGFailureInformation is to report the MCG failure to the SgNB in DC. One difference could be that it is now transmitted to the MgNB in SL relay. As the message name tells, it seems not necessary to further indicate the path where the failure is detected. Meanwhile, when the failure report is transmitted to the gNB, gNB may decide to recover the failed path so that MP relay operation is continued or to revert to a single path operation. To help the gNB decision, it would be beneficial for the gNB to acquire the latest buffer size information. One may think periodic transmission is sufficient, but it may not be sufficient to acquire the latest buffer information at an appropriate time while it is not possible to send the BSR over the indirect path when direct path is a problem. Therefore, it is suggested to include the buffer size information in the RLF report.

Proposal 2: Buffer size information is included when reporting the RLF.

In principle, the failure report is for fast link recovery and the the problematic path will be released eventually, in which case failure report is not necessary especially if the UE can still use the alive path to meet its service requirement even without the need of failed path. In another scenario, if fast link recovery is not possible e.g., due to lack of available relay UEs for indirect path or poor channel condition in indirect path, the failure report is not useful either. Thus, we see some conditions where SL RLF is not necessary even if RLF is detected on one path of split SRB1. Skipping RLF report based on the pre-configured condition would allow not performing unnecessary failure recovery procedure, which includes a couple of steps that may interfere continuous data transmission/reception on the alive path with no benefit.

Proposal 3: RAN2 discuss if the gNB can configure a condition not to send the RLF report even if the UE detects the RLF on one path while another path can be used for transmission of the RLF report.

## 2.2	Support of non-split SRB on the indirect path

It is still not determined whether to support non-split SRB on the indirect path.

Support of non-split SRB on the indirect path is motivated by the possibility of better link quality over the indirect path. However, it was argued that there is no such case due to the intrinsic limitations of PC5 link. In our view, there may be a case that indirect path is better than direct path in terms of channel quality, but it is uncertain how dynamically the SRB configuration can be changed, e.g., from non-split direct SRB to non-split indirect SRB and vice versa, based on the channel quality.

Given that RAN2 agreed to set the primary path always to the direct path for SRB with an assumption that the direct path is better in general, it would be simpler to apply the same rule, i.e., no support of non-split indirect SRB.

Proposal 4: Non-split SRB on the indirect path is not supported.

## 2.3	Primary path for split DRB

In RAN2#122, it was agreed that primary path of split SRB is always set to the direct path, but it is not clear for the split DRB.

It was understood that the direct path would ensure more reliable transmission in general, hence, RAN2 agreed to set the primary path for split SRB to the direct path to ensure reliable transmission of SRB. For DRB, however, data rate as well as reliability is an important KPI. In addition, having a flexibility to distribute traffic through the indirect path can be beneficial from the network perspective. For instance, the network may want to steer the low priority data over the indirect first, i.e., until the buffer size is below a threshold, while using the direct path first for the high priority data, which can be achieved by setting the primary path of low priority RB to the indirect path while setting the primary path of high priority RB to the direct path. Therefore, we suggest allowing primary path to be set to either the direct or the indirect path for split DRB.

Proposal 5: For split DRB, primary path can be either the direct or the indirect path.

## 2.4	Packet duplication

RAN2 agreed that, for Scenario-1/2, PDCP duplication of DRB is controlled by legacy Duplication Activation/Deactivation MAC CE and Duplication RLC Activation/Deactivation MAC CE and they are delivered via direct path. In addition, RAN2 also agreed that for Scenario-1/2, MP remote UE is configured with a single cell group, i.e., MCG, for the direct path, and SL configuration, for the indirect path.

The current Duplication A/D MAC CE and Duplication RLC A/D MAC CE includes Di field and RLCi field, respectively, which indicate activation or deactivation status for an RLC entity, and there can be a couple of issues.

For Duplication RLC Activation/Deactivation MAC CE, the RLCi field indicates RLC entity i where i is ascending order of logical channel ID of secondary RLC entities in the order of MCG and SCG. As both paths belong to MCG but logical channel ID is separate for Uu channel and PC5 channel, we need to discuss whether the RLC entity i indicates RLC entity i in ascending order of logical channel ID, what if the LCID is the same for Uu channel and PC5 channel, or whether to have a network restriction that the same logical channel ID for Uu and SL are not used.

Proposal 6: For Duplication RLC Activation/Deactivation MAC CE, RAN2 discuss which RLC entity the RLCi field indicates activation/deactivation.

For Duplication Activation/Deactivation MAC CE, when a MAC entity receives this MAC CE, it applies the activation/deactivation for the RLC entity associated with this MAC entity. This is because, in DC, MgNB and SgNB send the Duplication A/D MAC CE independently and the UE maintains two separate MAC entities. However, in SL MP relay, there is one scheduler sending this MAC CE while the UE maintains one MAC entity for Uu and SL. Thus, the interpretation may need to be changed. For example, Di field indicates activation deactivation status of PDCP duplication of secondary RLC entity associated with DRB i, which is corresponding to the non-primary path. It means that packet duplication through the primary RLC entity is not able to be turned off but only the duplication over the secondary RLC entity is turned on and off.

Proposal 7: When an MP remote UE receives a Duplication Activation/Deactivation MAC CE, the MP remote UE activates or deactivates the secondary RLC entity, i.e., RLC entity corresponding to the non-primary path, and the primary RLC entity, which is an RLC entity for primary path, is not impacted.

Although the MAC CE format is reused, the field description needs to be differentiate for DC and SL MP relay. In this case, it is conventional to define the MAC CE as a new MAC CE by assigning a new LCID. Alternatively, we can just update the field description of the legacy MAC CE while leaving it up to UE’s responsibility to interpret well based on UE’s configuration, i.e., if remote UE, field description for SL relay is applied.

From specification perspective, it would be logical and clean to define them as new MAC CEs while saving one LCID value sometime are considered important.

Proposal 8: For Duplication RLC Activation/Deactivation MAC CE and Duplication Activation/Deactivation MAC CE in SL MP relay, RAN2 discuss whether to assign a new LCID or to update the field description without assigning any LCID.

## 2.5		Impact of Relay UE’s handover and Indirect path release

- For Scenario-1/2, not pursue remote UE notifying network upon reception of notification message indicating relay UE handover. FFS whether rely on network to release configuration of relay UE at remote UE before relay UE handover, or rely on remote UE to suspend the indirect path upon reception of notification message indicating relay UE handover.

In Rel-17 U2N relay, when path switches from indirect to direct, the remote UE is reconfigured first and then the relay UE is released to ensure service continuity.

In Rel-18 multi-path scenario, if the remote UE suspends the indirect path upon relay UE’s handover, it will have impact on the remote UE as the indirect path will not operate fine for a while. To minimize or avoid any interruption to the remote UE, the network is likely to reconfigure the remote UE before relay UE’s handover so that the transmission path for traffic that are supposed to be transmitted over the indirect path, e.g., data from non-split indirect DRB or split DRB with primary path on the indirect path, is safely steered to the direct path. In our view, there is nothing to specify further for this because it is up to network implementation to reconfigure the remote UE before the relay UE’s handover and the remote UE doesn’t need to suspend the indirect path.

Proposal 9: In case of relay UE’s handover, the network releases the configuration of the relay UE at the remote UE before the relay UE’s handover, which requires neither network restriction nor additional UE behaviour to be specified in the specification.

The figure below is signalling procedure for the indirect path release from [POST122][403] email discussion, where the order of step 3 and 4 are FFS.

With the same reason above, we think it makes sense to reconfigure the remote UE first to switch the traffic to the direct path safely before releasing the indirect path.

Proposal 10. When indirect path is released, the network reconfigures the remote UE first and then reconfigure the serving relay UE, i.e., step 4 is prior to step 3 in the figure.

## 2.6		LCG allocation

As RAN2 agreed that both of direct and indirect paths belong to the MCG and one MAC entity in the UE side, there are some stage-3 aspects to discuss.

- Whether the Uu logical channel, i.e., logical channel carrying UL data, and SL logical channel, i.e., logical channel carrying SL data, can belong to the same LCG or not.

- Whether the maximum number of LCG is 8 or 16 in total.

For 1) there may be a problem if the Uu logical channel and the SL logical channel belong to the same LCG. Although the BSR trigger condition and BSR MAC CE format are well distinguished for Uu and SL in TS 38.321, the data volume calculation in TS 38.322 and TS 38.323 do not differentiate whether the data volume is to calculate only the UL data or SL data. As a result, if Uu logical channel and SL logical channel belong to the same LCG, Uu and SL data volume will be reported in one LCG together and the network may not be able tell how much UL grant and SL grant are needed separately. In order not to complicate the specification and implementation, it would be better not to assign the same LCG to both Uu and SL logical channels. Similar to CA, where each logical channel of split RB are assigned to different LCGs, it can be left up to network’s implementation and no UE impact is expected.

Proposal 11: It is up to network not to assign the same LCG to Uu logical channel and SL logical channel, hence, no specification impact.

For Uu-BSR, the maximum number of LCGs is 8. For SL-BSR, TS 38.321 also states that the maximum number of LCGs is 8. The question is whether the remote UE can be configured with total 16 LCGs or the maximum number of total LCGs, i.e., including Uu LCG and SL LCG, should be kept under 8.

If the maximum number of LCGs is 8 with multi-path, the granularity will decrease in buffer size reporting, which is not desirable from scheduling perspective. On the other hand, we don’t see any technical problem to support 16 LCGs in the UE side given that the maximum number of LCGs is already 256 for IAB. Therefore, we suggest that the maximum number of LCGs for one MAC is 16 for multi-path operation.

Proposal 12: The maximum number of LCGs for a MAC entity is 16 if multi-path is configured.

The next question is whether the LCG ID is separated for Uu LCG and SL LCG, i.e., from 0 to 7 for Uu LCG and 8 to 15 for SL LCG. As long as it is clear how the buffer size of each LCG is obtained, it wouldn’t cause any problem to assign the same LCG ID to both LCGs. On the other hand, assigning LCG ID from 8 to 15 to SL LCG may require change of the SL BSR MAC CE format because the LCG ID field is 3 bits in SL BSR MAC CE, which can indicate only up to LCG ID 7.

With Proposal 10, it is clear how the buffer size is calculated for each LCG, and buffer size for Uu LCG and SL BSR are not reported in the same MAC CE, i.e., they are reported in a separate MAC CE, hence, no confusion expected. Thus, it would be possible that Uu LCG and SL LCG have LCG ID value from 0 to 7 as today.

Proposal 13: The LCG ID for Uu LCG and SL LCG is from 0 to 7, hence, no specification impact.

## 2.7	Where to send RRCReconfigurationComplete message

The figure below represents signalling procedure when indirect path is added on top of the direct path, which is under discussion in [POST122][403] email discussion.

It still needs more discussion where to send the RRCReconfigurationComplete under which condition.

- If the target relay UE is in RRC_CONNECTED

If the target relay UE is in RRC_CONNECTED, it is not necessary to send the RRCReconfigurationComplete message via the indirect path, which is used to trigger the relay UE in RRC_IDLE/INACTIVE to RRC_CONNECTED. Thus the direct path should be the baseline if the target relay UE is in RRC_CONNECTED. Sending it to the indirect path only introduces unnecessary delay due to resource request/selection and scheduling procedures over two hops.

Observation 1: If the target relay UE is in RRC_CONNECTED, sending the RRCReconfigurationComplete message via the indirect path results in undesirable delay.

Proposal 14: If the target relay UE is in RRC_CONNECTED, the remote UE sends the RRCReconfigurationComplete message (at least) via the direct path.

- If the target relay UE is not in RRC_CONNECTED (related to step 5b)

In the figure above, step 5b needs further discussion. In RAN2#121bis-e, RAN2 agreed that the remote UE sends the RRCReconfigurationComplete message to the direct path when (1) primary path of split SRB1 is on direct path or (2) non-split SRB1 is configured on direct path. Thus, to send the RRCReconfigurationComplete message to either the direct path or the indirect path depending on the target relay UE’s RRC state, the gNB can set the primary path properly. For instance, when adding an indirect path, the gNB sets the primary path of the split SRB1 to the direct path if the target relay UE is in RRC_CONNECTED. If the target relay UE is not in RRC_CONNECTED, the gNB should set the primary path of the split SRB1 to the indirect path.

Observation 2: The gNB may set the primary path depending on the RRC status of the target relay UE.

However, the gNB may not want to determine the primary path based on the target relay UE’s RRC state but rather based on e.g., the link quality or traffic load. In this case, the gNB may need to indicate the path to send the RRCReconfigurationComplete on top of the primary path. Alternatively, the gNB may provide the RRC state of the target relay UE to the remote UE so that the remote UE sends the RRCReconfigurationComplete message to the intended path according to the target relay UE’s RRC state. It should be noted that this option creates an exceptional case because the remote UE will send the RRCReconfigurationComplete to the indirect path even though the primary path is on the direct path.

Observation 3: In order not to associate the primary path and target relay UE’s RRC state, the gNB needs to indicate either the path to send the RRCReconfigurationComplete message explicitly or the RRC state of the target relay UE to the remote UE.

With the observations 2 and 3, we propose,

Proposal 15. To send the RRCReconfigurationComplete message to either a direct or an indirect path depending on the target relay UE’s RRC state, RAN2 discuss the following options:

- Option 1. The gNB sets the primary path of the split SRB0 based on the RRC state of the target relay UE.

- Option 2. The gNB configures a path for the remote UE to send the RRCReconfigurationComplete message via separate parameter than the primary path.

- Option 3. The gNB indicates the RRC state of the target relay UE to the remote UE so that the remote UE sends the RRCReconfigurationComplete message regardless of the primary path.

If the remote UE sends the RRCReconfigurationComplete message over the direct path while the relay UE is not in RRC_CONNECTED, the remote UE needs to send PC5-RRC to the relay UE in order to initiate RRC connection establishment or resume procedure.

Proposal 16. The remote UE sends PC5-RRC to the target relay UE to bring the target relay UE to RRC_CONNECTED if the remote UE sends the RRCReconfigurationComplete message over the direct path as a result of option selected in Proposal 15.

# 3	Conclusion

In this contribution, we address open issues and some stage 3 issues. The observations and proposals are:

Proposal 1: For both scenario 1 and 2, the UE switches the primary path from the direct path to the indirect path upon detection of failure on the direct path.

Proposal 2: Buffer size information is included when reporting the RLF.

Proposal 3: RAN2 discuss if the gNB can configure a condition not to send the RLF report even if the UE detects the RLF on one path while another path can be used for transmission of the RLF report.

Proposal 4: Non-split SRB on the indirect path is not supported.

Proposal 5: For split DRB, primary path can be either the direct or the indirect path.

Proposal 6: For Duplication RLC Activation/Deactivation MAC CE, RAN2 discuss which RLC entity the RLCi field indicates activation/deactivation.

Proposal 7: When an MP remote UE receives a Duplication Activation/Deactivation MAC CE, the MP remote UE activates or deactivates the secondary RLC entity, i.e., RLC entity corresponding to the non-primary path, and the primary RLC entity, which is an RLC entity for primary path, is not impacted.

Proposal 8: For Duplication RLC Activation/Deactivation MAC CE and Duplication Activation/Deactivation MAC CE in SL MP relay, RAN2 discuss whether to assign a new LCID or to update the field description without assigning any LCID.

Proposal 9: In case of relay UE’s handover, the network releases the configuration of the relay UE at the remote UE before the relay UE’s handover, which requires neither network restriction nor additional UE behaviour to be specified in the specification.

Proposal 10. When indirect path is released the network reconfigures the remote UE first and then reconfigure the serving relay UE, i.e., step 4 is prior to step 3 in the figure.

Proposal 11: It is up to network not to assign the same LCG to Uu logical channel and SL logical channel, hence, no specification impact.

Proposal 12: The maximum number of LCGs for a MAC entity is 16 if multi-path is configured.

Proposal 13: The LCG ID for Uu LCG and SL LCG is from 0 to 7, hence, no specification impact.

Proposal 14: If the target relay UE is in RRC_CONNECTED, the remote UE sends the RRCReconfigurationComplete message via the direct path.

Proposal 15. To send the RRCReconfigurationComplete message to either a direct or an indirect path depending on the target relay UE’s RRC status, RAN2 discuss the following options:

- Option 1. The gNB sets the primary path of the split SRB0 based on the RRC status of the target relay UE.

- Option 2. The gNB configures a path for the remote UE to send the RRCReconfigurationComplete message via separate parameter than the primary path.

- Option 3. The gNB indicates the RRC status of the target relay UE to the remote UE so that the remote UE sends the RRCReconfigurationComplete message regardless of the primary path.

Proposal 16. The remote UE sends PC5-RRC to the target relay UE to bring the target relay UE to RRC_CONNECTED if the remote UE sends the RRCReconfigurationComplete message over the direct path as a result of option selected in Proposal 15.

# 4	Annex

#### 6.1.3.11	Duplication Activation/Deactivation MAC CE

The Duplication Activation/Deactivation MAC CE of one octet is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of a single octet containing eight D-fields. The Duplication Activation/Deactivation MAC CE is defined, for a MAC entity, as follows (Figure 6.1.3.11-1).

-	Di: This field indicates the activation/deactivation status of the PDCP duplication of DRB i where i is the ascending order of the DRB ID among the DRBs configured with PDCP duplication and with RLC entity(ies) associated with this MAC entity. The Di field is set to 1 to indicate that the PDCP duplication of DRB i shall be activated. The Di field is set to 0 to indicate that the PDCP duplication of DRB i shall be deactivated.

Figure 6.1.3.11-1: Duplication Activation/Deactivation MAC CE

NOTE:	The Duplication Activation/Deactivation MAC CE is not used if a DRB is configured with more than two RLC entities, i.e. with moreThanTwoRLC-DRB.

#### 6.1.3.32	Duplication RLC Activation/Deactivation MAC CE

The Duplication RLC Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.32-1).

-	DRB ID: This field indicates the identity of DRB for which the MAC CE applies. The length of the field is 5 bits;

-	RLCi: This field indicates the activation/deactivation status of PDCP duplication for the RLC entity i where i is ascending order of logical channel ID of secondary RLC entities in the order of MCG and SCG, for the DRB. The RLCi field is set to 1 to indicate that the PDCP duplication for the RLC entity i shall be activated. The RLCi field is set to 0 to indicate that the PDCP duplication for the RLC entity i shall be deactivated.

Figure 6.1.3.32-1: Duplication RLC Activation/Deactivation MAC CE