3GPP TSG-RAN WG2 Meeting #123	R2- 2308752

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	7.7.4.2

Source:	ETRI

Title:		Discussion on random access in the unchanged PCI scenario

Document for:	Discussion and Decision

1	Introduction

In the past meetings, the followings are agreed [1, 2].

RAN2 #121-bis-e agreements:

Agreements:

1.	In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1 (as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18)

2.	Remove the part in brackets “as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18” in the LS to RAN1. The action to RAN1 will also ask for feedback for the hard satellite switch (not only the soft satellite switch case), e.g. action to RAN1 is to see if there are any major technical issues (as in the agreement).

RAN2 #122 agreements:

Agreements:

- t-Service in SIB19 can also be interpreted by Rel-18 UE in Connected mode to know that a satellite change or feeder link change happens

- In hard switch unchanged PCI scenario (i.e. no handover), the UE needs to know the time the UE attempts to re-synchronize. (FFS whether a new “t-Start” / a t-gap is needed or whether t-Service can be reused (i.e. no other IE) if the gap is very short/zero).

In this paper, we provide our views on random access in the unchanged PCI scenario, focused on skipping the random access for uplink TA after satellite switch. We also discuss the assistance information required for uplink TA pre-compensation for the target satellite.

2	Discussion

2.1	Discussion on skipping random access in the unchanged PCI scenario

Figure 1. Unchanged PCI scenario

As shown in Figure 1, the satellite providing a service link to each UE in the quasi-Earth fixed cell is changed in the unchanged PCI scenario. That is, satellite switch occurs from the source satellite to the target satellite. Therefore, uplink TA also changes after the satellite switch.

In the past, RAN2 asked to RAN1 whether RACH-less handover is possible in “(4) Inter-satellite handover with same gateway/gNB” scenario [3]. RAN1 replied that “the RACH-less handover may be possible, assuming the following notes can be satisfied, when UE UL transmission synchronization can be maintained by applying pre-compensation using the assistance information, e.g., epoch time, ephemeris, common TA, of the target cell.” [4].

The PCI unchanged scenario is not an L3 HO scenario. However, taking into our consideration the reply from RAN1, we think random access after the satellite switch in the unchanged PCI scenario can be skipped. This is because the unchanged PCI scenario corresponds to “(4) Inter-satellite handover with same gateway/gNB” scenario in [4].

Random access after the satellite switch in the unchanged PCI scenario can be skipped if UE UL transmission synchronization can be maintained.

Since the PCI unchanged scenario is not an L3 HO scenario, HO signaling is not required for the UEs in the cell when the satellite switch occurs. Therefore, the HO storming problem never occurs in the unchanged PCI scenario

On the other hand, the RACH congestion problem still exists because each UE needs random access for uplink TA after the satellite switch. However, if UE UL transmission synchronization can be maintained through an appropriate pre-compensation for the target satellite, the RACH congestion can be also avoided.

If random access for uplink TA after satellite switch can be skipped, it seems that the unchanged PCI scenario will provide strong support for quasi-Earth fixed cell service.

Unchanged PCI scenario that does not require random access can avoid not only HO storming but also RACH congestion

In the unchanged PCI scenario, NW is able to help the UEs to apply uplink TA pre-compensation for the target satellite due to following reasons. Firstly, NW knows valid assistance information relevant to the target satellite, such as the ephemeris of the target satellite. Secondly, NW can provide such assistance information to the UEs before the satellite switch using the service link of the source satellite. Lastly, NW knows when the satellite switch occurs, that is, NW can inform the UEs when they have to apply pre-compensated TA to their uplink transmissions (for example, ‘t-Start’ or “t-Service + t-Gap” [2])

In the unchanged PCI scenario, random access after the satellite switch is not necessary if TA pre-compensation for the target satellite is applied.

2.2	Discussion on the assistance information relevant to the target satellite

As stated in the note of [4], gNB is expected to provide valid assistance information of the target cell to UE in order to avoid random access after the satellite switch

The assistance information relevant to the target satellite should be provided by the gNB before the satellite switch in order for TA pre-compensation for the target satellite. However, before the switch, a UE cannot obtain the assistance information because it is not in the current system information or anywhere else.

The assistance information relevant to the target satellite for TA pre-compensation is signalled to all UEs before the satellite switch.

In [2], the considered assistance information is such as epoch time, ephemeris, common TA, of the target cell, which are common to all UEs in the cell. Consequently, broadcast is a suitable signaling method for the assistance information.

The assistance information relevant to the target satellite for TA pre-compensation is broadcasted as system information to all UEs before the satellite switch.

3	Conclusions

In this paper, the following observations are made:

Observation 1	Random access after the satellite switch in the unchanged PCI scenario can be skipped if UE UL transmission synchronization can be maintained.

Observation 2	Unchanged PCI scenario that does not require random access can avoid not only HO storming but also RACH congestion

In this paper, we propose the followings:

Proposal 1	In the unchanged PCI scenario, random access after the satellite switch is not necessary if TA pre-compensation for the target satellite is applied.

Proposal 2	The assistance information relevant to the target satellite for TA pre-compensation is signalled to all UEs before the satellite switch.

Proposal 3	The assistance information relevant to the target satellite for TA pre-compensation is broadcasted as system information to all UEs before the satellite switch.

References

R2-2304201, Report from Break-Out Session on NR NTN and IoT NTN, RAN2 #121bis-e, Online, 17th - 26th Apr., 2023.

R2-2306541, Report from Break-Out Session on NR NTN and IoT NTN, RAN2 #122, Incheon, South Korea, 22th - 26th May, 2023

R2-2211017, LS on RACH-less handover in NTN, RAN2 #121, RAN2 #119bis-e, Online, 10th - 19th Oct., 2022.

R1-2213001, Reply LS on RACH-less handover in NTN, RAN1 #111, Toulouse, France, 14th – 18th Nov., 2022.