3GPP TSG-RAN WG2 #123                                                                                               R2-2308756

Toulouse, France, 21st Aug. – 25th Aug., 2023                                            Revision of R2-2306309

Agenda Item:	    7.4.3

Source: 	    MediaTek Inc.

Title:		    Discussion on subsequent CPAC

Document for:	    Discussion and decision

# 1 Introduction

In this contribution, we discuss the following objective #2 from mobility enhancement work item [1].

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

RAN2 has the following agreement for objective#2.

In RAN2#119e

- The selective activation of cell groups should correspond to support of subsequent conditional changes (CPC) after a cell group change (normal or conditional). CPA FFS.

- Initial focus on SCG

- There is interest to support delta configuration, to reduce the signalling overhead (FFS if some other objective should be achieved)

- FFS how many subsequent conditional changes are targeted (and what is the impact of such assumption).

- FFS whether there is a security issue: e.g. to determine vertical or horizontal key derivation, e.g. security parameters re-used as part of subsequent CG switch (for the case when UE goes back to a previous cell, maybe in another SN), and FFS on the procedure/method with which the UE derives the SN security, e.g. based on a prior MN config (without RRC CPC config at the time of SN switch).

In RAN2#119bis-e

- Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs):

a.	Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell.

b.	Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells.

c.	Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell.

- Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

- Confirm that we aim to support delta configuration, i.e. that there need to be a known reference.

- RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

In RAN2#120

Delta configuration

- A UE stores the reference configuration as a separate configuration.

- The reference configuration is managed separately

In RAN2#121

- Assume to support the following scenarios of SCG selective activation:

    - SN initiated intra-SN SCG selective activation

    - MN initiated inter-SN SCG selective activation

    - SN initiated inter-SN SCG selective activation

- It is assumed that if the UE need to be able to return to a current SCG  by conditional procedure, then the network could explicitly configure a candidate configuration for that  cell.

- In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case).

- R2 assumes that a CPA conditional configuration can be used for CPC (but with different triggering conditions)

Inter-node

- R2 understands that A target SN may include an indication in SN Addition Request Ack for each candidate target PSCell, denoting whether the associated SCG configuration is a delta with respect to the reference SCG configuration.

In RAN2#121bis

- For the reference configuration for SCG Selective Activation, aim at following similar design as LTM.

- For inter-SN SCG Selective Activation, the RRC reconfiguration message containing the Rel-18 CPC configurations provided to the UE is in MN format.

- For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC.

FFS on the following options for subsequent CPC:

Option 1: Source MN generates the execution conditions for all subsequent CPC.

Option 2: Candidate SN may generate execution conditions for subsequent CPC.

- For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. 
FFS if Candidate SN may generate/modify execution conditions for subsequent CPC

- Assume for now that there is only one reference configuration.

- The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations:

- Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty.

FFS whether MCG configuration is included.

FFS RRC model for the reference configuration.

- Initial List of candidate target PSCells (this list can be updated by the network, e.g., cells may be added or removed) with associated target SCG configurations. FFS whether the MCG configurations associated with the target SCG configurations are included.

3. The execution conditions associated with each candidate target PSCell.

a.	For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported.

b.	For SN initiated procedure, execution conditions based on events A3/A5 are supported.

- UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

In RAN2#122

- For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC.

- FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC

- The UE shall skip the condition evaluation for a candidate which is a current PScell.

- The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

- Will not spend specific efforts for supporting nested configurations for candidate cell configuration.

- Rapporteur take initiative on naming offline

- Terminology is “Subsequent CPAC”

In this paper, we continue to discuss further aspects on objective#2.

# 2 Discussion

## 2.1 Execution condition

RAN2 has agreed for SN-initiated SCG selective activation (SN-initiated subsequent CPAC), candidate SN generates execution conditions for subsequent CPC. It is still FFS whether MN-initiated case could follow similar design. In our view, it seems feasible to let candidate SN to generate the execution condition also in MN-initiated scenario. These is also more suitable as PSCell mobility is usually controlled by SN and MN is only change the PSCell for load balance purpose. The RRC signaling for execution condition could also be simplified if we have harmonized design for both MN-initiated and SN-initiated cases.

Proposal 1: For MN-initiated subsequent CPAC, candidate SN generates execution conditions for subsequent CPC.

If P1 is agreed, for both MN-initiated and SN-initiated scenario, the UE has to update the execution condition each time CPC/CPA is performed. Since it is candidate SN to generate the execution condition, the execution conditions for subsequent CPC will be based on SCG MeasConfig (not MCG MeasConfig even for MN-initiated cases). For RRC configuration, this could be implemented by a list of execution conditions for each candidate PSCell. Sample ASN.1 code is shown below.

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::= SEQUENCE {

condReconfigId-r16         CondReconfigId-r16,

condExecutionCond-r16      SEQUENCE (SIZE (1..2)) OF MeasId              OPTIONAL,  -- Need M

condRRCReconfig-r16        OCTET STRING (CONTAINING RRCReconfiguration)  OPTIONAL,  -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17   OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL  -- Need M

]],

[[

subsequentCPC-r18          SubsequenetCPC-r18                          OPTIONAL     -- Need M

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

SubsequenetCPC-r18       ::=     SEQUENCE {

condExecutionUpdtList-r18      SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondExecutionUpdt-r18

}

CondExecutionUpdt-r18    ::=     SEQUENCE {

condReconfigId-r18               CondReconfigId-r16,

condExecutionCondSCG-r18         SEQUENCE (SIZE (1..2)) OF MeasId

}

Proposal 2: For both MN-initiated and SN-initiated subsequent CPAC, a list of execution conditions based on SCG measConfig is provided for each candidate PSCell. Each entry of the list indicates the execution condition toward another candidate PSCell while the candidate PSCell is used.

With above proposals, event A3 or A5 is already supported for MN-initiated case for subsequent CPC (based on SCG MeasConfig). There is no need to support A3/A5 event based on MCG MeasConfig.

Proposal 3: For subsequent CPAC, do not support A3/A5 events based on MCG MeasConfig.

## 2.2 Candidate Configuration

In legacy CPC, the candidate configuration of the candidate PSCell is modelled as a complete RRC Reconfiguration message. There is no clear agreement for subsequent CPAC. We assume that same principle should be adopted. So we propose to confirm that candidate configuration is an RRC Reconfiguration message (within a container). It can be further discussed the configuration limitation for the candidate configuration. Note that in LTM, it is also agreed to use a RRC Reconfiguration message as candidate configuration (model 1).

Note that it is still FFS whether the MCG configurations associated with the target SCG are included in the candidate configuration. In our view, this should be up to network implementation as in legacy CPC.

Proposal 4:	For subsequent CPAC, the candidate configuration is modelled as a complete RRCReconfiguration message within a container (as in legacy CPAC). The MCG configuration could also be included in the candidate configuration.

In addition, as in legacy CPC, the candidate configuration is maintained by network and could be modified by the RRC reconfiguration. Again we propose to follow the same mechanism from CPC. Note that while modifying the candidate configuration, it is done by a replacement (not delta update).

Proposal 5:	For subsequent CPAC, the candidate configuration can be modified by RRC reconfiguration. When UE receives a new candidate configuration, the stored one is replaced with the newly received one (as in legacy CPC).

## 2.3 Reference configuration

For reference configuration, we haven’t discussed the RRC model for it. In our view, we think same model as candidate configuration should be used. A complete RRC reconfiguration message provide full flexibility and we can discuss if there is limitation on this.

Proposal 6:	For subsequent CPAC, the reference configuration is modelled as a complete RRCReconfiguration message within a container.

According to the discussion from previous meeting(s), companies are not sure whether MCG configuration should be included in reference configuration. Our view is that there is no strong need to have MCG configuration (MCG configuration could be included in candidate configuration). However, we also don’t see strong need to limit the flexibility. It could be up to network implementation as long as the complete RRC reconfiguration (see P6) is a reasonable/intended one.

Proposal 7:	MCG configuration could be included in the reference configuration, if needed.

The maintenance of reference configuration should be the same as maintenance of candidate configuration. The NW could add/modify/release the reference configuration and the modification is done by replacement of old configuration.

Proposal 8:	The reference configuration can be modified by RRC reconfiguration. When UE receives a new reference configuration, the stored one is replaced with the newly received one.

## 2.4 Delta Aspects

RAN2 agree to follow similar design as LTM for reference and delta reconfiguration. It is still FFS on the optionality of reference configuration. In our understanding, there are actually two different intended UE behavior while reference configuration is not provided

- UE behavior 1 – Not providing reference configuration is just another ASN.1 alternative for empty reference configuration. The UE apply the candidate configuration (in this case it is a complete configuration) to replace current configuration.

- UE behavior 2 – Not providing reference configuration is intended to say that the UE shall apply the candidate configuration as delta configuration from current configuration.

Observation 1: Different UE behavior should be clarified if “not providing reference configuration” is supported.

- UE behavior 1 – The UE shall apply the candidate configuration (which is a complete configuration) to replace current configuration.

- UE behavior 2 – The UE shall apply the candidate configuration as delta configuration from current configuration.

For UE behavior 1, we think that could be discussed further in ASN.1 review on how to indicate “empty” reference configuration. For UE behavior 2, it was discussed in LTM but not agreed. Although we are also fine with UE behavior 2, our assumption is to have only UE behavior 1 to speed up the progress. We think that reference configuration could be optional but the UE behavior is the same for “no reference configuration” or “empty reference configuration”. That is, the UE just replace current configuration with candidate configuration (i.e. the candidate configuration should be a complete configuration)

Overall, we would like to confirms the following delta configuration aspect.

Proposal 9: RAN2 confirms the following the delta aspect of applying the candidate configuration for subsequent CPAC.

- Step 1 - The candidate configuration is applied on top of the reference configuration to form a complete candidate configuration. This can be done before or upon execution condition is met (up to UE implementation).

- Step 2 - While execution condition is met, the UE apply the complete candidate configuration to replace the current configuration.

- Note: In case reference configuration is empty or there is no reference configuration, the UE just performs step 2 above.

It is clear that using complete configuration could work for subsequent CPAC. Using reference configuration requests much more testing effort as it is completely new procedure in RRC. Since using reference configuration is just a signaling optimization for this feature, we should make the reference configuration a mandatory part for subsequent CPAC. Therefore, we propose to confirm that reference configuration for subsequent CPAC is optional UE capability.

Proposal 10: Supporting of reference configuration for subsequent CPAC is optional UE capability.

## 2.5 Handling of S-CPAC configuration in legacy scenario

It has been agreed in RAN2 that S-CPAC configuration should be released upon PCell change (at least for inter-MN case). In our understanding, this is more like a stage 2 description on network behavior. From UE perspective (seems in general the UE does not know PCell change is intra-MN or inter-MN), it should just follow the network command to keep or release a configuration.

Similar principle as in SCG release. If the network think the S-CPAC configuration could be reuse after SCG release (i.e. SCG part is somehow a complete configuration, could be used both for PSCell addition and PSCell change), network could keep it. Otherwise, it can use explicit signaling to release the S-CPAC configuration.

Proposal 11: The UE does not release subsequent CPAC configuration autonomously upon SCG Release or PCell change. The network should use explicit signaling to release it if the configuration is no longer suitable.

On the other hand, for RRC re-establishment (where the mobility is not controlled by network), we suggest to remove the subsequent CPAC configuration autonomously as in legacy CPAC.

Proposal 12: The UE shall release subsequent CPAC configuration autonomously upon RRC re-establishment.

# 3 Conclusions

Based on the discussion in section 2, we propose the following observations and proposals:

Execution condition

Proposal 1: For MN-initiated subsequent CPAC, candidate SN generates execution conditions for subsequent CPC.

Proposal 2: For both MN-initiated and SN-initiated subsequent CPAC, a list of execution conditions based on SCG measConfig is provided for each candidate PSCell. Each entry of the list indicates the execution condition toward another candidate PSCell while the candidate PSCell is used.

Proposal 3: For subsequent CPAC, do not support A3/A5 events based on MCG MeasConfig.

Candidate configuration

Proposal 4:	For subsequent CPAC, the candidate configuration is modelled as a complete RRCReconfiguration message within a container (as in legacy CPAC). The MCG configuration could also be included in the candidate configuration.

Proposal 5:	For subsequent CPAC, the candidate configuration can be modified by RRC reconfiguration. When UE receives a new candidate configuration, the stored one is replaced with the newly received one (as in legacy CPC).

Reference configuration

Proposal 6:	For subsequent CPAC, the reference configuration is modelled as a complete RRCReconfiguration message within a container.

Proposal 7:	MCG configuration could be included in the reference configuration, if needed.

Proposal 8:	The reference configuration can be modified by RRC reconfiguration. When UE receives a new reference configuration, the stored one is replaced with the newly received one.

Delta Aspects

Observation 1: Different UE behavior should be clarified if “not providing reference configuration” is supported.

- UE behavior 1 – The UE shall apply the candidate configuration (which is a complete configuration) to replace current configuration.

- UE behavior 2 – The UE shall apply the candidate configuration as delta configuration from current configuration.

Proposal 9: RAN2 confirms the following the delta aspect of applying the candidate configuration for subsequent CPAC.

- Step 1 - The candidate configuration is applied on top of the reference configuration to form a complete candidate configuration. This can be done before or upon execution condition is met (up to UE implementation).

- Step 2 - While execution condition is met, the UE apply the complete candidate configuration to replace the current configuration.

- Note: In case reference configuration is empty or there is no reference configuration, the UE just performs step 2 above.

Proposal 10: Supporting of reference configuration for subsequent CPAC is optional UE capability.

Handling of S-CPAC configuration in legacy scenario

Proposal 11: The UE does not release subsequent CPAC configuration autonomously upon SCG Release or PCell change. The network should use explicit signaling to release it if the configuration is no longer suitable.

Proposal 12: The UE shall release subsequent CPAC configuration upon RRC re-establishment.

# 4 References

[1] RP-231475, “Revised WI: Mobility Enhancements”, MediaTek