3GPP TSG-RAN WG2 Meeting #123 	                                                    R2-2308758

Toulouse, 20 August – 27 August 2023

Agenda item:	7.17.2

Source:	China Telecom

Title:	Procedures for MUSIM temporary capability restriction

WID/SID:	NR_DualTxRx_MUSIM - Release 18

Document for:	Discussion and Decision

1	Introduction

- Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities

- Support “early indication” from UE to network during RRC connection setup/resume procedure.

- FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted.

- FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

- UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.

In this contribution, we will discuss the further details of common framework and the general procedure for both the reactive and proactive approach for MUSIM UE temporary capability restriction.

2	Discussion

2.1 General UE capability restriction procedure

This figure provides a common procedural framework for the reactive and proactive approach for MUSIM purpose.

When UE is in RRC_CONNECTED state in NR NW A and is moving to RRC_CONNECTED state from RRC_IDLE/RRC_INACTIVE state in NR NW B, there is three cases:

Case1: NW A has occupied full capabilities and UE needs to release partial capabilities to establish connection to NW B.

Case2: NW A has occupied partial capabilities, but its SCell/SCG is incompatible with NW B’s camping cell.

Case3: NW A has occupied partial capabilities, and has no negative impact on connection establishment on NW B instantly.

For case 1, due to NW A has occupied full capabilities, there is no UE RF resources to initial setup/resume procedure on NW B and UE has to lower UE capabilities on NW A. In this case, UE can execute in reactive way, sends the related request reduced capabilities changes to NW A (e.g., max MIMO layers) to NW A through UAI. To avoid potential confliction that can be caused by RRC reconfiguration in NW A, affected capabilities can also be involved in the UAI (proactive way).

For case 2, due to the conflicts on SCell/SCG, it is necessary to release the SCell/SCG firstly on NW A in order to successfully establish RRC connection on NW B. To avoid potential future conflicts as much as possible, the UE can also proactively report the capabilities that may be affected in the future (e.g., affectedBandCombinationList).

For case 3, UE can report capabilities that may be affected in the future to avoid potential conflicts (proactive way), or Step3~Step5 can be omitted since there is no conflicts happen when UE establish connection on NW B. The report maybe depends on UE implementations if network allowed.

Based on the above analysis of capability reporting for the three cases, it can be concluded that the common signalling framework can be used for both MUSIM UE reactive and proactive capability restriction reporting, but the reporting contents may varies depends on different scenarios and also when the UE will report the information. Considering reusing the existing capability reporting mechanism, the temporary capability restriction can be report via UAI.

After Step10, UE may be configured with basic configuration (e.g., SISO), however, the remaining UE capability may support 1×2 MIMO in NW B. So, it is beneficial to inform NW B about the available capabilities can be used in NW B and also the affected capabilities.

If Step3~Step5 are omitted, UE can inform NW A with the restricted capability via UAI in Step11b.

Observation 1: No matter proactive approach or reactive approach, common signalling framework can be used.

Proposal 1: It is not necessary to explicitly distinguish reactive and proactive approach in specifications, but may include different IEs in UAI. Both reactive and proactive UE capabilities restrictions can be included in one UAI.

If UE reports a UAI including IE definitions of “releasedCapability” and “reducedCapability” to NW A, an in-time response from NW A is expected, otherwise it will cause much delay in connection establishment on NW B. So, it is beneficial to introduce a wait timer to specify the UE behaviours when no network response is received . When the timer expires, UE can e.g. either reduce capability locally or request to release connection in NW A (using Rel-17 MUSIM leave indication), or decide not to respond the paging from NW B. It’s suggested to define a certain UE behaviour in RAN2 specifications.

Observation 2: it will be more controllable to define the UE behaviour when no NW response was received after the expiry of wait timer when reduced and released capability restrictions are included in UAI.

Proposal 2: Introduce a wait timer for reactive capabilities restrictions reporting to address the UE behaviour when no response was received from NW. When the timer expiries, UE can e.g. either reduce capability locally or request to release connection in NW A (using Rel-17 MUSIM leave indication), or not respond the paging from NW B.

If UE reports UE capability restriction frequently through UAI, it may cause large signaling overhead and unnecessary RRC reconfiguration. Introducing a prohibit timer is another effective solution. UE starts the prohibit timer while sending UAI to NW A and another UAI is not allowed until the timer expiry. When the timer expired, UE can report such UAI again.

Proposal 3: Introduce a prohibit timer for proactive capability restriction reporting to prevent the frequently UAI reporting. UAI cannot report again until the timer expiry. And it’s FFs whether the wait timer and prohibit timer can be merged into one single timer.

2.2 Further detail of early indication

In RAN2#121bis meeting, ran2 has agreed to support “early indication” from UE to NW during RRC connection setup/resume procedure and some FFS are still need to discussed.

- Support “early indication” from UE to network during RRC connection setup/resume procedure.

- FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted.

- FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

During setup/resume procedure, due to the very limited size of the RRC Setup/Resume Request message, only baseline parameters will be included, as Rel-18 MUSIM is an optional feature, it is more appropriate to include early indication in RRC Setup/Resume Complete message. If the RRC Resume procedure is failed due to the band confliction, UE can request RRC setup after then.

Proposal 4: It is more appropriate to send early indication in RRC Setup/Resume Complete message.

Since UE must report capability restriction under the control of the network, the indication of whether the network supports early indication of temporary capability restriction during RRC Setup/Resume procedure should be sent before RRC connection establishment. Directly, the network can indicate that if it supports early indication of temporary capability restriction in Msg5 in the system Information.

Comparing with the Broadcasting System Information, on-demand SI can be more power saving and reduce the signalling overhead. The network allocates the specified preamble index in the System Information. UE send PRACH carried the special preamble to request to NW for indication of whether it supports early indication of temporary capability restriction in Msg5. The network responds in the MSG2 RAR message, and carries the message in the SI of the corresponding request.

Proposal 5: Introduce an indication in SIB1/on-demand SI to indicate whether the UE is allowed to send temporary capability restriction indication in Msg5.

2.3 Discussion of requesting SCell/SCG to be released in proactive way.

RAN2 has agreed that UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose and remained a FFS on how/whether this works for the proactive case. In proactive way, UE can not only report the band combinations/bands affected by camping frequency in NW B but also report the band combinations/bands that may affected in the future (which can be derived based on the UE’s knowledge). UE can report both constrained/affected band combinations/bands as affectedBandCombinationList contained in UAI to NW A.

Proposal 6: Compared to explicit request in reactive way, proactive way favours implicit request. In proactive case, UE can indicate a list of constrained/affected band combinations/bands(e.g., band combinations/bands affected by camping frequency in NW B, band combinations/bands may affected in the future) to NW A via UAI.

3	Conclusion

Observation 1: No matter proactive approach or reactive approach, common signalling framework can be used.

Observation 2: it will be more controllable to define the UE behaviour when no NW response was received after the expiry of wait timer when reduced and released capability restrictions are included in UAI.

Proposal 1: It is not necessary to explicitly distinguish reactive and proactive approach in specifications, but may include different IEs in UAI. Both reactive and proactive UE capabilities restrictions can be included in one UAI.

Proposal 2: Introduce a wait timer for reactive capabilities restrictions reporting to address the UE behaviour when no response was received from NW. When the timer expiries, UE can e.g. either reduce capability locally or request to release connection in NW A (using Rel-17 MUSIM leave indication), or not respond the paging from NW B.

Proposal 3: Introduce a prohibit timer for proactive capability restriction reporting to prevent the frequently UAI reporting. UAI cannot report again until the timer expiry. And it’s FFs whether the wait timer and prohibit timer can be merged into one single timer.

Proposal 4: It is more appropriate to send early indication in RRC Setup/Resume Complete message.

Proposal 5: Introduce an indication in SIB1/on-demand SI to indicate whether the UE is allowed to send temporary capability restriction indication in Msg5.

Proposal 6: Compared to explicit request in reactive way, proactive way favours implicit request. In proactive case, UE can indicate a list of constrained/affected band combinations/bands(e.g., band combinations/bands affected by camping frequency in NW B, band combinations/bands may affected in the future) to NW A via UAI.

References

- RP-220955 Revised WID:  Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, RAN meeting #95e, Release 18.