3GPP TSG-RAN WG2 Meeting #123        	R2-2308771

Toulouse, France, August 21-25, 2023

Source:	vivo

Title:          Discussion on the issue of unpredictable measurement report sequence

Agenda Item:	7.24.2

Document for:	Discussion and Decision

- Introduction

At the last RAN2 meeting, an issue on unpredictable measurement report sequence was raised and discussed in [1][2]. RAN2 reached the following agreements:

- The issue is about latency from MO configuration to Measurement Report for the most interesting frequency(ies) e.g. for the service of the UE. More specifically at the time of measurement report, the network cannot know which frequencies the UEs has already measured, so there is no way for the network to decide to wait or not for another potential measurement report (expect the network to act immediately on the first measurement report). - There is support to attempt a solution for Rel-18. Solution discussion next meeting.

In this contribution we will further discuss the issue and potential solutions.

- Discussion

The measurement model is as below (extracted from [3]):

Figure 9.2.4-1: Measurement Model NOTE 1: K beams correspond to the measurements on SSB or CSI-RS resources configured for L3 mobility by gNB and detected by UE at L1. - A: measurements (beam specific samples) internal to the physical layer. - Layer 1 filtering: internal layer 1 filtering of the inputs measured at point A. Exact filtering is implementation dependent. How the measurements are actually executed in the physical layer by an implementation (inputs A and Layer 1 filtering) is not constrained by the standard. - A1: measurements (i.e. beam specific measurements) reported by layer 1 to layer 3 after layer 1 filtering. - Beam Consolidation/Selection: beam specific measurements are consolidated to derive cell quality. The behaviour of the Beam consolidation/selection is standardised and the configuration of this module is provided by RRC signalling. Reporting period at B equals one measurement period at A1. …

For a measured frequency, the UE firstly performs the L1 samples at multiple time points (i.e., point A in the measurement model). Then the UE performs the Layer 1 filtering based on the multiple L1 samples to obtain the L1 measurement result. Further the L1 measurement result is reported by layer 1 to layer 3. After cell quality derivation and layer 3 filter the cell quality is used for evaluation of reporting criteria.

If there are multiple frequencies to be measured, L1 measurement sampling for each frequency is staggered in turn. An example is showed in Figure 1:

Figure 1

If the UE receives the measurement configuration for f1/f2/f3 with same report configuration and there are enough good cells for each frequency, the first measured frequency (f1 in the above example) will firstly trigger the measurement report. Before the measurement report is triggered, there are the TimeToTrigger duration which is used to ensure the stability of the cell signal. During TimeToTrigger at least one measurement by layer 1 to layer 3 occurs, otherwise the the TimeToTrigger is meaningless since cell quality is not updated during TimeToTrigger. Therefore, we can conclude that other measured frequencies (f2/f3) have been measured but it is likely that the corresponding TimeToTrigger timers are running, not expired. However, the sequence of the measured frequencies is left to UE implementation. Different UEs may have different implementation as described in [1], which causes unpredictable measurement report sequence and further the UE handover to an unexpected frequency.

Observation: If the UE receives the measurement configuration for f1/f2/f3 with same report configuration and there are enough good cells for each frequency, the first measured frequency will firstly trigger the measurement report. Other measured frequencies (f2/f3) have been measured but it is likely that the corresponding TimeToTrigger timers are running and not expired.

To address the issue, the following solutions were presented in [2] in the last RAN2 meeting:

- Option 1: Network provides the sequence of MeasId, and UE measures and evaluates each frequency one by one. - Option 2: Network provides recommended priority order for the configured MeasObject or MeasId. - Option 3: Introducing new reporting event, i.e., one frequency (low-priority) is higher than a threshold and the other frequency (high-priority) is lower than another threshold.

For Option 1 and Option 2, the network controls the measurement order of measured frequencies. RAN4 impact is forseen, but the majority of companies do not like RAN4 involvement and want to keep this within RAN2 scope.

For Option 3, it is possible that there are multiple high-priority frequencies. It is difficult to define the new report event for multiple high-priority frequencies. This has a great impact on the specification.

Besides the above options, we think that the following options can be considered:

- Option 4: reuse the existing report event, but the report is triggered when an additional condition is fulfilled, e.g., there is no cell of high-priority frequencies which fulfills the event entry condition.

- Option 5: enhance the measurement report to carry the best cell’s measurement result of high-priority frequencies.

With option 5, the network may need to wait for potential measurement report of high-priority frequencies after receiving the measurement report of low-priority frequency, which increase the complexity of network implementation.

Proposal: Besides Option1/2/3, RAN2 to consider the following options:

- Option 4: reuse the existing report event, but the report is triggered when an additional condition is fulfilled, e.g., there is no cell of high-priority frequencies which fulfills the event entry condition.

- Option 5: enhance the measurement report to carry the best cell’s measurement result of high-priority frequencies.

- Conclusion

In this contribution we discuss the issue of unpredictable measurement report sequence and have the following observation and proposal:

Observation: If the UE receives the measurement configuration for f1/f2/f3 with same report configuration and there are enough good cells for each frequency, the first measured frequency will firstly trigger the measurement report. Other measured frequencies (f2/f3) have been measured but it is likely that the corresponding TimeToTrigger timers are running and not expired.

Proposal: Besides Option1/2/3, RAN2 to consider the following options:

- Option 4: reuse the existing report event, but the report is triggered when an additional condition is fulfilled, e.g., there is no cell of high-priority frequencies which fulfills the event entry condition.

- Option 5: enhance the measurement report to carry the best cell’s measurement result of high-priority frequencies.

- Reference

[1]. R2-2305774	Discussion on the issue of unpredictable measurement sequence for inter-frequency measurement reporting and specification impact	CMCC, Ericsson, CATT

[2]. R2-2306762	[AT122][003][TEI18] Inter-frequency Measurements (CMCC)

[3]. 3GPP TS 38.300 v17.3.0