3GPP TSG-RAN WG2 Meeting #123	 R2-2308779

Toulouse, France, August 21-25, 2023

Agenda Item:	7.16.2.1

Source:	AT&T

Title:	Architecture and LCM aspects of AI/ML for NR air interface

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], 3GPP has agreed to study the framework for AI/ML for air-interface corresponding to target use cases considering aspects such as performance, complexity, and potential specification aspects. Some of the aspects of the study item include RAN2-led objectives:

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

RAN2 should study how the life cycle management (LCM) for a given AI/ML model can be supported by existing Uu signaling and procedures and if any enhancements are needed, specifically in the areas of model delivery/updates, data collection, and model monitoring. This contribution discusses the protocol aspects of an AI/ML framework applied to the NR air interface, specifically the details and requirements for model and functionality identification.

# Model/functionality identification and LCM

The 5G industry trends which enable network virtualization and deployment of low-latency/high bandwidth services are also making application of power Artificial Intelligence (AI) tools such as machine learning (ML) algorithms to 5G networks feasible and scalable.  These algorithms rely on historical data for deriving system models and training as well as real-time or near-real-time data collection to adapt to different network conditions. Furthermore, a variety of use cases can be supported by AI/ML techniques as noted in the SID including CSI feedback optimization, beam management, and positioning. Different use cases can have vastly different requirements in terms of the impact on network nodes or functionalities. This implies that the appropriate implementation of different AI/ML techniques may involve multiple interfaces, signalling procedures, and processing requirements (including requirements on data aggregation or co-location with different nodes/functions).

For one-sided AI/ML models that reside at the UE/network, one or multiple AI/ML models can be possibly deployed and used at the UE/network. Similarly, for two-sided AI/ML models, there may be multiple AI/ML models that can be possibly deployed. It is key then to monitor the performance of these models for various reasons: (1) Depending on the type of model used, these models may not have performance guarantees, (2) deployment options and environment characteristics may change after deployment, resulting in a sub-optional performance (3) when multiple models are available at the UE/network, model selection decision needs to be made, and this decision cannot be left to implementation (4) for one-sided models at UE the model or input/output parameters of models may be modified over time, from a third party cloud, without notifying the network, resulting in a performance degradation.

For the UE/network to be able to make certain decisions, based on the model performance monitoring or otherwise, a model identification and LCM procedure needs to be agreed on. This identification procedure needs to be flexible so it can cover all different types of models, where the model inference resides, and support all collaboration levels between the network and the UE.  The model and/or functionality identification procedure can also be used to indicate the support of functionalities applicable to the different AI/ML use cases in various deployment scenarios, network conditions, and use cases.

During RAN1#112bis the following was agreed for AI/ML functionality identification and functionality-based LCM and AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models:

For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models:

- Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

- Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

  - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities

  - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level

  - FFS: Other aspects that may constitute Functionality

FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda.

For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models:

  - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

    - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda.

    - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s).

    - FFS: relationship between functionality-based LCM and model-ID-based LCM

Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

In addition, during RAN1#113, the following agreements were made regarding the identification of model/functionalities based on the LCM:

Agreement

For model identification of UE-side or UE-part of two-sided models, categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact (if any).

- Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signaling

  - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signaling after model identification.

  - FFS: Spec impact to other WGs

- Type B: Model is identified via over-the-air signaling,

  - Type B1:

    - Model identification initiated by the UE, and NW assists the remaining steps (if any) of the model identification

      - the model may be assigned with a model ID during the model identification

    - FFS: details of steps

  - Type B2:

    - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps (if any) of the model identification

      - the model may be assigned with a model ID during the model identification

    - FFS: details of steps

- Note: The support and applicability of each model identification Type is a separate discussion. This study does not imply that model identification is necessary.

Agreement

For functionality/model-ID based LCM,

- Once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring.

Agreement

- Once models are identified, UE can indicate supported AI/ML model IDs for a given AI/ML-enabled Feature/FG in a UE capability report as starting point.

  - FFS: applicability to model identification, Type A, type B1 and type B2

    - FFS: Using a procedure other than UE capability report

  - Note: model identification using capability report is not precluded for type B1 and type B2

During RAN2#121bis the following was agreed in RAN2:

R2 assumes that Information such as FFS:vendor info, applicable conditions, model performance indicators, etc. may be required for model management and control, and should, as a starting point, be part of meta information.

Model ID can be used to identify model or models for the following LCM purposes:

- model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step).

- (e.g. for so called “model ID based LCM”)

If model transfer/delivery is supported, model ID can be used for model transfer/delivery LCM purpose.

How to achieve globality of the Model ID is FFS.

As discussed in previous meetings, the level of details regarding a given model can be different based on the use case and application. For instance, for a one-sided model which does not require any network collaboration, a functionality-based LCM could be applied where the necessary model functionalities are reported via control plane signalling as part of regular UE capability reporting to correctly identify and differentiate models which have different use cases, model structure, model parameters and/or training datasets. However, to support other parts of LCM such as model transfer/delivery, activation/deactivation, multi-vendor two-sided model pairing, and testing/calibration performed by a network operator, in addition to any functionality-based LCM meta info, a model ID could be configured/exchanged since for these purposes, the granularity of the model ID needs to support device or vendor identification, model version tracking.

Proposal 1: For AI/ML-enabled features/FGs indicated by capability signaling, meta info can be provided for functionality-based LCM functions including model transfer/delivery, activation/deactivation, multi-vendor two-sided model pairing, model-specific performance monitoring, and testing and calibration performed by a network operator.

Proposal 2: Meta info should include applicable conditions for model activation/deactivation, model identification (optionally) associated with one or multiple model IDs, and model-specific performance monitoring criteria.

Proposal 3: The applicable conditions for activation/deactivation and performance monitoring may include site-specific criteria as well as associated NW/UE configurations or capabilities.

# Conclusion

In this contribution, we discussed the AI/ML model LCM over the NR air interface. The following proposals were made:

Proposal 1: For AI/ML-enabled features/FGs indicated by capability signaling, meta info can be provided for functionality-based LCM functions including model transfer/delivery, activation/deactivation, multi-vendor two-sided model pairing, model-specific performance monitoring, and testing and calibration performed by a network operator.

Proposal 2: Meta info should include applicable conditions for model activation/deactivation, model identification (optionally) associated with one or multiple model IDs, and model-specific performance monitoring criteria.

Proposal 3: The applicable conditions for activation/deactivation and performance monitoring may include site-specific criteria as well as associated NW/UE configurations or capabilities.

# Reference

[1] RP-213599 Study on AI/ML for Air-Interface.