3GPP TSG-RAN WG2 Meeting #123		R2-2308790

Toulouse, Aug 21 – 25, 2023

Agenda Item:	7.17.4

Source: 	LG Electronics

Title:  	MUSIM Gap Priority

Document for:	Discussion

Introduction

According to the last meeting in RAN2, we have following agreements for MUSIM gap priority [1]:

1: Introduce 1 optional per-UE capability bit (without xDD/FRx differentiation) to indicate MUSIM gap priority configuration and preference. A UE supporting this feature shall also support musim-GapPreference-r17. 2: Introduce a new indication in the OtherConfig to indicate whether UE is allowed to report MUSIM gap priority preference via UAI. 4: The existing IE GapPriority-r17 is re-used to configure the priority for periodic MUSIM gap. 9: RAN2 assumes no RAN4 impact is expected on maximum UL power change due to R18 MUSIM. Can re-discuss if critical issues are found in RAN2. No consensus on P3 and P5.

In this contribution, we would like to discuss rest issues in the MUSIM gap priority.

Discussion

According to the last RAN2 agreements, the following issues mainly needs to conclude:

No consensus on P3 and P5. Proposal 3: RAN2 to discuss how UE indicates periodic MUSIM gap priority preference - Option 1: UE indicates an absolute priority for all or a subset periodic MUSIM gaps by taking into account of the Type-2 MG gap priority - Option 2: UE indicates a relative priority for all or a subset periodic MUSIM gaps, i.e. the priority is relative just among the MUSIM gaps - Option 3: wait RAN4 progress/feedback Proposal 5: RAN2 to discuss how to configure the priority when network accepts gap priority preference for a periodic MUSIM gap - Option A: Network configures the priority which is equal to the absolute value provided by the UE if Option 1 in Proposal 3 is agreed. FFS whether network can still change the absolute priorities while keeping the relative priorities among MUSIM gaps - Option B: Network configures the priority which is aligned with the relative value provided by the UE.

First, we discuss how UE indicates periodic MUSIM gap priority preference.

RAN2 has already agreed to use the existing IE GapPriority-r17 when the UE informs the network of gap priority preferences. Thus, using a relative values could introduce unnecessary uncertainty. If RAN2 doesn’t reverts the last agreement using the existing IE GapPriority-r17, it would be clearer that the UE indicates the absolute value of all currently configured gaps to the network.

In addition, we don’t think that RAN2 should wait RAN4 progress/feedback for this since how to indicate the gap priority preferences between the network and the UE is just up to RAN2 decision.

Proposal 1.	When requesting periodic MUSIM gap(s), UE indicates an absolute priority values for all or a subset periodic MUSIM gaps by taking into account of the Type-2 MG gap priority.

The next issue is how to configure the priority when network accepts gap priority preference for a periodic MUSIM gap.

RAN2 had previously agreed not to modify the MUSIM gap configuration requested by the UE. However, as this is for the MUSIM gap patterns to utilize, gap priority values is not necessary to keep to exactly as requested.

For example, before receiving gap priority preferences, the network might have been considering changes to the priority of configured Type-2 MGs. Then the absolute gap priority may need to be changed.

We think the most important is the relative priority order among MUSIM gaps as requested by the UE. Thus, the network is allowed to change the absolute priority values but keeping the relative priority for MUSIM gaps.

Proposal 2.	When receiving priorities for periodic MUSIM gap(s), the UE may receive changed priority values while keeping the relative priorities among MUSIM gaps.

Conclusion

We propose to discuss and decide on the following proposals:

Proposal 1.	When requesting periodic MUSIM gap(s), UE indicates an absolute priority values for all or a subset periodic MUSIM gaps by taking into account of the Type-2 MG gap priority.

Proposal 2.	When receiving priorities for periodic MUSIM gap(s), the UE may receive changed priority values while keeping the relative priorities among MUSIM gaps.

Reference

- R2-2304202 Report on LTE legacy, XR, QoE and MUSIM, Nokia