3GPP TSG-RAN WG2 Meeting #123     			                  R2-2308798

Toulouse, France, 21 - 25 Aug 2023                                      Revision of R2-2306289

Agenda item:	7.8.3

Source:	LG Electronics Inc.

Title:	Flight Path Reporting

Document for:	Discussion and Decision

# 1. Introduction

In this contribution, we continue the discussion on flightpath delta reporting.

# 2. Discussion

Updated Flightpath Delta Reporting

During the RAN2 meeting, an updated flight path indication was agreed upon. As for the triggering condition of this updated flight path indication, it can be configured based on either delta time or delta distance. Meanwhile, the delta signaling involving only updated waypoints in the flight path information remains for FFS.

RAN2 #119bis

- Allow the flight path to be updated.  FFS on the details.

RAN2 #120

- UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report.

- UAI message can also be used to indicate the UE has flight path availability.

- FFS whether and what triggering conditions are specified for flight update.  FFS The maximum number of waypoints within flight path plan is left FFS

RAN2 #121bis

- Flightpath update indication in UAI is configurable by the network

- As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication.  FFS if further differentiation is needed if we decide to have delta signaling

RAN2 #122

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

Given that a flight path deviation meeting the delta condition exists, the UE can identify the updated waypoints. This approach allows the UE to efficiently mitigate signaling overhead by including solely those waypoints that have been updated within the flight path information.

However, reporting the entire flight path to the network could be a straightforward approach. This is because the network can easily apply the flight path information without the need for an extra step of comparing the delta and previous information. Furthermore, if the network does not have the initial flight path information, it is necessary to include all waypoints within the flight path information.

Consequently, it is essential for the network's configurability to determine whether all waypoints are necessary or if only delta waypoints are required. To achieve this, an indication can be introduced to specify the required information.

For example, if the network requests delta information, the UE incorporates only updated waypoints. Conversely, for the entire information, all waypoints are included:

1> if flightPathInfoReq field is present and the UE has flight path information available: 2> if the includeUpdateOnly is set to TRUE; and 2> if the list of waypoints along the flight path is changed compared to last time the UE reported this information: 3> include the flightPathInfoReport and set it to include the list of changed waypoints along the flight path 2> else: 3> include the flightPathInfoReport and set it to include the list of waypoints along the flight path; 2> if the includeTimeStamp is set to TRUE: 3> set the field timeStamp to the time when UE intends to arrive to each waypoint if this information is available at the UE; #### – FlightPathInfoReportConfig The IE FlightPathInfoReportConfig specifies flight path information report configuration. FlightPathInfoReportConfig information element -- ASN1START FlightPathInfoReportConfig-r18 ::= SEQUENCE { maxWayPointNumber-r18 INTEGER (1..maxWayPoint-r18), includeTimeStamp-r18 ENUMERATED {true} OPTIONAL includeUpdateOnly-r18 ENUMERATED {true} OPTIONAL } -- ASN1STOP FlightPathInfoReportConfig field descriptions. maxWayPointNumber Indicates the maximum number of way points UE can include in the flight path information report if this information is available at the UE.. includeTimeStamp Indicates whether time stamp of each way point can be reported in the flight path information report if time stamp information is available at the UE.. includeUpdateOnly Indicates whether only changed waypoint can be reported in the flight path information report..

We propose to introduce an indication indicating whether all waypoints are necessary or if only delta waypoints are required.

Proposal 1. To introduce an indication indicating whether all waypoints are necessary or if only delta waypoints are required

# 3. Conclusion

Follow is our proposal:

Proposal 1. To introduce an indication indicating whether all waypoints are necessary or if only delta waypoints are required

# 4. References

[1] 36.331 17.5.0