3GPP TSG-RAN WG2 Meeting # 123	R2-2308813

# Toulouse, France, 21 – 25 August 2023

Agenda item:	7.4.2.2

Source:	Google

Title:	Discussion on L1 measurement RS configuration

Document for:	Discussion & Decision

- Introduction

In RAN2#122, the following agreements were made.

R2-2306769	Summary of [AT122][005][Mob18] LTM L1 measurement aspects	Ericsson

DISCUSSION

P6

-	Nokia wonder what this mean. What is the intention vs the baseline. Ericsson think this is not clear yet.

-	Ericsson think we should reuse the existing reporting.

- For L1 measurements for LTM

The RS configuration is provided to the UE per LTM candidate cell.

RAN2 assumes that Each candidate DU needs to know the RS configuration of each candidate DUs in order to provide the LTM candidate configuration.

RAN2 assumes that The CU transmits to each C-DU the RS configuration of S-DU (if this is an LTM candidate cell) and/or other C-DUs, to generate the corresponding L1 configuration for LTM.

RAN2 assumes C-DU generates the RS configuration and send to the CU. The CU transmits to the Source DU the RS configuration per LTM candidate cell and the associated LTM candidate (when the CU receives LTM candidate configuration(s) from the C-DU). It is up to RAN3 whether the RS configuration is sent before (or at the same time of) the C-DU creates the LTM candidate configuration (and whether is semi-statis or UE associated).

The RS configuration and/or CSI resource configuration for measuring LTM candidate cells is included in the LTM-Config IE and is a separate configuration, e.g. outside of the LTM candidate configuration.

CSI reports for LTM candidates (neighbour cell reports for the purpose of LTM cell switch) are configured by the serving cell in an IE that is like CSI-ReportConfig for LTM within the ServingCellConfig since this is the cell in which the report is to be transmitted.

RAN2 assumes the following about CSI measurement reporting for LTM (final decision up to RAN1):

- a.	UE reports all measured LTM candidate cells in a single report; or

- b.	UE reports one or a subset of measured LTM candidate cell(s) in a report.

RAN2 to send an LS to RAN1 RAN3 RAN4, offline. Can also consider whether we should ask questions, continue in the offline [005]

In this document, we would like to revisit the agremments.

- Discussion

RAN2 received the following RAN1 agreements in RAN1 LS (R2-2302412/R1-2302194).

RAN1 has discussed the following configuration options for L1 measurement configurations for SSB till RAN1#112: - Option 1) Configurations for L1 measurement RS is provided under ServingCellConfig for the serving cells - is useful to reuses the mechanism for Rel-17 ICBM and necessary information to support inter-frequency measurement will be added there. - Option 2) Configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells - is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations for L1 measurement RS provided under CellGroupConfig for the candidate cells] - Option 3) Configurations for L1 measurement RS is provided under CellGroupConfig for the candidate cells - can achieve the similar benefit as Option 2) by directly referring to the candidate cell configurations.

As described in option 1) in the RAN1 LS, RAN1 indicates that it is benefitial to reuse the existing mechanism for L1 measurement RS configuration.

The introduction of the new L1 measurement configuration for LTM agreed upon by RAN2 has significant impact on RAN1, RAN2, RAN3 as described below.

- RAN2 must develop new signaling and corresponding procedure text to implement the new L1 measuremen RS configuration in the RRC specification. Although some draft implementation has been introduced in the LTM RRC running CR, it still takes more time to discuss and polish the implementation.

- RAN3 also needs to develop new signaling and and corresponding procedure text to support the new L1 measuremen RS configuration in the RAN3 specifications (e.g, 38.473 F1AP specification).  RAN3 has not started discussing how to support exchange of the new L1 measurement RS configurations for LTM between gNB-DUs. Therefore, RAN3 takes much more time to discuss and develop new siganling to support it.

- The existing CSI report framework for L1 measurement RS configuration and L1 measurement reporting is well-defined in the current RAN1 specifications. However, to support the new L1 measurement RS configuration for LTM, RAN1 must develop new L1 measurement reporting for the new L1 measurement RS configuration. Even RAN1 can reuse the existing L1 measurement reporting, RAN1 still needs to develop corresponding changes for the new L1 measurement RS configuration. Additionally, the RRC ASN.1 signaling design for the new L1 measurement RS configuration also needs input from RAN1.

From the above, we can see that the introduction of the new L1 measurement RS configuration causes a significant workload for all the three RAN working groups.

In addition, downscoping the LTM work item was discussed in the RAN#100 meeting because companies are concerned about completing the LTM work item on time. The RAN1 August meeting is the last meeting for RAN1 to complete Release 18 functions, so there is limited time for RAN1 to develop and complete their work for the new L1 measurement configuration and reporting. The introduction of the new L1 measurement RS configuration could delay the completion of the LTM work item.

Based on the above, we propose to reuse the existing L1 measurement RS configuration for LTM. This would save a significant amount of workload in the RAN working groups. As indicated in the RAN1 LS, only some necessary information needs to be added to support iner-frequncy measurement and it does not take much effort.

Proposal: The existing L1 measurement RS configration (i.e., CSI-MeasConfig) is reused for LTM instead of introducing the L1 measurement RS configuration for LTM.