3GPP TSG-RAN WG2 #123	R2-2308817

Toulouse, France, 21st – 25th August 2023

Source:	NTT DOCOMO, INC.

Title:	Open issues on Unified TCI framework extension

Document for:	Discussion and decision

Agenda Item:	7.20.3

- Introduction

In the last meeting, RAN2 reached following conclusions on Unified TCI framework extension [1].

RAN2 confirm the following working assumption as an agreement: - Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation. For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE, with the following high level design principles: - If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRP (i.e., as configured by RRC for both TRPs). - The following information can be indicated by the MAC CE (for joint DL/UL TCI mode): - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs, - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s) FFS for the separate DL/UL TCI mode.

This contribution provides our view on how we should progress in eUTCI discussion.

- Discussion

  - RRC parameter list from RAN1

RAN2 have received the RRC parameter list. Parameters related to eUTCI is cited in Annex A [2]. Listed parameters are:

- applyIndicatedTCIState-r18 (in ControlResourceSet)

- applyIndicatedTCIState-r18 (in PUCCH-Resource and PUCCH-ResourceGroup-r16)

- applyIndicatedTCIState-r18 (in ConfiguredGrantConfig)

- applyIndicatedTCIState-r18 (in CSI-AssociatedReportConfigInfo)

- applyIndicatedTCIState-r18 (in SRS-ResourceSet)

- tciSelection-PresentInDCI-r18 (in BWP-DownlinkDedicated)

- cjtSchemePDSCH-r18 (in ServingCellConfig)

There seems to be no functional difficulty, thus we think RAN2 can start to introduce these parameters.

Proposal 1.	Start discussion to introduce fields related to eUTCI to reflect parameters listed in RAN1 parameter list.

  - UE feature list from RAN1

RAN2 have received the UE feature list. Features related to eUTCI is cited in Annex B [3]. However, all the features include FFSes to be discussed more in RAN1. We think RAN2 can wait for RAN1 to address FFS.

Proposal 2.	RAN2 wait for RAN1 progress before starting discussion on implementation of UE features.

  - New MAC CE for sDCI based mTRP

As said in Introduction, RAN2 already agreed to introduce a new TCI State (de)activation MAC CE. RAN1 made following agreement about requirement on MAC CE.

RAN1#112bis-e [4]

Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design) - The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint

In short;

Observation 1.	A new MAC CE shall include information below:

1. Which TCI States this MAC CE indicates, i.e.;

- for a serving cell configured with joint TCI mode, all, or sub-set of {first joint TCI state, second joint TCI state}.

- for a serving cell configured with separate TCI mode, all, or sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}.

2. Mapping of each TCI State to the existing TCI field in a DCI format 1_1/1_2.

In the last RAN2 meeting, we agreed requirements for a new MAC CE when the serving cell is joint TCI State mode [1].

RAN2 confirm the following working assumption as an agreement: - Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation. For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE, with the following high level design principles: - If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRP (i.e., as configured by RRC for both TRPs). - The following information can be indicated by the MAC CE (for joint DL/UL TCI mode): - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs, - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s) FFS for the separate DL/UL TCI mode.

To make discussion progress one more step, we believe it helps to agree on requirements for a new MAC CE when the serving cell is separate UL/DL TCI State mode.

Proposal 3.	RAN2 confirm that a new MAC CE for serving cell configured with separate UL/DL TCI States shall include;
1. which TCI States this MAC CE indicates, i.e., all, or sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}
2. mapping of each TCI State to the existing TCI field in a DCI format 1_1/1_2

By the way, the existing MAC CE for activation/deactivation of joint TCI States is specified as a figure below [5]. Note that Pi indicates whether i-th TCI codepoint includes separate UL and DL TCI States or a single DL/joint TCI State and U/L indicates whether the TCI state ID in the same octet is for joint/downlink or uplink TCI state. These fields are not sufficient to accommodate the granularity for multi-TRP, i.e., unable to indicate first or second TCI State. That was why RAN2 agreed to introduce a new MAC CE.

Figure 6.1.3.47-1: Unified TCI state activation/deactivation MAC CE

However, we would like to note that the requirement differs depending on the serving cell is configured of joint TCI or separate TCI. If we use separate MAC CEs for the two cases, we can introduce a new MAC CE for joint TCI, without increasing number of necessary Octets compared with the existing unified TCI state activation/deactivation MAC CE. (Of course, MAC CE for separate TCI State will be still FFS.)

Proposal 4.	RAN2 discuss whether separate MAC CEs are introduced for joint TCI State and separate DL/UL TCI States respectively.

- Summary and proposal

Proposal 1.	Start discussion to introduce fields related to eUTCI to reflect parameters listed in RAN1 parameter list.

Proposal 2.	RAN2 wait for RAN1 progress before starting discussion on implementation of UE features.

Observation 1.	A new MAC CE shall include information below:

1. Which TCI States this MAC CE indicates, i.e.;

- for a serving cell configured with joint TCI mode, all, or sub-set of {first joint TCI state, second joint TCI state}.

- for a serving cell configured with separate TCI mode, all, or sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}.

2. Mapping of each TCI State to the existing TCI field in a DCI format 1_1/1_2.

Proposal 3.	RAN2 confirm that a new MAC CE for serving cell configured with separate UL/DL TCI States shall include;
1. which TCI States this MAC CE indicates, i.e., all, or sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}
2. mapping of each TCI State to the existing TCI field in a DCI format 1_1/1_2

Proposal 4.	RAN2 discuss whether separate MAC CEs are introduced for joint TCI State and separate DL/UL TCI States respectively.

- References

[1]	R2-2306552, “Report from NR MIMO evolution session,” Session Chair (CATT), RAN2#122.

[2]	R1-2306270, “LS on Rel-18 higher-layers parameter list,” RAN1#113.

[3]	R1-2306225, “LS on Rel-18 RAN1 UE features list for NR after RAN1#113,” RAN1#113.

[4]	“Final Report of 3GPP TSG RAN WG1 #112bis-e v1.0.0,” RAN1.

[5]	TS 38.321, V17.5.0.

Annex A: RRC parameter list from RAN1

| WI code | Sub-feature group | RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range | Required for initial access or IDLE/INACTIVE | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_MIMO_evo_DL_UL-Core | mTRP-UTCI | ControlResourceSet | applyIndicatedTCIState-r18 | New | This parameter is used to inform the UE shall apply the first, the second, both, or none of the indicated joint/DL TCI states to PDCCH reception(s) a CORESET for S-DCI based MTRP operation | {the first, the second, both, none} | UE-specific | From RAN1 perspective, it is up to RAN2 to introduce a new RRC parameter with values {the first, the second, both} and use it along with the legacy parameter followUnifiedTCIstate or introduce a new RRC parameter with values {the first, the second, both, none} without using the legacy parameter followUnifiedTCIstate. |
| NR_MIMO_evo_DL_UL-Core | mTRP-UTCI | PUCCH-Resource and PUCCH-ResourceGroup-r16 | applyIndicatedTCIState-r18 | New | This parameter is used to inform the UE shall apply the first, the second, or both of the indicated joint/UL TCI states to PUCCH transmission(s) corresponding to a PUCCH resource or PUCCH resource group. For M-DCI based MTRP operation, the candidate values can be {the first, the second}, and the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. | {the first, the second, both} | UE-specific |  |
| NR_MIMO_evo_DL_UL-Core | mTRP-UTCI | ConfiguredGrantConfig | applyIndicatedTCIState-r18 | New | This parameter is used to inform the UE shall apply the first, the second, or both of the indicated joint/UL TCI states to PUSCH transmission(s) corresponding a Type1-CG configuration. For M-DCI based MTRP operation, the candidate values can be {the first, the second}, and the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. | {the first, the second, both} | UE-specific |  |
| NR_MIMO_evo_DL_UL-Core | mTRP-UTCI | CSI-AssociatedReportConfigInfo | applyIndicatedTCIState-r18 | New | This parameter is used to inform the UE shall apply the first or the second indicated joint/DL TCI states to an aperiodic CSI-RS resource or an aperiodic CSI-RS resource set. For M-DCI based MTRP operation, the first and the second indicated joint/DL TCI states correspond to the indicated joint/DL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. | {the first, the second} | UE-specific |  |
| NR_MIMO_evo_DL_UL-Core | mTRP-UTCI | SRS-ResourceSet | applyIndicatedTCIState-r18 | New | This parameter is used to inform the UE shall apply the first or the second indicated joint/UL TCI states to an SRS resource set. For M-DCI based MTRP operation, the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. | {the first, the second} | UE-specific | When two SRS resource sets for CB/NCB are configured, the UE does not expect the following 。 to be configured with the first indicated UL/joint TCI state which is to be applied to the second SRS resource set 。 to be configured with the second indicated UL/joint TCI state which is to be applied to the first SRS resource set |
| NR_MIMO_evo_DL_UL-Core | mTRP-UTCI | BWP-DownlinkDedicated | tciSelection-PresentInDCI-r18 | New | This parameter is used to indicate if [TCI selection field] is present or absent in DCI format 1_1 and DCI format 1_2 for a DL BWP | enabled | UE-specific |  |
| NR_MIMO_evo_DL_UL-Core | mTRP-UTCI | ServingCellConfig | cjtSchemePDSCH-r18 | New | This parameter is used to configure CJT Tx scheme for PDSCH reception | enabled | UE-specific |  |

Annex B: UE feature list from RAN1

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-1-1 | Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC | FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: separate row for TRP-specific BFR with unified TCI framework FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-1a | Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC | FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated joint TCI states per CC |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-2 | Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC | FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated DL TCI states per CC FFS: maximum number of activated UL TCI states per CC FFS: separate row for TRP-specific BFR with unified TCI framework |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-2a | Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC | FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-3 | Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP | FFS: separate rows for resource set |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-4 | Support of two TCI states for CJT Tx scheme for PDSCH | FFS: how to signal QCL subset |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-7 | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-7a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-9 | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-9a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-12 | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-13 | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP |  |  |  |  |  |  |  |  |  |  |  |