3GPP TSG-RAN WG2 Meeting #123											   R2-2308819

Toulouse, France, 21-25 Aug, 2023

Source: 	Xiaomi

Title:  	Discussion on subsequent CPAC

Agenda Item:	7.4.3

Document for:	Discussion and Decision

Introduction

A WID on Further NR mobility enhancements [1] has been approved. In the WID, one of the objectives is to specify mechanism and procedures of NR-DC with selective activation of the cell groups, as described below:

2. To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements: - To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4] Note 5: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

In RAN2#122 meeting, RAN 2 agreed terminology is “Subsequent CPAC”. In this contribution, we provide our understandings for the remaining issues of selective activation of the cell groups (i.e. subsequent CPAC).

Discussion

  - Configuration of subsequent CPAC

1) ASN.1 structure of Subsequent CPAC

In the running CR of subsequent CPAC [2], Rel-16 ConditionalReconfiguration IE has been reused for the configuration of subsequent CPAC. For Rel-16/17 CPC/CPA, UE should remove all conditional configurations after PSCell change/addition, but UE keep these configurations for Rel-18 subsequent CPAC. When using the same IE for the configuration of Rel-16/17 CPC/CPA and Rel-18 subsequent CPAC, UE cannot know whether to remove the candidate configuration after PSCell change/addition or not.

Observation 1: For the configuration of subsequent CPAC, using the legacy ConditionalReconfiguration IE without any change might result in the misunderstanding in UE, i.e. whether the candidate configuration can be used for Rel-18 subsequent CPAC; i.e. whether to keep the candidate configuration after PSCell change/addition or not.

To solve the above issue, a new indicator can be introduced in Rel-16 ConditionalReconfiguration IE to indicate whether the candidate configuration can be used for Rel-18 subsequent CPAC.

For the flexible conditional configuration, a separate indicator per candidate PSCell can be introduced, which can be used to support the coexistence of legacy CPC/CPA and subsequent CPAC, i.e. legacy CPC/CPA and Rel-18 subsequent CPAC can be configured simultaneously in the Rel-16 ConditionalReconfiguration.

Proposal 1: A new indicator per candidate can be introduced in the conditional reconfiguration to identify whether the associated candidate configuration is used for subsequent CPAC or not.

2) Conditions of Subsequent CPAC

In RAN2 #121 meeting [3], the following scenarios have been supported for subsequent CPAC:

- SN initiated intra-SN SCG selective activation.

- MN initiated inter-SN SCG selective activation.

- SN initiated inter-SN SCG selective activation.

For intra-SN, the conditions of subsequent CPAC should be generated by SN, which is same with Rel-16 intra-SN CPC.

For inter-SN subsequent CPAC, RAN2 has discussed which nodes to generate the execution conditions for initial and subsequent CPAC. RAN2 reached the following agreements [4-5]:

- For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC.

FFS on the following options for subsequent CPC:

Option 1: Source MN generates the execution conditions for all subsequent CPC.

Option 2: Candidate SN may generate execution conditions for subsequent CPC.

- For SN initiated inter-SN SCG selective activation source SN generates the execution conditions for the initial CPC.

- For SN initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC.

For MN initiated subsequent CPAC, whether candidate SN can generate execution conditions for subsequent CPC is FFS.

In the procedure of Rel-18 subsequent CPAC, the serving MN will not be changed, and if the serving MN is changed, the configurations of subsequent CPAC should be released. Hence, the serving MN can always deeply involve the procedure of subsequent CPAC. To align with Rel-17 MN initiated CPC/CPA, execution conditions are always generated by the serving MN for MN initiated inter-SN subsequent CPAC.

Proposal 2: For MN initiated inter-SN subsequent CPAC, the serving MN generates the execution conditions for the initial CPAC and all subsequent CPC.

In RAN2#121bis [4], for MN initiated procedure, execution conditions based on event A4 are supported. Whether A3/A5 are supported is FFS. In WID [1], subsequent CPAC can be used for frequent SCG changes to make better coverage. Hence, event A4 configured for load balancing is not appropriate for subsequent CPAC. Based on measurement results from the initial source SN and/or UE, source MN can configure suitable parameters of event A3/A5 for UE to support CPC in subsequent CPAC.

Proposal 3: For MN initiated inter-SN subsequent CPAC, source MN can configure event A3/A5 for CPC.

In RAN2#122 [5], RAN2 has agreed for SN initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC. But how to provide the conditions generated by candidate SN to UE via RRC signalling is FFS. There are two options can be considered for the configuration of execution conditions generated by candidate SN.

- Option 1: Using nested configuration to configure the execution conditions generated by candidate SN.

For option 1, the execution conditions can be included in the candidate cell configuration (i.e. the RRCReconfiguration associated with the candidate PSCell), and the execution conditions for subsequent CPC are contains in the conditionalReconfiguration included in the RRCReconfiguration, which is the change provided in the Running CR [2].

- Option 2: Introducing a new field in CondReconfigToAddMod IE to include a list of execution conditions generated by candidate SN. An exemplary ASN.1 structure is shown as below:

CondReconfigToAddModList information element

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=       SEQUENCE {

condReconfigId-r16                    CondReconfigId-r16,

condExecutionCond-r16                 SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16                   OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17              OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]],

[[

condExecutionCondSubsequentCPCList-r18 SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigExecCondSCG-r17 OPTIONAL     -- FFS

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP

Each condition included in the list is associated with one other candidate cell, and when the candidate cell becomes the serving cell, the condition can be used for subsequent CPC.

Proposal 4：There are two options can be considered for the configuration of execution conditions generated by candidate SN for subsequent CPC.

- Option 1: Using nested configuration to configure the execution conditions generated by candidate SN.

- Option 2: Introducing a new field in CondReconfigToAddMod IE to include a list of execution conditions generated by candidate SN.

3) Reference configuration

To support delta configuration, RAN2 introduced a separate reference configuration for subsequent CPAC and LTM. There are still some remaining issues for the reference configuration in LTM and subsequent CPC. And the similar issues are discussing in LTM. For subsequent CPAC, we can wait more progress on LTM CR [6] and the ASN.1 structure and RRC procedure can follow LTM principle.

Proposal 5：For the reference configuration of subsequent CPAC, the ASN.1 structure and RRC procedure can follow LTM principle and wait more progress on LTM CR.

  - CPA for subsequent CPAC

In previous meetings, RAN2 has agreed to support “CPA” selective activation of cell groups and a CPA conditional configuration can be used for CPC (but with different triggering conditions).

For CPA in Rel-18 subsequent CPAC, there are some issues can be discussed:

1) Whether to support subsequent CPA.

To support subsequent CPA, UE can perform PSCell addition without reconfiguration and re-initiation of CPA which can reduce signalling overhead (i.e. maintaining candidate PSCell configurations for CPA after SCG release.).

For subsequent CPA, UE will not autonomously remove subsequent CPA configurations when SCG release. If it is needed, the network can initiate the removal of subsequent CPAC configurations by the explicit RRC indication (i.e. condReconfigToRemoveList)

Proposal 6: Subsequent CPA can be supported and UE can maintain conditional configurations for subsequent CPA after SCG release in Rel-18.

2) How to configure the CPA conditional configuration with different triggering conditions (for CPA and CPC)

According to the scenarios of subsequent CPAC, the following cases can be considered:

- Case 1: The CPA conditional configuration can be used for MN initiated CPA and SN initiated CPC.

- Case 2: The CPA conditional configuration can be used for MN initiated CPA and MN initiated CPC.

In the current ASN.1 structure for conditional reconfiguration, each conditional configuration can include one condition from MN or one condition from SN associated with the candidate.

For case 1, we need to achieve “the condition from MN and the condition from SN can be configured simultaneously in one conditional reconfiguration associated with the same candidate PSCell.” And the current ASN.1 structure of CondReconfigToAddMod without any change can achieve case 1.

However, for case 2, current fields in CondReconfigToAddMod cannot be used to configure different conditions generated by MN for CPC and CPA separately. Hence, a new field is needed. Similar to R16 condExecutionCond, the new field can also be used to configure a condition generated by MN.

To use a unified method for case 1 and case 2, a new field can be introduced in CondReconfigToAddMod to configure the CPA condition for CPA with different triggering conditions. And legacy fields can be used to configure CPC condition generated by MN or SN.

The following ASN.1 example can be considered:

CondReconfigToAddModList information element

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]],

[[

condExecutionCondCPA-r18         SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- FFS

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP

The condExecutionCondCPA can be introduced to configure the CPA condition for the case where one conditional configuration with two different conditions. The condExecutionCondCPA also is generated by MN and the associated measId(s) refer to the measConfig associated with MCG.

To support one conditional configuration used for CPA and CPC with different trigger conditions, the legacy field (i.e. condExecutionCond and condExecutionCondSCG) is used to configure the CPC condition and the new field (i.e. condExecutionCondCPA) can be used to configure CPA conditions.

Proposal 7: A new field can be introduced in CondReconfigToAddMod IE to support one conditional configuration including one CPA condition and one CPC condition. The legacy field (i.e. condExecutionCond or condExecutionCondSCG) is used to configure the CPC condition and the new field is used to configure the CPA condition.

  - Release of subsequent CPAC

In the current specification, the UE may receive two independent conditionalReconfiguration in NR-DC [7]:

-	a conditionalReconfiguration associated with MCG, that is included in the RRCReconfiguration message received via SRB1; and

-	a conditionalReconfiguration, associated with SCG, that is included in the RRCReconfiguration message received via SRB3, or, alternatively, included within a RRCReconfiguration message embedded in a RRCReconfiguration message received via SRB1.

-	the UE maintains two independent VarConditionalReconfig, one associated with each conditionalReconfiguration;

In Rel-18, both inter-SN and intra-SN subsequent CPAC are supported and can be configured by MCG conditionalReconfiguration and SCG conditionalReconfiguration separately. And inter-SN subsequent CPC configuration and subsequent CPA configuration can be stored in MCG VarConditionalReconfig and intra-SN subsequent CPC configuration can be stored in SCG VarConditionalReconfig.

For the following cases, whether to release or maintain subsequent CPAC configuration can be discussed:

- Case 1: Upon SCG release

For intra-SN subsequent CPC, all configurations are generated by the serving SN. Hence, UE should remove all associated conditional configurations for intra-SN subsequent CPC upon SCG release as in legacy. The legacy SCG release procedure can be reused and UE shall remove all the entries within the SCG VarConditionalReconfig upon SCG release.

For inter-SN subsequent CPC and subsequent CPA, the conditional configurations are generated by the serving MN and included in MCG VarConditionalReconfig. Maybe some of these conditional configurations can continue to be used for subsequent CPA and future subsequent CPC. Hence, UE will not autonomously remove the condition configurations for subsequent CPA and inter-SN subsequent CPC. If needed, the network can initiate the removal of subsequent CPAC configuration by the explicit RRC indication (i.e. condReconfigToRemoveList-r16).

Proposal 8: UE should remove all conditional configurations for SN initiated intra-SN subsequent CPC, i.e. UE shall remove all the entries within the SCG VarConditionalReconfig upon SCG release.

Proposal 9: Upon SCG release, UE shall not autonomously remove the condition configurations for subsequent CPA and inter-SN subsequent CPC, which are associated with MCG VarConditionalReconfig. And the network can explicitly remove these conditional configurations via condReconfigToRemoveList, if needed.

- Case 2: Upon PCell change

In previous meeting, RAN2 has agreed:

- In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case).

For inter-MN PCell change, the CPA/CPC configuration for subsequent CPAC should be released. But for the intra-MN PCell change, due to the unchanged MN, UE could maintain these CPA/CPC configurations for subsequent CPAC. To achieve the removal of CPC/CPA configurations upon inter-MN PCell change, the legacy indication (i.e. condReconfigToRemoveList) can be reused.

Proposal 10: Upon PCell change, whether to keep or release subsequent CPAC configurations is up to the network configuration and the legacy indication (i.e. condReconfigToRemoveList) can be reused to achieve the removal of subsequent CPAC configurations for inter-MN PCell change.

- Case 3: Upon RRC reestablishment

Upon the initiation of RRC re-establishment, if CHO recovery is triggered, whether to remove subsequent CPAC configurations can follow the principle upon PCell change.

If UE initiates the RRC re-establishment and CHO recovery is not triggered, UE shall remove all configurations for subsequent CPAC, which is similar to the legacy procedure without spec impact.

Proposal 11: If UE initiates the RRC re-establishment and CHO recovery is not triggered, UE shall remove all configurations of subsequent CPAC, which is similar to the legacy CPAC.

- Case 4: Upon going to RRC_IDLE/RRC_INACTIVE

As in the legacy CPAC, it is nature to release all conditional configurations configured for subsequent CPAC upon going to RRC_IDLE/RRC_INACTIVE.

Proposal 12: Upon going to RRC_IDLE/RRC_INACTIVE, UE should remove all subsequent CPAC configurations, which is similar to the legacy CPAC.

  - Security issues for subsequent CPAC

Security issues for subsequent CPAC have been discussed in previous meetings, and RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security. It is necessary to avoid key reuse in subsequent CPAC without RRC Reconfiguration.

SA3 wants to update on the state of the security analysis on selective SCG.

SA3 has considered different alternative solutions to address the issue and concluded to have a SN Counter based solution for fresh key derivation for selective SCG activation.

Following is the solution details from security perspective:

1. The MN provides to the UE the SN Counter values for each SN, and the UE stores these values along with the CPC.

2. The UE derives the KSN using the KgNB together with an unused SN Counter value pre-provisioned by the MN. The UE changes the SN Counter upon every SN change.

SA3 is offering a joint conference call that can be used to discuss and to enhance the above described solution details.

SA3 also discussed the security issue to avoid key reuse in subsequent CPAC, and provided the solution in SA3 LS [8]:

For the solution provided by SA3, for each candidate SN, the MN provides to the UE the SN Counter values. A SN Counter value list associated with each SN can be included in ConditionalReconfiguration. Upon the execution of inter-SN subsequent CPAC, the UE will use an unused counter value in the SN Counter value list associated with the candidate SN in sequence, to update the security key KSN.

Proposal 13: The SN Counter value list associated with each SN can be included in the ConditionalReconfiguration.

Proposal 14: Upon SN change, UE will use the SN Counter value in the list in sequence, to update the security key of the associated candidate SN.

However, the SN Counter value list is associated with candidate SN not candidate configuration. UE cannot know which candidate cells for subsequent CPAC are associated with the same or different candidate SN. In other words, UE cannot know the association between the SN Counter values and the candidate cells.

To associate the SN Counter value list and candidate cell configurations, a method similar to the indicator for L2 reset in LTM [7] can be considered, and candidate configuration sets can be introduced and one or more candidate configurations are inside in the same set associated with the same candidate SN (i.e. associated with the same SN Counter value list).

A configuration set ID can be included in each conditional reconfiguration (i.e. CondReconfigToAddMod) associated with each candidate cell to indicate whether the UE should perform SN key update upon the execution of subsequent CPAC. If the configuration set ID for the candidate cell is the same as the configuration set ID for the serving cell, the UE shall not perform SN key update, otherwise, UE shall derive a new KSN.

Proposal 15: Candidate configuration set IDs can be introduced to determine if to perform security key update. Upon execution of subsequent CPAC, if the set ID for the candidate cell is the same as the set ID for the serving cell, the UE shall not perform SN key update, otherwise, UE shall derive a new KSN.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Configuration of subsequent CPAC

Observation 1: For the configuration of subsequent CPAC, using the legacy ConditionalReconfiguration IE without any change might result in the misunderstanding in UE, i.e. whether the candidate configuration can be used for Rel-18 subsequent CPAC; i.e. whether to keep the candidate configuration after PSCell change/addition or not.

Proposal 1: A new indicator per candidate can be introduced in the conditional reconfiguration to identify whether the associated candidate configuration is used for subsequent CPAC or not.

Proposal 2: For MN initiated inter-SN subsequent CPAC, the serving MN generates the execution conditions for the initial CPAC and all subsequent CPC.

Proposal 3: For MN initiated inter-SN subsequent CPAC, source MN can configure event A3/A5 for CPC.

Proposal 4：There are two options can be considered for the configuration of execution conditions generated by candidate SN for subsequent CPC.

- Option 1: Using nested configuration to configure the execution conditions generated by candidate SN.

- Option 2: Introducing a new field in CondReconfigToAddMod IE to include a list of execution conditions generated by candidate SN.

Proposal 5：For the reference configuration of subsequent CPAC, the ASN.1 structure and RRC procedure can follow LTM principle and wait more progress on LTM CR.

CPA for subsequent CPAC

Proposal 6: Subsequent CPA can be supported and UE can maintain conditional configurations for subsequent CPA after SCG release in Rel-18.

Proposal 7: A new field can be introduced in CondReconfigToAddMod IE to support one conditional configuration including one CPA condition and one CPC condition. The legacy field (i.e. condExecutionCond or condExecutionCondSCG) is used to configure the CPC condition and the new field is used to configure the CPA condition.

Release of subsequent CPAC

Proposal 8: UE should remove all conditional configurations for SN initiated intra-SN subsequent CPC, i.e. UE shall remove all the entries within the SCG VarConditionalReconfig upon SCG release.

Proposal 9: Upon SCG release, UE shall not autonomously remove the condition configurations for subsequent CPA and inter-SN subsequent CPC, which are associated with MCG VarConditionalReconfig. And the network can explicitly remove these conditional configurations via condReconfigToRemoveList, if needed.

Proposal 10: Upon PCell change, whether to keep or release subsequent CPAC configurations is up to the network configuration and the legacy indication (i.e. condReconfigToRemoveList) can be reused to achieve the removal of subsequent CPAC configurations for inter-MN PCell change.

Proposal 11: If UE initiates the RRC re-establishment and CHO recovery is not triggered, UE shall remove all configurations of subsequent CPAC, which is similar to the legacy CPAC.

Proposal 12: Upon going to RRC_IDLE/RRC_INACTIVE, UE should remove all subsequent CPAC configurations, which is similar to the legacy CPAC.

Security issues for subsequent CPAC

Proposal 13: The SN Counter value list associated with each SN can be included in the ConditionalReconfiguration.

Proposal 14: Upon SN change, UE will use the SN Counter value in the list in sequence, to update the security key of the associated candidate SN.

Proposal 15: Candidate configuration set IDs can be introduced to determine if to perform security key update. Upon execution of subsequent CPAC, if the set ID for the candidate cell is the same as the set ID for the serving cell, the UE shall not perform SN key update, otherwise, UE shall derive a new KSN.

Reference

[1] RP-231475, MediaTek Inc., “Revised WID on Further NR mobility enhancements”

[2] R2-230xxxx, “38.331 Running CR for selective activation of SCGs for NR-DC”

[3] R2-2302401, “Report of 3GPP TSG RAN WG2 meeting #121”

[4] R2-2306553, “Report of 3GPP TSG RAN WG2 meeting #121bis-e”

[5] R2-xxxxxxx, “Report of 3GPP TSG RAN WG2 meeting #122”

[6] R2-xxxxxxx, “RRC running CR for LTM”

[7] 3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”

[8] S3-233200, “LS on Security Solution for Selective SCG”