3GPP TSG-RAN WG2 Meeting #123	R2-2308823

# Toulouse, France, 21– 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | draft | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.306 running CR for mobile IAB capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mobile_IAB-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introducing Rel-18 mobile IAB feature to 38.306. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - mobile-IAB-r18 capability added to 4.2.15.2 - Add note to 4.2.15.9 and 4.2.15.10 to indicate that MR-DC and NR-DC related parameters shall be ignored by an IAB node indicating mobile-IAB-r18. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mobile IAB feature cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.15.2, 4.2.15.9, 4.2.15.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.331 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

#### 4.2.15.2	General Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| bh-RLF-DetectionRecovery-Indication-r17 Indicates whether the IAB-MT supports BH RLF detection indication and BH RLF recovery indication handling as specified in TS 38.340 [23] | IAB-MT | No | No | No |
| bh-RLF-Indication-r16 Indicates whether the IAB-MT supports BH RLF indication handling as specified in TS 38.331 [9] and in TS 38.340 [23] | IAB-MT | No | No | No |
| directSN-AdditionFirstRRC-IAB-r16 Indicates whether the IAB-MT supports direct SN addition in the first RRC connection reconfiguration after RRC connection establishment. | IAB-MT | No | No | No |
| mobile-IAB-r18 Indicates whether the IAB-MT supports mobile IAB functionality as specified in TS 38.300 [28]. An IAB-MT with this capability shall provide mobile-IAB indication in Msg5. | IAB-MT | No | No | No |

Next Modified Subclause

#### 4.2.15.9	MR-DC Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| f1c-OverEUTRA-r16 Indicates whether the IAB-MT supports F1-C signalling over DLInformationTransfer and ULInformationTransfer messages via MN when IAB-MT operates in EN-DC mode, as specified in TS 36.331 [17]. | IAB-MT | No | No | No |
| scg-DRB-NR-IAB-r16 Indicates whether the IAB-MT supports SCG DRB with NR PDCP when IAB-MT operates in EN-DC mode. | IAB-MT | No | No | No |
| interNR-MeasEUTRA-IAB-r16 Indicates whether the IAB-MT supports NR measurement and reports while in EUTRA connected and event B1-based measurement and reports while in EUTRA connected. | IAB-MT | No | No | No |

NOTE:	In this release of the specification, if an IAB-node indicates support for mobile-IAB-r18, MR-DC parameters shall be ignored.

#### 4.2.15.10	NRDC Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| f1c-OverNR-RRC-r17 Indicates whether the IAB-MT supports F1-C signalling over DLInformationTransfer and ULInformationTransfer messages via MN when IAB-MT operates in NR-DC and MN is the non-F1-termination node or via SN when IAB-MT operates in NR-DC and SN is the non-F1-termination node, as specified in TS 38.401 [33] and TS 37.340 [7]. | IAB-MT | No | No | No |
| simultaneousRxTx-IAB-MultipleParents-r17 Indicates the support of simultaneous transmission and reception of an IAB-node from multiple parent nodes. | BC | No | No | No |

NOTE:	In this release of the specification, if an IAB-node indicates support for mobile-IAB-r18, NR-DC parameters shall be ignored.

End of Changes

# Annex: Agreements from RAN2 meetings

Relevant agreements are highlighted.

RAN2#119:

The following Points are Endorsed, i.e. for the plan for next meeting (after one round of discussion at R2 119-e):

- P1: RAN2 to discuss scenarios, if and where enhancements to cell (re-)selection to/from the mobile IAB-node apply, e.g. based on mobile IAB-node broadcast parameter (this point doesn’t preclude other potential usage of Bcast info).

- P2: Can discuss whether The mobile IAB-MT need to send a mobile-IAB indication (capability or mobility) to the IAB-donor-CU,

- P3: For “dual-DU-way” of doing full migration, RAN2 may discuss whether the legacy UE should see the two logical cells/DUs as separate or same physical cell(s), and what procedure(s) the legacy UE needs to perform in either case.

- P4: RAN2 may discuss whether there are issues with PCI partitioning that needs to/can be addressed (to be used in applicable scenario), if any found within R2 scope. May discuss need for and feasibility from R2 point of view of a dynamic PCI change mechanism. May also discuss whether enhancements to/vs current UE/MT reporting are useful/necessary to improve PCI collision detection.

- P5: RAN2 may discuss whether there is a problem of RACH configuration collision between mobile IAB and stationary network from RAN2 perspective and/or whether RAN2 should ask RAN1 to consider RAN1-related aspects.

The method of not broadcasting “iab-Support” indication, is sufficient to prevent other IAB-node from accessing mobile IAB (without further spec impact).

R2 assumes RACH-less procedure may be considered for on-board RRC_CONNECTED UEs, which are to be handed over together with the mobile IAB-node (would depend also on the assumptions for UL synch).

R2 assumes that CHO or delayed RRC config could be the baseline for group mobility (FFS if could be applicable for mobility of IAB MT), i.e. with a preparation in advance (not immediately) of the execution.

R2 assumes that Mobile IAB connecting to a stationary (intermediate) IAB node is/can be supported. R2 assumes this can be supported with no (or limited) impact.

RAN2#119bis:

UE capability signalling is the baseline to let CU know that the MT is a “mobile-IAB” type. FFS early mobile-IAB indication, e.g. in Msg5.

Regarding moving status/mode indication, R2 observes that legacy reporting of mobility state (e.g. mobilityState-r16) could be reused, and maybe also current location reporting from the UE. FFS whether any of this need to be enhanced or complemented, e.g. for the potential purpose of predictive mobility.

FFS if to Introduce that stationary network broadcasts indication of “supporting mobile-IAB” (into intended for the Mobile IAB MT)

RAN2 confirms that Mobile IAB need to work with legacy UEs.

RAN2 observes that a UE could potentially consider itself on-board of a mobile-IAB cell, if the UE camps on/connects to a mobile IAB cell during a long period (i.e. the UE then need to know that this is such a cell). FFS the time. FFS if this is needed.

RAN2 assume below for the UEs working in the mobile IAB cell (may be obvious):

- Assumption 1: From the NW perspective of mobile-IAB cell, the principle of setting the legacy parameters (including cell (re)selection, cell reservations and access restrictions) does not change, compared to the legacy IAB cell.

- Assumption 2: No spec impact to legacy UEs behaviors.

- Assumption 3: Any R18 newly broadcasted info of mobile-IAB cell (if agreed) does not forbid/control the access of legacy UEs.

- Assumption 4: Non-enhanced UEs (including legacy UEs and R18 UEs not supporting the enhancement) just ignore the R18 newly broadcasted info of mobile-IAB cell (if agreed).

RAN2 assumption: For the mobile IAB cell broadcasting info:

- 1 bit mobile-IAB cell type indication is introduced, to assist mobility in Idle/Inactive mode for Rel-18 UEs (FFS if to assist UE to know it is onboard, if this need to be known)

- FFS how this is used (might be implementation specific).

RAN2 has from the Mobile IAB WI perspective not identified any modifications to prevent the surrounding UE from accessing the mobile IAB-node, but believes that SA2 may be working on Rel-18 solutions that may be applicable (wait for SA2)

RAN2 assumes that O1 and O3 above could work, and FFS if O2 above (new trigger etc) is needed.

RAN2 focuses on the scenario where, during full migration, the UE sees the two logical DU cells as different physical cells (e.g. with different PCI if same carrier), and where the two logical DU cells use separate physical resources (i.e., different carriers, or orthogonal time and frequency resources of the same carrier, as supported by legacy L1).

No LS is needed

RAN2#120:

R2 assumes that It is up to RAN3 or SA2 to decide whether to support early mobile IAB indication in Msg5 because it depends whether donor CU needs to select an AMF supporting mobile IAB.

R2 assumes that Donor CU can determine mobile IAB node's moving status via legacy reporting (e.g. mobility state and UE location / velocity specified in SON/MDT), i.e. R2 assumes enhanced / new reporting is not needed.

A mobile IAB node may camp on and connect to legacy Rel-16/Rel-17 IAB capable cell.

R2 assumes "supporting mobile-IAB" indication is provided by Rel-18 Mobile IAB capable parent cell.

Regarding the assumed mobile-IAB cell type indication, RAN2 assumes is may be specified if some related UE behaviour is specified.

RAN2 assumes that PCI collision can be avoided, by reconfigurations, and this may be handled by RAN3. If RAN3 finds issues that RAN2 should work on then RAN2 can work. e.g. based on LS.

RAN2 understands that RACH interference and collisions may be avoided by RACH configuration, and RACH configurations can e.g. be exchanged by Xn, so RACH interference and collisions better be handled between RAN3 and RAN1, if needed.

RAN2#121:

Postponed, AMF selection in the base-station is a Ran3 function, Ran2 expect RAN3 to ask for it if support for this is needed

Working Assumption: support to have UE prioritization in cell reselection for mIAB cell(s), at least for inter-frequency cell-reselection.

FFS if UE search and measure for mIAB cells on different frequencies is unspecified (autonomous search), FFS if such search can be done without assistance frequency information.

For the upstream data handling at the BAP of mobile IAB MT, one common default BAP configuration to be used by both logical DUs is the baseline. RAN2 to further discuss the need of using logical-DU-specific default BAP configuration (e.g. when the two logical DUs use different donor-DUs).

For the upstream data handling at the BAP of mobile IAB MT, RAN2 assume that the F1AP BAP configuration for each logical DU should be configured/controlled by the DU’s respective donor-CU via the corresponding F1AP connection (To be confirmed by RAN3).

For the downstream data handling arriving at the mobile IAB node, RAN2 assume upper layers (e.g. IP layer) can differentiate the data to different logical DUs based on e.g. the IP address, i.e. no need to introduce logical-DU-specific BAP address. (To be confirmed by RAN3).

RAN2#121bis:

RACH-less for mIAB scenario, if agreed in the end, will cover only the case of same-TA.

Feasibility of beam handling during RACH-less HO in the mIAB WI is FFS (and this need to be addressed for RACH-less to be supported for mIAB).

RAN2 discuss further the following options to support beam operation for the first UL transmission/DL reception towards the target logical DU in RACH-less HO during DU migration:

- Option 1: (Explicit approach) Explicit beam information is included in HO command. FFS the details.

- Option 2: (Implicit approach) UE re-uses the same beam status as in the source cell (the beam information is not carried explicitly in HO command).

RACH-less HO with same TA with security key change is in scope for served UEs during mIAB DU migration. FFS UL grant and HO completion procedure in mIAB RACH-less HO.

FFS: May support CHO with CondT1 if it is “for free”, i.e. if TS impact is just to slightly modify the description to make it also applicable to TN.

The mobile IAB-MT to include a mobile-IAB indication in Msg. 5.

R2 assumes that a mobile IAB node is not required to receive the system information of neighbour cells for reporting of measurements (i.e. it will not refrain from reporting measurements of cells that are not broadcasting the “mobile iab Support” indication, and this is acc to current R2 TS).

R2 clarifies that A donor broadcasting the “supporting mobile-IAB” indication first checks the UE capability of an IAB node before configuring child nodes for the IAB node or sending a handover request for the node, no impact to RAN2 TS.

RAN2#122:

RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured).

For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report).

for the UL grant and HO completion in RACH-less HO:

- 1. Both type-1 configured grant and dynamic grant are supported

- 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI).

Send LS to RAN3 to check whether there are issues / feasibility concerns

We will send LS, will just ask SA2 to provide more details using CAG for mIAB, in order to determine AS impacts, if any.

R2 considers that UEs can use the mIAB-cell indication, to prioritize (cell and/or freq) when the UE is camped on the mIAB cell, and FFS to prioritize when the UE is not yet camped on the mIAB cell. FFS if it can be specified the detailed condition for when to apply such prioritization (for either case), RAN2 considers condition based on cell dwelling timer or Mobility state.

R2 direction (solution agreements at later stage, no other directions will be considered):

- RAN2 acknowledges following two problems to be addressed for idle/inactive UEs:

- - Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low.

- - Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells.

- - Such UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.

P1a: RAN2 assumes that there is no need to introduce logical-DU-specific default BAP configuration in mobile IAB from RAN2 perspective, unless requested by RAN3 otherwise (no LS for now).

P1b: RAN2 understands that the F1AP (re)configured BAP configuration to one DU will not impact/override the usage of default BAP configuration by another DU.

P2: RAN2 assumes there may be redundant BAP configuration entries for non-F1-U traffic and it is up to IAB node's implementation to decide which entry is selected. FFS if there is any specification impact.