# 3GPP TSG RAN2#123  	                                                 R2-2308837

# Toulouse, France, Aug 21st – 25th, 2023

Agenda item:	7.16.2.2

Title:	Further Discussion on Purpose Driven Data Collection

Source:	ZTE Corporation, Sanechips

Document for:	Discussion and Decision

Introduction

- In the RAN2#122 meeting, the following agreements regarding the data collection are achieved:

- P5a: For the data generation entity and termination entity deployed at different entities, RAN2 assumes: For CSI enhancement and beam management use cases: - - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server. - - For NW-sided model inference, input data can be generated by UE and terminated at gNB. - - For UE-side model inference, input data/assistance information can be generated by gNB and terminated at UE. - - For model monitoring at NW side, performance metrics can be generated by UE and terminated at gNB. - For positioning enhancement use case: - - For model training, training data can be generated by UE/gNB and terminated at LMF/OTT server. - - For NW-sided model inference, input data can be generated by UE/gNB and terminated at LMF and/or gNB. - - For UE-side model inference, input data/assistance information can be generated by LMF/gNB and terminated at the UE. - - For model monitoring at NW side, performance metrics can be generated by UE/gNB and terminated at LMF.

At the end of the RAN2#122 meeting, all above agreements are sent to RAN1 for the evaluation of data collection for each LCM purpose and use case. In our understanding, before the LS back from RAN1, RAN2 can select some suitable candidates framework for each LCM purpose of each use case in advance. Then RAN2 can start the continuous work on the selected frameworks for possible enhancements on the basis of the evaluation result received from RAN1.

Discussion

General

In the post-email discussion[1], the functionality mapping to the logical entity is discussed and some proposals are concluded. In the proposals, some non-RAN-logical-entity has been getting involved such as OTT server, OAM. Intuitively, it must have the impact on the data collection analysis, for example, the data collection terminated between OTT server and NW, and the data collection terminated between UE and OAM, and so on…. However, in most case of candidate framework, the OAM and OTT server will not be involved, so before the detail discussion, we would like to stress that:

The data collection part between OTT server and UE; gNB and OAM is transparent to the RAN specification. In other word, if the data collection terminate point is OTT server, then the communication between UE and OTT server is ignored for the study, else if the data collection terminate point is OAM, then the communication between gNB and OAM is ignored for the study.

AI based CSI feedback enhancement

Regarding the model training, in [1], the mapped logical entity is gNB, OAM, OTT server, UE, for discussion easily, the mapped logical entity with an FFS is excluded in this paper.

So the potential terminated point of data collection is shown as below:

1: data collection terminated UE(OTT server) and gNB (OAM)

2[FFS]: data collection between UE(OTT server) and CN(FFS)

The candidate framework between UE and gNB is shown as below:

- Immediate MDT

- L3 measurement/report

- L1 measurement/report

- UAI

- Early Measurements

All the immediate MDT and L3 measurement, early measurement are RRC signaling based framework, and L1 measurement is PUCCH based framework. We can select one RRC signaling based framework and one PUCCH based framework as candidate framework for model training:

For AI based CSI feedback enhancement, RAN2 assumes L1 measurement framework and/or Immediate MDT can be applied to the data collection of model training for AI based CSI feedback.

Regarding the model inference, in[1], the mapped logical entity is gNB and/or UE,.

The potential terminated point of data collection for model inference is gNB and UE. As model training, the L1 measurement and Immediate MDT can be taken into account, Considering the model inference for CSI reporting is delay sensitive, the MDT is no longer suitable.

For AI based CSI feedback enhancement, RAN2 assumes L1 measurement framework can be applied to data collection of model inference

Regarding the model inference, in[1], the mapped logical entity is gNB and/or UE.

The potential terminated point of data collection for model inference is gNB and UE, as model inference, the model monitoring for CSI feedback is a delay sensitive function, the data collection framework shall be the same with model inference:

For AI based CSI feedback enhancement, RAN2 assumes L1 measurement framework can be applied to data collection of model monitoring.

AI based beam management

Regarding the model training for AI based BM, in [1], the mapped logical entity is OTT server, UE, gNB, OAM. Just note that, for discussion easily, the mapped logical entity with an FFS is excluded in this paper.

So the potential terminated point of data collection is data collection terminated UE(OTT server) and gNB (OAM)

The candidate framework between UE and gNB is shown as below:

- Immediate MDT

- L3 measurement/report

- L1 measurement/report

- UAI

- Early Measurements

As the same as AI based CSI feedback, we can select one RRC signaling based framework and one PUCCH based framework as candidate framework for model training:

For AI based BM, RAN2 assumes L1 measurement framework and/or Immediate MDT can be applied to the data collection of model training for AI based beam management.

Regarding the model inference, in[1], the mapped logical entity is gNB and/or UE,.

And the model inference of the AI based BM is also delay sensitive, so

For AI based BM, RAN2 assumes L1 measurement framework can be applied to data collection of model inference

Regarding the model monitoring, in[1], the mapped logical entity is gNB and/or UE.

And the model monitoring of the AI based BM is also delay sensitive,

For AI based BM, RAN2 assumes L1 measurement framework can be applied to data collection of model monitoring.

AI based Positioning

For all cases for AI based positioning, no matter for which case, the signalling exchange is only via LPP signalling between UE and LMF.

For AI based positioning without gNB sided model, RAN2 assumes LPP protocol should be applied for the data collection terminated at UE and LMF.

- In [1], for AI based positioning with gNB sided mode (case 3a), the data collection terminated between LMF and gNB is detected, the LPP signalling does not support the direct communication between LMF and gNB, so we propose:

Add the NRPPa protocol into candidates framework for data collection, RAN2 assumes NRPPa protocol should be applied for the data collection terminated at gNB and LMF

Conclusion

In this contribution, we provide our further views on the identified issues for general aspects of common AI/ML framework. We have the following observations and proposals:

Observation 1:	The data collection part between OTT server and UE; gNB and OAM is transparent to the RAN specification. In other word, if the data collection terminate point is OTT server, then the communication between UE and OTT server is ignored for the study, else if the data collection terminate point is OAM, then the communication between gNB and OAM is ignored for the study.

Proposal 1:	For AI based CSI feedback enhancement, RAN2 assumes L1 measurement framework and/or Immediate MDT can be applied to the data collection of model training for AI based CSI feedback.

Proposal 2:	For AI based CSI feedback enhancement, RAN2 assumes L1 measurement framework can be applied to data collection of model inference

Proposal 3:	For AI based CSI feedback enhancement, RAN2 assumes L1 measurement framework can be applied to data collection of model monitoring.

Proposal 4:	For AI based BM, RAN2 assumes L1 measurement framework and/or Immediate MDT can be applied to the data collection of model training for AI based beam management.

Proposal 5:	For AI based BM, RAN2 assumes L1 measurement framework can be applied to data collection of model inference

Proposal 6:	For AI based BM, RAN2 assumes L1 measurement framework can be applied to data collection of model monitoring.

Proposal 7:	For AI based positioning without gNB sided model, RAN2 assumes LPP protocol should be applied for the data collection terminated at UE and LMF.

In [1], for AI based positioning with gNB sided mode (case 3a), the data collection terminated between LMF and gNB is detected, the LPP signalling does not support the direct communication between LMF and gNB, so we propose:

Proposal 8:	Add the NRPPa protocol into candidates framework for data collection, RAN2 assumes NRPPa protocol should be applied for the data collection terminated at gNB and LMF

References

[1]: Report of [Post122][060][AIML] Mapping of functions to physical entities (CMCC)