# 3GPP TSG RAN2#123 Meeting  	                                          R2-2308841

# Toulouse, France, Aug 21st – 25th, 2023

Agenda item:	7.4.2.3

Title:	Further Discussion on RACH-Less LTM Execution

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In the RAN2#122 meeting, we have the following agreements regarding the RACH-less LTM execution:

R2 assumes RRCReconfigurationComplete message is always sent at each LTM execution.

In RACH-less LTM, RRCReconfigurationComplete can be the content of the first UL MAC PDU/transmission to indicate UE arrival, i.e. no need to introduce any new signaling to indicate UE arrival (for the MCG-switch case)

For RACH-based LTM, the UE considers that LTM execution procedure is successfully completed when the RACH is successfully completed.

For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data.

Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell:

the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch.

- upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant.

Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization

If the TA maintenance etc for candidate cell(s) in the UE is needed, the TA(s) associated with candidate cell(s) can be maintained during LTM (TDB exactly which cells decide stage-3).

In above agreements, it generally described the guidelines of UE behavior regarding LTM execution for below two parts:

- Part I: The very first transmission for RACH-less LTM execution.

- Part II: TA maintenance during LTM execution.

However, for Part 1, it has not been determined yet how UE process the CG and/or DG to perform the very first UL transmission with DCCH message (i.e.RRCReconfigurationComplete) for LTM execution, and whether the potential enhancements are needed. For Part 2, it needs to be discussed whether and how UE to maintain the known TA for the candidate cell. In this contribution, we target to share our views on above issues.

Discussion

The first UL transmission

The first UL transmission by using the CG

In RAN2#122 meeting, it is determined that:

Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization

For implementing the agreement, one type 1 configured grant configuration should be configured for each candidate cell and provide a number of CG resources for UE to perform the first UL transmission. In LTE RACH-less handover, the configured grant configuration for LTM shall be released if the HO is failed or successful, in our understanding, we can have a same mechanism in NR RACH-less LTM.

A type 1 configured grant configuration for LTM is optionally configured to UE within the container in each LTM-Candidate; the configured grant configuration for LTM should be released if the CG-LTM is considered as complete.

In the agreement, it also mentioned that the CG occasion is selected by UE according to the beam indicated in the LTM MAC CE, for supporting this, the CG occasion shall be associated with the SSB which is similar as CG-SDT.

Similar as CG-SDT, some kind of mapping between SSB and CG resource shall be provided to UE, and UE will determine the CG resource for the initial UL transmission based on the SSB selected during LTM procedure.

Given that the target DU can derive the beam information according to the RO where the very first UL transmission is received, the CG based LTM execution so called as CG-LTM can be applied to both intra-DU LTM and inter-DU LTM case.

CG-LTM can be applied to both intra-DU LTM and inter-DU LTM cases.

According to the observation 1, the association between SSB and CG occasion shall be configured, and UE need to select the CG occasion on the basis of the TCI-state indicated by LTM Cell Switch MAC CE for the first UL transmission .

Mapping between SSB and CG occasion should be configured in RRC signalling, and the detail should be discussed and determined in RAN1.

UE select the SSB which has a QCL relationship with the TCI-state indicated by LTM Cell Switch MAC CE, and consider the corresponding configured grant as valid for the first UL transmission with DCCH message.

During the time period between the first UL transmission and the LTM completion, there maybe several Uplink grants. There maybe UM data loss risk if UE use those UL grants and the LTM is not considered as successful, and hence it is suggested those configured UL grants between the first UL transmission and LTM completion shall be ignored.

All UL grants preallocated by CG-LTM shall be ignored if the first transmission with DCCH message have been performed and CG-LTM has not been determined as complete.

In some case, the beam indicated by LTM Cell switch MAC CE is determined by source cell on the basis of the L1 measurement result from UE which may be not robust enough, sometime, the beam is not ideal, for example, the beam have a high RSRP but with lower SINR. If UE perform the first UL transmission with such beam, in the inter-DU LTM, the target cell maybe not aware of UE arrival, and UE will be stuck in the target cell until the supervisor timer is expired.

In the inter-DU LTM case, the DU may be not aware of UE arrival due to the loss of the very first UL transmission with DCCH message which may result in there is no any DG for the retransmission , at last , the RRC re-establishment procedure will be triggered as long as the supervisor timer is expired .

In this sense, we can consider to introduce a mechanism of automatic (re)transmission with configured grant, such as: the CG-SDT/NRU like retransmission timer based automatic re-transmission with configured grant, or the NRIIOT like automatic transmission with configured UL grant.

Introduce the automatic (re)transmission of the first UL transmission with configured grant for CG-LTM, the detail solution (e.g. CG-SDT like mechanism, or NRIIOT like mechanism) is FFS.

the first UL transmission by using the DG

According to the below agreements,

the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch.

    - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant.

The yellow highlighted assumption will delay the LTM procedure with around 10~15 ms due to the inter-DU communication, but the average delay of CFRA only have 5~10 ms. We do not see any necessity to have such assumption. In our understanding, if there are SRS resources and/or PUCCH for SR are configured in the target DU and UE have an available UL TA information, UE should send the SRS when switching into the target cell or trigger the SR due to the lack of UL grant resources, the target cell can obtain the beam information from SRS/PUCCH for SR with beam reciprocity. But all these contents shall be discussed in RAN1 instead of RAN2.

Upon cell switch decision, R2 assumes that the source DU can obtain the beam information via the SRS/ PUCCH resource of SR with beam reciprocity, ask RAN1 about the availability.

If source DU cannot obtain the beam information via SRS/ PUCCH resources of SR, the DG-LTM is only limited to the case of intra-DU LTM which is handled by NW implementation.

Part II: TA maintenance during LTM

For TA maintenance, the following agreement is achieved:

If the TA maintenance etc for candidate cell(s) in the UE is needed, the TA(s) associated with candidate cell(s) can be maintained during LTM (TDB exactly which cells decide stage-3).

There are three cases about the candidate cells whose value can be obtained and maintained in UE :

- Case 1: The candidate cells whose TA has been obtained via LTM Cell Switch MAC CE of the historical LTM.

- Case 2: The candidate cells are the current serving cells of the UE whose TA values have been obtained.

- Case 3: The candidate cells whose TA is obtained from UE based TA measurement.

In the first two cases, the NW not only have a knowledge about the TA obtained as both case 1 and case 2 but also the TA obtained from early RACH, then it seems there is no need to maintain the TA value which is obtained like case 1 and case 2 at UE side, the NW can forward the TA information of the candidate cell to UE via LTM Cell Switch MAC CE. In the third case, NW have no idea about how many and what candidate cells the corresponding TA has been obtained by UE, then the TA maintenance of the candidate cell in the UE is needed.

Only TA maintenance at UE side is needed for the case of UE based TA acquisition, if the UE based TA acquisition is supported.

the detail TA handling method regarding the UE based TA acquisition is postponed until the evaluation is finished in RAN4.

In addition, in the LTE RACH less handover, The TA of both STAG and PTAG are provided in the RRC signaling, we understand there is some benefits if the TA of STAG is provided to UE,

In the LTM, it is agree that one or more target SCells can be indicated as activated after cell switch in order to resume the UP data transmission as soon as possible. However, the TA value of the STAG the SCell belongs to should be considered as invalid and UL is considered as non-sync. As a result, the UL transmission should be delayed until to the valid TA is indicated by NW.

In the current LTM Cell Switch MAC CE, only TA value of PTAG is present in the MAC CE.

In the inter-DU LTM case, source DU have no idea about the target SCells information

In case of intra-DU LTM, the TA value of the STAG can be optionally present in the LTM Cell Switch MAC CE.

Conclusion

In this contribution, we provide our further views on the identified issues for general aspects of common AI/ML framework. We have the following observations and proposals:

Observation 1:	Similar as CG-SDT, some kind of mapping between SSB and CG resource shall be provided to UE, and UE will determine the CG resource for the initial UL transmission based on the SSB selected during LTM procedure.

Observation 2:	In the inter-DU LTM case, the DU may be not aware of UE arrival due to the loss of the very first UL transmission with DCCH message which may result in there is no any DG for the retransmission , at last , the RRC re-establishment procedure will be triggered as long as the supervisor timer is expired .

Observation 3:	In the LTM, it is agree that one or more target SCells can be indicated as activated after cell switch in order to resume the UP data transmission as soon as possible. However, the TA value of the STAG the SCell belongs to should be considered as invalid and UL is considered as non-sync. As a result, the UL transmission should be delayed until to the valid TA is indicated by NW.

Observation 4:	In the current LTM Cell Switch MAC CE, only TA value of PTAG is present in the MAC CE.

Observation 5:	In the inter-DU LTM case, source DU have no idea about the target SCells information

Proposal 1:	A type 1 configured grant configuration for LTM is optionally configured to UE within the container in each LTM-Candidate; the configured grant configuration for LTM should be released if the CG-LTM is considered as complete.

Proposal 2:	CG-LTM can be applied to both intra-DU LTM and inter-DU LTM cases.

Proposal 3:	Mapping between SSB and CG occasion should be configured in RRC signalling, and the detail should be discussed and determined in RAN1.

Proposal 4:	UE select the SSB which has a QCL relationship with the TCI-state indicated by LTM Cell Switch MAC CE, and consider the corresponding configured grant as valid for the first UL transmission with DCCH message.

Proposal 5:	All UL grants preallocated by CG-LTM shall be ignored if the first transmission with DCCH message have been performed and CG-LTM has not been determined as complete.

Proposal 6:	Introduce the automatic (re)transmission of the first UL transmission with configured grant for CG-LTM, the detail solution (e.g. CG-SDT like mechanism, or NRIIOT like mechanism) is FFS.

Proposal 7:	Upon cell switch decision, R2 assumes that the source DU can obtain the beam information via the SRS/ PUCCH resource of SR with beam reciprocity, ask RAN1 about the availability.

Proposal 8:	If source DU cannot obtain the beam information via SRS/ PUCCH resources of SR, the DG-LTM is only limited to the case of intra-DU LTM which is handled by NW implementation.

Proposal 9:	Only TA maintenance at UE side is needed for the case of UE based TA acquisition, if the UE based TA acquisition is supported.

Proposal 10:	the detail TA handling method regarding the UE based TA acquisition is postponed until the evaluation is finished in RAN4.

Proposal 11:	In case of intra-DU LTM, the TA value of the STAG can be optionally present in the LTM Cell Switch MAC CE.

References

[1]: Report of [Post122][060][AIML] Mapping of functions to physical entities (CMCC)