# 3GPP TSG RAN2#123 Meeting 	                                          R2-2308844

# Toulouse, France, Aug 21st – Aug 25th, 2023

Agenda item:	7.20.3

Title:	Further consideration on unified TCI State Extension for SDMT

Source:	ZTE Corporation, Sanechips

Document for:	Discussion and Decision

Introduction

In RAN2#122 meeting, the unified TCI state indication MAC CE for SDMT (i.e. single DCI mTRP) is discussed and the following agreement is achived:

For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE, with the following high level design principles:

    - If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRP (i.e., as configured by RRC for both TRPs).

- The following information can be indicated by the MAC CE (for joint DL/UL TCI mode):

    - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs,

    - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s)

FFS for the separate DL/UL TCI mode.

And there is an FFS is left, in this contribution, we would like to share our views on this FFS

Discussion

Compare to the joint TCI mode, the unified TCI state MAC CE for separate mode shall have the indication to indicate how many and/or which type of TCI state is present, in our understanding, such indication shall share the field with the indication for joint mode which indicates the unified TCI state is for one of the TRPs or for both TRPs.

The following information ca be indicated by the MAC CE (for separate DL/UL TCI mode):

  - If the unified TCI state is for the first/second DL/UL TRP.

With above proposal and the agreed contents in the previous meeting, the unified TCI state MAC CE for SDMT is shown as below:

Fig.1 One illustration of the unified TCI state for SDMT

The Ti field is to indicate the which TRP the TCI state is present for, for example,

For joint mode:

Ti=0000, It means no TCI state field is present for i-th TCI code point, Ti=0001, it means the TCI state field is present for the first TRP only for the i-th TCI code point, Ti=0010, It means the TCI state field is present for the second TRP only for the i-th TCI code point. Ti=0011, It means that the TCI state fields are present for both TRPs.

For separate mode:

Ti=0000, It means no TCI state field is present for i-th TCI code point; Ti=0001, it means the TCI state field is present for the first DL TRP only for the i-th TCI code point; Ti=0010, it means the TCI state field is present for the first UL TRP only for the i-th TCI code point; Ti=0100, it means the TCI state field is present for the second DL TRP only for the i-th TCI code point; Ti=1000, it means the TCI state field is present for the seond UL TRP only for the i-th TCI code point;

Take the illustration above as baseline for unified TCI state MAC CE for SDMT

Conclusion

In this contribution, we provide our further views on the identified issues for general aspects of common AI/ML framework. We have the following observations and proposals:

未找到目录项。

Proposal 1:	The following information ca be indicated by the MAC CE (for separate DL/UL TCI mode):

a)	If the unified TCI state is for the first/second DL/UL.

Proposal 2:	Take the illustration above as baseline for unified TCI state MAC CE for SDMT

References

[1]: Report from NR MIMO evolution session