3GPP TSG-RAN WG2 Meeting #123						  R2-2308849

Toulouse, France, August 21 – August 25, 2023

Agenda item:	6.1.3.2

Source:			MediaTek Inc.

Title:	Further considerations on new signaling of maximum aggregated bandwidth

Document for:	Discussion and Decision

- Background

During online discussion in RAN2#122 meeting, the working assumption is concluded:

Working Assumption that we go with P1-P6 in R2-2304856, under the condition that BW compatibility is supported. Next meeting describe how BW compatibility can work and confirmation of WA (or the opposite if serious issues are found)

In this contribution, we aim to provide clarification on the behavior of the new signaling solution compared to that of the legacy solution. Additionally, we will discuss important parameters that were not included in RAN4 LS, such as MIMO layer capability. Furthermore, we will address the need for signaling design to accommodate various UE modem envelope requirements.

- Channel bandwidth and CA configuration tables

Channel bandwidth and NR CA configuration tables excerpt from TS 38.101-1 as the reference for the discussion in this paper.

Table 2-1: Channel bandwidths for NR bands n25, n41 and n66 (a part of TS 38.101-1 Table 5.3.5-1 Channel bandwidths for each NR band)

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n25 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 354 | 40 | 453,4 |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 354 | 40 | 453,4 |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 | 354 | 40 | 453,4 |  |  |  |  |  |  |
| n41 | 15 |  | 10 | 15 | 20 | 254 | 30 | 354 | 40 | 454 | 50 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 254 | 30 | 354 | 40 | 454 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  | 10 | 15 | 20 | 254 | 30 | 354 | 40 | 454 | 50 | 60 | 70 | 80 | 90 | 100 |
| n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 354 | 40 | 454 |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 354 | 40 | 454 |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 | 354 | 40 | 454 |  |  |  |  |  |  |

Table 2-2: NR CA configurations and BCSs defined for inter-band CA n25A-n41A-n66A (a part of TS 38.101-1 Table 5.5A.3.2-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands))

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |

- Discussion

  - Backward compatibility

This paper considers 3 different kinds of network for backward compatibility analysis:

- The legacy networks those that only support legacy BCS(s) (such as BCS0, BCS1 and so on) which were introduced before BCS4/5.

- The upgraded networks those that support both legacy BCS(s) and BCS4/5.

- The new networks those that support the new aggregated bandwidth signaling for BCS4/5.

Similarly, we will use the terms like the legacy UE, the upgraded UE and the new UE whenever there is a need to distinguish them in subsequent sections.

We use the example from the RAN4 LS to illustrate the backward compatibility issue. It is the UE with a maximum aggregated bandwidth limitation 140MHz for DL CA_n25A-n41A-n66A. The legacy, the upgraded and the new UE behavior is discussed in the following sections.

    - The legacy UE behavior

Considering a maximum aggregated bandwidth is 140MHz due to RF and/or baseband limitation for a legacy UE that supports BCS0 of DL CA_n25A-n41A-n66A. The legacy UE reports CA band combination capabilities as shown in the Table 3-1 to reflect these aforementioned limitations.

Table 3-1 The capabilities of DL CA_n25A-n41A-n66A from the legacy UE

| BC | BCS: 0 |  |  |
| --- | --- | --- | --- |
|  | n25 BWClassDL: A | n41 BWClassDL: A | n66 BWClassDL: A |
| FSPC-DL | CBW (MHz) | CBW (MHz) | CBW (MHz) |
|  | 20 | 100 | 20 |
|  | 10 | 90 | 40 |
|  | 20 | 80 | 40 |

For the legacy networks, the upgraded networks and the new networks, all channel bandwidth combinations in the Table 3-1 could be configured. None of them exceeds the maximum aggregated bandwidth limitation.

    - The upgraded UE behavior

Given that maximum aggregated bandwidth is 140MHz due to RF and/or baseband limitation for an upgraded UE that supports both BCS0 and BCS5 of DL CA_n25A-n41A-n66A. The upgraded UE reports CA band combination capabilities as shown in the Table 3-2 to reflect these aforementioned limitations. The channel bandwidths belonging to the BCS0 defined for CA_n25A-n41A-n66A are highlighted in green. For rest of all channel bandwidth combinations, they were supported in the BCS5.

Table 3-2 The capabilities of DL CA_n25A-n41A-n66A from the upgraded UE

| BC | BCS: 0, 5 |  |  |
| --- | --- | --- | --- |
|  | n25 BWClassDL: A | n41 BWClassDL: A | n66 BWClassDL: A |
| FSPC-DL | CBW (MHz) | CBW (MHz) | CBW (MHz) |
|  | 5 | 100 | 35 |
|  | 10 | 100 | 30 |
|  | 15 | 100 | 25 |
|  | 20 | 100 | 20 |
|  | 25 | 100 | 15 |
|  | 30 | 100 | 10 |
|  | 35 | 100 | 5 |
|  | 5 | 90 | 45 |
|  | 10 | 90 | 40 |
|  | 15 | 90 | 35 |
|  | 20 | 90 | 30 |
|  | 25 | 90 | 25 |
|  | 30 | 90 | 20 |
|  | 35 | 90 | 15 |
|  | 40 | 90 | 10 |
|  | 45 | 90 | 5 |
|  | 15 | 80 | 45 |
|  | 20 | 80 | 40 |
|  | 25 | 80 | 35 |
|  | 30 | 80 | 30 |
|  | 35 | 80 | 25 |
|  | 40 | 80 | 20 |
|  | 45 | 80 | 15 |
|  | 25 | 70 | 45 |
|  | 30 | 70 | 40 |
|  | 35 | 70 | 35 |
|  | 40 | 70 | 30 |
|  | 45 | 70 | 25 |
|  | 35 | 60 | 45 |
|  | 40 | 60 | 40 |
|  | 45 | 60 | 35 |
|  | 45 | 50 | 45 |

The legacy networks could still configure 3 channel bandwidth combinations (the ones high-lighted in green) of BCS0 for the UE. None of them exceeds the maximum aggregated bandwidth limitation. For the upgraded networks and the new networks, all channel bandwidth combinations in the Table 3-2 could be configured because they are considered as valid CA configurations supported either in the BCS0 or in the BCS5.

    - The new UE behavior

By introducing the new signaling for maximum aggregated bandwidth [4][5], there is no longer a need for channel bandwidth permutation in the featureSetCombinations and featureSets (The row number is decreased from 32 to 1). Instead, the new UE only needs to report a single row indicating the maximum channel bandwidth supported for every NR carrier component as shown in the Table 3-3. This reduces signaling overhead.

Table 3-3 The capabilities of DL CA_n25A-n41A-n66A from the new UE

| BC | BCS: 0, 5 | AggBW-FDD-DL: 40MHz | AggBW-TDD-DL: 100MHz |
| --- | --- | --- | --- |
|  | n25 BWClassDL: A | n41 BWClassDL: A | n66 BWClassDL: A |
| FSPC-DL | CBW (MHz) | CBW (MHz) | CBW (MHz) |
|  | 45 | 100 | 45 |

In the legacy networks and the upgraded networks, for example the channel bandwidth combination 20+100+40 (which is a fallback of 45+100+45) belongs to the BCS0 will be regarded as supported by the UE. However, its aggregated bandwidth is 160MHz which exceeds the 140MHz limitation. Such a CA configuration is unsupported for the new UE. When a UE receives a CA configuration with unsupported aggregated bandwidth it results in reconfiguration error or unspecified UE behavior.

Similarly, the channel bandwidth combination 40+100+40 will be considered supported by the legacy networks and the upgraded networks when the support of BCS1 is also indicated by the UE. In this case it will result in the same problem even if the aggregated bandwidth limitation is up to 160MHz.

Apparently, the problem of unsupported CA configuration would arise when the UE’s aggregated bandwidth limitation is lower than the maximum aggregated bandwidths of the legacy BCS(s).

Observation 1: There is backward compatibility issue while reporting new aggregative bandwidth capability to the network that does not support the new signaling.

    - Summary

We’re summarize the back compatibility analysis for 3 different UE/networks in the Table 3-4.

Table 3-4 Interoperability evaluation of the UEs and the networks

|  | The legacy networks (supports BCS 0 only) | The upgraded networks (supports BCS0, 5) | The new networks (supports BCS0, 5 and new signaling) |
| --- | --- | --- | --- |
| The legacy UE reporting | No interoperability problem foreseen | No interoperability problem foreseen | No interoperability problem foreseen |
| The upgraded UE reporting | No interoperability problem foreseen | No interoperability problem foreseen | No interoperability problem foreseen |
| The new UE reporting | Unsupported CA configuration (CA bandwidth configuration from the network overrun the aggregated bandwidth limitation of the UE.) | Unsupported CA configuration (CA bandwidth configuration from the network overrun the aggregated bandwidth limitation of the UE.) | No interoperability problem foreseen |

To avoid NBC issues, we think there are two solution options:

Option 1 (UE-based): The UE uses the new signaling for aggregated bandwidth capability reporting only when the maximum aggregated bandwidth limitation is equal to or higher than that defined in the legacy BCS(s).

Option 2 (Network-based): The network asks for the new aggregated bandwidth capability by a new capability enquiry filter.

We figure that the Option 1 doesn’t serve the purpose of introducing new aggregated bandwidth signaling well. In addition, Option 2 is normal practice to handle this kind of backward compatible issue. Therefore, we recommend adding a capability enquiry filter to request the UE capability reporting in the new signaling for the new networks.

-- ASN1START

-- TAG-UE-CAPABILITYREQUESTFILTERNR-START

UE-CapabilityRequestFilterNR ::=            SEQUENCE {

frequencyBandListFilter   FreqBandList                       OPTIONAL,   -- Need N

nonCriticalExtension      UE-CapabilityRequestFilterNR-v1540 OPTIONAL

}

UE-CapabilityRequestFilterNR-v1540 ::=      SEQUENCE {

srs-SwitchingTimeRequest  ENUMERATED {true}                  OPTIONAL,  -- Need N

nonCriticalExtension      UE-CapabilityRequestFilterNR-v1710 OPTIONAL

}

UE-CapabilityRequestFilterNR-v1710 ::=      SEQUENCE {

sidelinkRequest-r17       ENUMERATED {true}                  OPTIONAL,  -- Need N

nonCriticalExtension      UE-CapabilityRequestFilterNR-v17xy OPTIONAL

}

UE-CapabilityRequestFilterNR-v17xy ::=      SEQUENCE {

supportedAggBW-r17        ENUMERATED {true}                  OPTIONAL,  -- Need N

nonCriticalExtension      SEQUENCE {}                        OPTIONAL

}

-- TAG-UE-CAPABILITYREQUESTFILTERNR-STOP

-- ASN1STOP

Proposal 1: Introduce a capability enquiry filter to request the reporting of the new signaling to prevent interoperability issues.

  - Signaling design

We are continuing our examination of alternative UE behaviors that may arise due to different capability limitations in various dimensions and/or modem envelope requirements imposed by the ecosystem.

The UE capability envelope of CA band combination is formulated by following dimensions (parameters):

- The frequency band (per BC / xDD,FRx).

- The carrier component, associated with the bandwidth class (per Band per BC).

- The channel bandwidth (per FSPC).

- The MIMO layer (per FSPC).

The resources of each dimension can be utilized in conjunction with each other without changing the overall modem capability. For instance, the UE can exchange some MIMO layers for larger channel bandwidths in certain carrier components of CA frequency bands. This is done to meet specific compliance requirements set by the network operators while keeping the same peak data rate (throughput).

    - FDD+TDD aggregated bandwidth

As per Table 3-2, the upgraded UE can indicate the following supported aggregated bandwidth capability information for DL CA_n25A-n41A-n66A through existing CA capability signaling (shown in the Table 3-5):

Table 3-5 The supported xDD aggregated BW combinations of DL CA_n25A-n41A-n66A in CA capability signaling from the upgraded UE

| Total aggregated bandwidth = 140MHz |  |  |
| --- | --- | --- |
| Pair# | Aggregated BW of FDD bands (MHz) | Aggregated BW of TDD bands (MHz) |
| 1 | 40 | 100 |
| 2 | 50 | 90 |
| 3 | 60 | 80 |
| 4 | 70 | 70 |
| 5 | 80 | 60 |
| 6 | 90 | 50 |

The new signaling should have the capability to deliver the same capability information as before. However, we think that current proposal [4][5] of new signaling cannot achieve this without changes in the ASN.1 definition.

If we follow the original proposal by RAN4, a quick solution is to have a list structure for the new aggregated bandwidth capability parameter then a UE could report a list of supported aggregated bandwidth combinations as shown in the Table 3-6.

Table 3-6 The list of xDD aggregated bandwidth capabilities of DL CA_n25A-n41A-n66A

| BC | BCS: 0, 5 | AggBW-FDD-DL: 40MHz | AggBW-TDD-DL: 100MHz |
| --- | --- | --- | --- |
|  |  | AggBW-FDD-DL: 50MHz | AggBW-TDD-DL: 90MHz |
|  |  | AggBW-FDD-DL: 60MHz | AggBW-TDD-DL: 80MHz |
|  |  | AggBW-FDD-DL: 70MHz | AggBW-TDD-DL: 70MHz |
|  |  | AggBW-FDD-DL: 80MHz | AggBW-TDD-DL: 60MHz |
|  |  | AggBW-FDD-DL: 90MHz | AggBW-TDD-DL: 50MHz |
|  | n25 BWClassDL: A | n41 BWClassDL: A | n66 BWClassDL: A |
| FSPC-DL | CBW | CBW | CBW |
|  | 45 | 100 | 45 |

A corresponding ASN.1 definition would be like:

CA-ParametersNR-v17xy ::= SEQUENCE {

supportedAggBW-InterBandCA-ListFR1-r17 ::= SEQUENCE (SIZE (1..FFS)) OF SupportedAggBW-InterBandCA-FR1-r17      OPTIONAL

}

SupportedAggBW-InterBandCA-FR1-r17 ::=        SEQUENCE {

supportedAggBW-FDD-DL-r17			SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-FDD-UL-r17			SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-TDD-DL-r17			SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-TDD-UL-r17			SupportedAggBandwidth-r17      OPTIONAL

}

-- ASN1START

-- TAG-SUPPORTEDAGGBANDWIDTH-START

SupportedAggBandwidth-r17 ::=      ENUMERATED {mhz200, mhz300, mhz400, mhz500, mhz600, mhz700, mhz800, mhz900, mhz1000, mhz1100, mhz1200, mhz1300, mhz1400, mhz1500, mhz1600, mhz1700, mhz1800, mhz1900, mhz2000, mhz2100, mhz2200, mhz2300, mhz2400, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

-- TAG-SUPPORTEDAGGBANDWIDTH-STOP

-- ASN1STOP

We believe that the value FFS (the number of elements in the list) should be sufficiently large to accommodate various types of aggregated bandwidth requirements; However, this could result in additional signaling overhead and considerable testing effort.

Therefore, we propose to add new parameters that indicate the total aggregated bandwidth (FDD+TDD) for DL and UL respectively. Together with the separate FDD/TDD maximum aggregated bandwidth suggested by RAN4, the network can determine which pairs in Table 3-5 are supported by the UE.

For example, the UE may support only Pair 1 to 4 of Table 3-5 according to its modem envelop calculation (as in Table 3-7). The UE could report a total aggregated FDD+TDD bandwidth of 140MHz and two separate limitations on FDD – 70MHz and TDD 100MHz (as show in Table 3-8). Note this is under the assumption that FDD bandwidth may request more UE resource to support.

Table 3-7 The supported pairs according to UE implementation

| Total aggregated bandwidth = 140MHz |  |  | Supported by UE modem envelope |
| --- | --- | --- | --- |
| Pair# | Aggregated BW of FDD bands (MHz) | Aggregated BW of TDD bands (MHz) |  |
| 1 | 40 | 100 | Yes |
| 2 | 50 | 90 | Yes |
| 3 | 60 | 80 | Yes |
| 4 | 70 | 70 | Yes |
| 5 | 80 | 60 | No |
| 6 | 90 | 50 | No |

Table 3-8 The aggregated bandwidth capabilities signaling in a single element design

| BC | BCS: 0, 5 | AggBW-Total-DL: 140MHz |  |
| --- | --- | --- | --- |
|  |  | AggBW-FDD-DL: 70MHz | AggBW-TDD-DL: 100MHz |
|  | n25 BWClassDL: A | n41 BWClassDL: A | n66 BWClassDL: A |
| FSPC-DL | CBW | CBW | CBW |
|  | 45 | 100 | 45 |

The proposed ASN.1 definition is like:

CA-ParametersNR-v17xy ::= SEQUENCE {

supportedAggBW-TotalDL-r17				SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-TotalUL-r17				SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-InterBandCA-FR1-r17 ::=        SEQUENCE {

supportedAggBW-FDD-DL-r17			SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-FDD-UL-r17			SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-TDD-DL-r17			SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-TDD-UL-r17			SupportedAggBandwidth-r17      OPTIONAL

}

}

Proposal 2: Introduce a new parameter for total aggregated FDD+TDD bandwidth (together with the separate FDD/TDD maximum aggregated bandwidth suggested by RAN4).

    - Aggregated bandwidth and MIMO layer

From UE implementation point of view, it is possible that UE can support higher aggregated bandwidth if the MIMO layer is reduced. For example, the UE may want to report maximum aggregated bandwidth of 180MHz is case of 2 MIMO layers and 140MHz in case of 4 MIMO layers. With the new signaling, the UE may report these in two different BC as shown in the Table 3-9.

Table 3-9 Two BC reporting in new signaling for DL CA_n25A-n41A-n66A

| BC#1 | BCS: 0, 5 |  | AggBW-Total-DL: 180MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AggBW-FDD-DL: 90MHz |  | AggBW-TDD-DL: 100MHz |  |
|  | n25 BWClassDL: A |  | n41 BWClassDL: A |  | n66 BWClassDL: A |  |
| FSPC-DL | CBW | MIMO | CBW | MIMO | CBW | MIMO |
|  | 45 | 2 | 100 | 4 | 45 | 2 |
| BC#2 | BCS: 0, 5 |  | AggBW-Total-DL: 140MHz |  |  |  |
|  |  |  | AggBW-FDD-DL: 70MHz |  | AggBW-TDD-DL: 100MHz |  |
|  | n25 BWClassDL: A |  | n41 BWClassDL: A |  | n66 BWClassDL: A |  |
| FSPC-DL | CBW | MIMO | CBW | MIMO | CBW | MIMO |
|  | 45 | 4 | 100 | 4 | 45 | 4 |

We think that this kind of reporting should be allowed as the new signaling should still allow the UE to report the different capabilities combinations resulting from the resource exchange mentioned in the section 3.2.

Proposal 3: RAN2 confirms that the UE may report same band combination but with different maximum aggregated bandwidth and maximum MIMO layer in two different BC entry.

In this regard, the number of BC elements in the supported CA band combination list is increased by implementing a new per-BC capability signaling. The main question is whether reducing signaling overhead through this method is worthwhile. We are now concerned that it may go against principles outlined in the RAN2 UE capability guideline [6][7].

Therefore, we can consider advanced approach to have the aggregated bandwidth parameter elements reported in according with the position of FeatureSets in the FeatureSetsPerBand. By this way, the UE doesn’t need to report a separate band combination to explicitly advertise fallback band combination. The logical concept of signaling design is illustrated in the Table 3-10.

Table 3-10 The proposed design of new signaling for DL CA_n25A-n41A-n66A

| BC#1 | BCS: 0, 5 |  | AggBW-Total-DL: 180MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AggBW-FDD-DL: 90MHz |  | AggBW-TDD-DL: 100MHz |  |
|  |  |  | AggBW-Total-DL: 140MHz |  |  |  |
|  |  |  | AggBW-FDD-DL: 90MHz |  | AggBW-TDD-DL: 100MHz |  |
|  | n25 BWClassDL: A |  | n41 BWClassDL: A |  | n66 BWClassDL: A |  |
| FSPC-DL | CBW | MIMO | CBW | MIMO | CBW | MIMO |
|  | 45 | 2 | 100 | 4 | 45 | 2 |
| FSPC-DL | CBW | MIMO | CBW | MIMO | CBW | MIMO |
|  | 45 | 4 | 100 | 4 | 45 | 4 |

The proposed ASN.1 definition could be something like the TP in blue and in orange below:

CA-ParametersNR-v17xy ::= SEQUENCE {

supportedAggBW-FeatureSetsPerBandList-r17 ::= SEQUENCE (SIZE (1.. maxFeatureSetsPerBand)) OF SupportedAggBW-FeatureSetsPerBand-r17      OPTIONAL

}

SupportedAggBW-FeatureSetsPerBand-r17 ::= SEQUENCE {

supportedAggBW-TotalDL-r17				SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-TotalUL-r17				SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-InterBandCA-FR1-r17 ::=        SEQUENCE {

supportedAggBW-FDD-DL-r17			SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-FDD-UL-r17			SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-TDD-DL-r17			SupportedAggBandwidth-r17      OPTIONAL,

supportedAggBW-TDD-UL-r17			SupportedAggBandwidth-r17      OPTIONAL

}

}

(... skipped ...)

-- ASN1START

-- TAG-FEATURESETCOMBINATION-START

FeatureSetCombination ::=       SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand

FeatureSetsPerBand ::=          SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet

FeatureSet ::=                  CHOICE {

eutra                           SEQUENCE {

downlinkSetEUTRA                FeatureSetEUTRA-DownlinkId,

uplinkSetEUTRA                  FeatureSetEUTRA-UplinkId

},

nr                              SEQUENCE {

downlinkSetNR                   FeatureSetDownlinkId,

uplinkSetNR                     FeatureSetUplinkId

}

}

-- TAG-FEATURESETCOMBINATION-STOP

-- ASN1STOP

We suggest to have a list design with corresponding entries of supported aggregated bandwidth parameters. Each entry of SupportedAggBW-FeatureSetsPerBand-r17 is mapped to each combination of FeatureSets at the same position in the FeatureSetsPerBand, which is logically shown in the Table 3-10.

Proposal 4:  RAN2 to have a list design for the new signaling of aggregated bandwidth.

We have provided corresponding 38.331/38.306 CR in R2-2308851/R2-2308852 based on above proposals.

  - FR2 Applicability

In general, it is preferable for bandwidth capability signaling to be NR frequency range agnostic. We think that the RAN4 proposal [1] is technically feasible to be applied for FR2 inter-band CA. The new networks should be able to correctly derive complete capabilities even when the FR2 inter-band CA band combination comprises the intra-band CA or other types of lower order band combinations.

We also believe that all proposals in this paper are applicable for the FR2 case. However, the current ASN.1 definition of the bandwidth enumeration proposal [4] may require extension to accommodate the current maximum aggregated bandwidth defined for FR2 CA, which is 3000MHz. Since there is still uncertainty regarding the 3000MHz aggregated bandwidth defined in the TS 38.101-2. We believe that RAN2 requires the input from RAN4 on this matter.

-- ASN1START

-- TAG-SUPPORTEDAGGBANDWIDTH-START

SupportedAggBandwidth-r17 ::=      ENUMERATED {mhz200, mhz300, mhz400, mhz500, mhz600, mhz700, mhz800, mhz900, mhz1000, mhz1100, mhz1200, mhz1300, mhz1400, mhz1500, mhz1600, mhz1700, mhz1800, mhz1900, mhz2000, mhz2100, mhz2200, mhz2300, mhz2400, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

-- TAG-SUPPORTEDAGGBANDWIDTH-STOP

-- ASN1STOP

Considering the numerous unresolved matters that need clarification for FR1 CA cases, we believe it is premature to discuss covering FR2 CA now.

Proposal 5: Postpone the discussion of covering FR2 CA until the FR1 part is consolidated.

- Conclusion

In previous sections we made the following observations and/or proposals:

Observation 1: There is backward compatibility issue while reporting new aggregative bandwidth capability to the network that does not support the new signaling.

Proposal 1: Introduce a capability enquiry filter to request the reporting of the new signaling to prevent interoperability issues.

Proposal 2: Introduce a new parameter for total aggregated FDD+TDD bandwidth (together with the separate FDD/TDD maximum aggregated bandwidth suggested by RAN4).

Proposal 3: RAN2 confirms that the UE may report same band combination but with different maximum aggregated bandwidth and maximum MIMO layer in two different BC entry.

Proposal 4:  RAN2 to have a list design for the new signaling of aggregated bandwidth.

Proposal 5: Postpone the discussion of covering FR2 CA until the FR1 part is consolidated.

- References

- R2-2302439	LS on UE signalling for the maximum aggregated bandwidth for FR1 CA (R4-2303685; contact: Qualcomm)

- R2-2304444	Summary of [AT121bis-e][011][NR17] UE Caps BW related Corrections (Qualcomm)

- R2-2203899	Introduction of BCS4 and BCS5	TS 38.306 CR0669 rev3

- R2-2304857	Introduction of maximum aggregated bandwidth for FR1 inter-band CA    Qualcomm Incorporated    CR    Rel-17    38.331    17.4.0    4080    -    C    NR_BCS4-Core

- R2-2304858	Introduction of maximum aggregated bandwidth for FR1 inter-band CA    Qualcomm Incorporated    CR    Rel-17    38.306    17.4.0    0911    -    C    NR_BCS4-Core

- R2-2002378	Guidelines for UE capabilities

- R2-2306810	Further Guidelines on UE capability definitions