3GPP TSG-RAN WG2 Meeting #123	R2-2308853

# Toulouse, France, 21 - 25 Aug, 2023

Agenda Item:    7.11.2.2

Source:         Xiaomi

Title:         	Discussion and draft TP on the PDCP operation for the support of multicast reception in RRC_INACTIVE state

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to summarize the PDCP operation issue to support the multicast reception in RRC_INACTIVE state, and provide our considerations and the draft TP accordingly.

Discussion

  - PDCP operation during RRC state transition.

Currently, when UE receives RRCRelease with suspendconfig, the UE shall suspend the multicast MRB(s) and indicate the PDCP suspend of these multicast MRB(s), as specified in TS 38.331 quote below.

2>	suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0 and broadcast MRBs;

2>	indicate PDCP suspend to lower layers of all DRBs and multicast MRBs;

To support multicast reception in RRC_INACTIVE, when the UE transitions from RRC_CONNECTED to RRC_INACTIVE, if the multicast MRB in RRC_CONNECTED can continue to be used in RRC_INACTIVE, the UE should not suspend that MRB and should not indicate PDCP suspend.

And if the multicast MRB in RRC_CONNECTED cannot continue to be used in RRC_INACTVE, UE should suspend the MRB for the multicast reception in RRC_CONNECTED state and establish the new multicast MRB for the multicast reception in RRC_INACTIVE, which includes PDCP re-establishment.

Based on the analysis above, during the RRC state transition from RRC_CONNNECTED to RRC_INACTIVE, RRC should not indicated the PDCP suspend for the multicast MRB in RRC_CONNECTED which can continue to be used in RRC_INACTVE. And the PDCP entity only follows the RRC indication without extra impact.

Proposal 1: For the multicast MRB in RRC_CONNECTED which can continue to be used in RRC_INACTVE, UE should not suspend that MRB and not indicate the PDCP suspend.

  - PDCP operation during RRC_INACTIVE mobility

In Rel-17, PDCP count sync is supported for multicast reception during handover. For multicast reception in RRC_INACTIVE in Rel-18, PDCP count sync should be also supported during RRC_INACTIVE mobility. During the RAN2#121 post-meeting email discussion, we discussed about the support of PDCP count sync for multicast reception in RRC_INACTIVE and there still exist some issues to be resolved. [1]

# The UE awareness of the PDCP count sync state during INACTIVE mobility

To support the PDCP count sync for multicast reception in RRC_INACTIVE during INACTIVE mobility, the first issue is how UE gets aware of PDCP count is not synchronized between the source and target cells. For this issue, following options were proposed by companies in the previous meeting:

- Option 1: The network indicates UE whether the PDCP count is synchronized in the neighbouring cells, and the synchronisation in terms of MBS QoS flow to MRB mapping and PDCP SN size of the corresponding MRB among the cells are achieved by means of network implementation.

- Option 2: UE assumes that the PDCP count is not synchronized upon cell reselection.

For option 1, during the previous meeting, many companies raised the strong concerns on the feasible and network complexity. Different from the multicast reception in RRC_CONNECTED state, it is hard for network to know when the UE will reselect to which cell during the INACTIVE mobility. Besides, the explicitly network indication depends the network awareness of the sync state between the serving cell and the neighbour cells. At least for the inter-gNB scenario, it will introduce much more network complexity to exchange and maintain the PDCP count sync state among the inter-gNB cells for multicast reception in RRC_INACTIVE state.

In such case, compared to the network explicitly indication which is complex and not flexible, UE can simply assume the PDCP count is not sync between the source cell and the target cell during the INACTIVE mobility.

Observation 1: It introduces network complexity to indicate UE the PDCP count sync state between the serving cell and inter-gNB neighbour cells.

Proposal 2: Upon cell reselection, UE assumes that the PDCP count of the MRB configured for multicast reception in RRC_INACTIVE state is not synchronized in the target cell.

# PDCP count synchronisation operation

Upon cell reselection, to ensure the PDCP count sync in the target cell during the INACTIVE mobility, UE needs to re-initialize the PDCP count. However, once the PDCP count is re-initialized, the PDCP SDUs in the reception buffer which is not delivered to the upper layer will be discarded, which will cause the data loss. As such, to avoid such data loss, the PDCP SDUs in the reception buffer need to be delivered to the upper layer before the PDCP count re-initialisation. And to reduce the spec impact, UE can perform the PDCP count initialisation via PDCP entity re-establishment procedure which includes the delivery of all stored PDCP SDUs to the upper layers before PDCP count initialisation.

Observation 2: PDCP count re-initialisation may cause data loss as the PDCP SDUs in the reception buffer will be discarded.

Proposal 3: Upon cell reselection, UE re-initializes the PDCP count of the MRB configured for multicast reception in RRC_INACTIVE state via the PDCP entity re-establishment.

# The setting of initial PDCP count value

As for how to set the initial value for the MRB configured for the multicast reception in RRC_INACTIVE, following options were proposed during the previous meeting.

- Option 1:  UE acquires the initial PDCP count via MCCH or resuming RRC connection.

- Option 2: Same mechanism as the R17 broadcast MRB, i.e., the SN part is set to (x – 0.5 × 2[PDCP-SN-SizeDL–1]) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU, and the initial value of the HFN part of RX_DELIV is set by UE implementation.

For option 1, considering that congestion scenario is the main motivation for multicast reception in RRC_INACTIVE, the PDCP count re-initialization via initiating the RRC resume procedure may worsen the network congestion which is not expected. And for the MCCH provision approach, as the network may not know which cell UE is camping on or whether it is synced or not during the INACTIVE mobility, the initial PDCP count value should be always provided in MCCH, which introduces extra signalling overhead.

From our understanding, the motivation for re-initializing PDCP count is to prevent it from reaching its maximum value. If the maximum value is reached, data packets that need to be delivered to the RRC layer may be discarded, as the PDCP count cannot wrap around. As such, we prefer to re-initialize the PDCP count of the MRB for multicast reception in the RRC_INACTIVE state based on the same mechanism as R17 MBS broadcast, which relies on UE implementation without network involvement, i.e., the SN part is set to (x – 0.5 × 2[PDCP-SN-SizeDL–1]) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU, and the initial value of the HFN part of RX_DELIV is set by UE implementation.

Observation 3: It introduces extra signalling overhead and complexity to re-initialize the PDCP count via the RRC configuration.

Proposal 4: Upon cell reselection, UE sets the initial PDCP count of the MRB for the multicast reception in RRC_INACTIVE state based on the same mechanism as R17 MBS broadcast.

Proposal 5: RAN2 considers the draft TP in the Annex as baseline.

Conclusions

According to the analysis given above, we have the following observations and proposals:

PDCP operation during RRC state transition from RRC_CONNECTED to RRC_INACTIVE.

Proposal 1: For the multicast MRB in RRC_CONNECTED which can continue to be used in RRC_INACTVE, UE should not suspend that MRB and not indicate the PDCP suspend.

PDCP operation during RRC_INACTIVE mobility.

# The UE awareness of the PDCP count sync state during INACTIVE mobility

Observation 1: It introduces network complexity to indicate UE the PDCP count sync state between the serving cell and inter-gNB neighbour cells.

Proposal 2: Upon cell reselection, UE assumes that the PDCP count of the MRB configured for the multicast reception in RRC_INACTIVE state is not synchronized in the target cell.

# PDCP count sync operation

Observation 2: PDCP count re-initialisation may cause data loss as the PDCP SDUs in the reception buffer will be discarded.

Proposal 3: Upon cell reselection, UE re-initializes the PDCP count of the MRB configured for the multicast reception in RRC_INACTIVE state via the PDCP entity re-establishment.

# The setting of initial PDCP count value

Observation 3: It introduces extra signalling overhead and complexity to re-initialize the PDCP count via the RRC configuration.

Proposal 4: Upon cell reselection, UE sets the initial PDCP count of the MRB for the multicast reception in RRC_INACTIVE state based on the same mechanism as R17 MBS broadcast.

Proposal 5: RAN2 considers the draft TP in the Annex as baseline.

References

- R2-23xxxxx Report of [Post121][607][eMBS] UP issues for Multicast in RRC Inactive (Apple)

Annex: Draft TP 38.323

### 5.1.2	PDCP entity re-establishment

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this clause for Uu or PC5 interface. After performing the procedures in this clause, the UE shall follow the procedures in clause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the ROHC protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the EHC protocol for uplink if drb-ContinueEHC-UL is not configured in TS 38.331 [3];

-	for AM DRBs, reset the UDC compression buffer to all zeros and prefill the dictionary if drb-ContinueUDC is not configured in TS 38.331 [3];

-	for SRBs and UM DRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers, and;

-	for AM DRBs for Uu interface whose PDCP entities were suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, for each PDCP SDU already associated with a PDCP SN:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer, as specified in clause 5.2.1;

-	for AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	If drb-ContinueUDC is configured and if the PDCP SDU has been compressed before:

-	submit the PDCP SDU previously compressed to integrity protection and ciphering function;

-	else:

-	perform uplink data compression of the PDCP SDU as specified in clause 5.14.4, and submit the PDCP SDU to integrity protection and ciphering function;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1.

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the clause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	for SRBs, UM DRBs and UM MRBs, if t-Reordering is running:

-	stop and reset t-Reordering;

-	for UM DRBs and UM MRBs, deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	for AM DRBs and AM MRBs for Uu interface, perform header decompression using ROHC for all stored PDCP SDUs if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for AM DRBs for PC5 interface, perform header decompression using ROHC for all stored PDCP IP SDUs;

-	for AM DRBs and AM MRBs for Uu interface, perform header decompression using EHC for all stored PDCP SDUs if drb-ContinueEHC-DL is not configured in TS 38.331 [3];

-	for UM DRBs, AM DRBs, UM MRBs and AM MRBs, reset the ROHC protocol for downlink and start with NC state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs, AM DRBs, UM MRBs and AM MRBs, reset the EHC protocol for downlink if drb-ContinueEHC-DL is not configured in TS 38.331 [3];

-	for SRBs and UM DRBs, set RX_NEXT and RX_DELIV to the initial value;

-	for UM MRBs and AM MRBs, set RX_NEXT and RX_DELIV to the initial value if initialRX-DELIV is configured in TS 38.331 [3];

-	for MRBs configured for the multicast reception in RRC_INACTIVE state, set RX_NEXT and RX_DELIV to the initial value.

Editor’s Note: The RLC mode of MRB configured for the multicast reception in RRC_INACTIVE state can be further discussed. If the RLC mode is AM, the PDCP status report needs to be disabled as no UL resources are available.

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure.

NOTE 1:	After PDCP re-establishment on a sidelink ‎SRB/DRB, UE determines when to transmit and receive with the new key and discard the old key as specified in TS ‎‎33.536 [14].‎

NOTE 2:	At PDCP re-establishment, the MRB type (i.e. UM MRB or AM MRB) is determined by the target configuration.‎

## 7.1	State variables

This clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this clause are normative.

All state variables are non-negative integers, and take values from 0 to [232 – 1].

PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-SizeUL] – 1] or 0 to [2[pdcp-SN-SizeDL] – 1] or 0 to [2[sl-PDCP-SN-Size] – 1].

The transmitting PDCP entity shall maintain the following state variables:

a)	TX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0, except for SRBs configured with state variables continuation. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

The receiving PDCP entity shall maintain the following state variables:

a)	RX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, and for broadcast MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For broadcast MRBs and multicast MRBs configured for the multicast reception in RRC_INACTIVE state upon cell reselection as specified in TS 38.304, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

NOTE 1:	For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, it is up to UE implementation to select the HFN part for RX_NEXT such that initial value of RX_DELIV should be a positive value.

NOTE 2:	For broadcast MRBs and multicast MRBs configured for the multicast reception in RRC_INACTIVE state upon cell reselection as specified in TS 38.304, the initial value of the HFN part of RX_NEXT is set by UE implementation.

b)	RX_DELIV

This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, and for MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, the initial value of the SN part of RX_DELIV is (x – 0.5 × 2[sl-PDCP-SN-Size–1]) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For broadcast MRBs and multicast MRBs configured for the multicast reception in RRC_INACTIVE state upon cell reselection as specified in TS 38.304, the initial value of the SN part of RX_DELIV is set to (x – 0.5 × 2[PDCP-SN-SizeDL–1]) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs, the initial value of RX_DELIV is set, if provided, by initialRX-DELIV in TS 38.331 [3]. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

NOTE 3:	For broadcast MRBs and multicast MRBs configured for the multicast reception in RRC_INACTIVE state upon cell reselection as specified in TS 38.304, the initial value of the HFN part of RX_DELIV is set by UE implementation.

c)	RX_REORD

This state variable indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.