3GPP TSG-RAN WG2 Meeting #123	R2-2308867

Toulouse, France, August 21-25, 2023

Source:	TCL

Title:	Data collection for AIML methods

Agenda item:	7.16.2.2

Document for:	Discussion and Decision

- Introduction

Considering different LCM purposes/functions (inference, monitoring, offline training) and use-cases have their own requirements for data collection, clarifying these requirements is useful to evaluate the applicability of each existing data collection framework to use in LCM purposes per use cases. Although during previous RAN2 meetings, it was also suggested to study data collection considering following metrics:

a) The content of the data;

b) The data size;

c) Latency, periodicity;

d) Configuration-related requirements and etc,

These requirements highly depend on RAN1 input. The latest RAN2 #122 meeting [1] has spent a lot of time to discuss the draft of LS out on Data Collection, which was approved in the LS on Data Collection Requirements and Assumptions [2]. In this LS, it includes two parts:

Part A: RAN2 made some working assumptions that need RAN1 confirmation;

Part B: RAN2 identified some aspects that need RAN1 input to facilitate further discussion in RAN2.

Hence, for evaluating the existing possible data collection mechanisms, it is a consensus to wait for RAN1 reply. In this contribution, we will continue to analyze the remaining open issues about data collection from RAN2 perspective.

- Discussion

In the LS [2], it has made some RAN2 assumptions in followings:

Assumption 1: RAN2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive): - For model inference of the UE-sided model, input data for model inference is available inside the UE. - For UE-side (real-time) monitoring of the UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW.

For Assumption 1, we think it is premature to make the assumption for UE-sided model. Given the RAN1 AIML-based use cases, which can be categorized as either one-sided model or two-sided model, we understand that the “UE-sided model” in the LS refers to the UE in one-sided model and UE-part of two-sided model.

Let's consider the example for CSI compression using two-sided model use case, based on RAN1 discussion, we can find that the essential contents of the ground-truth CSI at least include channel matrix so far. Although the data can be obtained by the UE by implementation, the corresponding measurement configuration and reporting need to be enhanced. For model monitoring, we think only considering input data is not enough, as the decision may reside on NW side although UE side process the monitoring calculations, hence, the monitoring results are needed to be reported to NW by the UE, which may affect the measurement configuration and reporting from the NW. Moreover, for model inference and model monitoring, the quality of input data is important, some assistance information seems like necessary, which may be included in the existing data collection frameworks, so it may also impact the corresponding configuration.

Based on the discussions above, we conclude that whether there is the spec impact or not, which highly depends on RAN1 discussion. It is noted that the LS’s aim is to assist the applicability of existing data collection frameworks. Therefore, Assumption1, which is not considered urgent present, is taken into account.

Proposal 1: RAN2 is suggested not to make Assumption1 at the moment.

In the previous RAN2 meeting, for analyzing data collection frameworks, a consensus was reached regarding the prioritization of three aspects of LCM, including model inference, model monitoring, and (offline) model training. Meanwhile, the report of [Post122] [060][AIML] Mapping of functions to physical entities (CMCC)[2] has discussed the physical entities of some model operations per use case.

For model training, we copy the discussion result [3] about the mapping of model training to physical entities per use case, as shown in Table1

Table1.Mapping of model training to physical entities

| Sub use cases | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| CSI compression with two-sided model | Model training (offline training) | gNB, OAM, OTT server, UE, [FFS: CN] |
| beam management with UE-side model | Model training (offline training) | UE-side OTT server, UE, [FFS: gNB, OAM, CN] |
| beam management with NW-side model | Model training (offline training) | gNB, OAM, [FFS: CN, OTT server] |
| positioning with UE-side model (case 1 and 2a) | Model training (offline training) | LMF, UE-side OTT server, UE, [FFS: OAM, CN] |
| positioning with LMF-side model (case 2b and 3b) | Model training (offline training) | LMF |
| positioning with gNB-side model (case 3a) | Model training (offline training) | gNB, OAM, LMF |

In the CSI compression with two-sided model, both gNB and UE are capable of performing the model training. As a result, either the gNB or the UE can be the terminating node for the training data, we can deduce that:

    - - For gNB side training, the training data is collected from UE to gNB:

    - - For UE side training, the training data is collected from gNB to UE.

Therefore, the training data can be transferred at Uu for gNB side training and UE side training, which may result in some RAN2 spec impact. It does not mean that excluding the transmission of training data between UE and gNB is out of 3GPP, since the offline training has no latency requirement. While for OTT severs, whatever UE-side OTT server or NW-side OTT server, once the model training data transmission involves OTT servers, it is out of 3GPP scope; for OAM, it is also not in the scope of RAN2; for CN node, we are still unclear why RAN1 use case data needs to involve CN node, it seems like no strong motivation for UE to use CN node, similar with OAM, it should be confirmed by SA2.

In beam management with UE-side model, it is possible for the gNB to perform model training, the gNB typically has a higher capability and can collect extensive training data from different UEs. This abundance of data can potentially enable the gNB to train a more suitable AI/ML model with better generalization ability. The analysis of other mapped entities in, such as the UE-side OTT server, UE, OAM, and CN, is similar with the analysis in CSI compression with two-sided model case, However, further discussion of these entities should not be included in RAN2. And regarding the beam management with NW-side model, the analysis of gNB, OAM, CN and OTT is also similar with the analysis in CSI compression with two-sided model case.

In positioning with UE-side model (case 1 and 2a), LMF is a positioning node introduced in CN, it naturally will be considered in the discussion, but the training data collection between UE and LMF seems like no RAN2 impact and it is within SA2 scope. In the same way, UE-side OTT server, CN, OAM, the related analysis is the same as above.

For mapping physical entities of model inference and model monitoring, [2] gives a very clear summary. Similar with above discussion, we suggest to narrow down the discussion. Hence, considering various possible mapped entities and time imitation, it is better to prioritize the mapped entities which are in RAN2 scope, e.g, UE, gNB.

Proposal 2: Prioritize the mapped entities within RAN2 scope (e.g, UE, gNB.) for model training, model inference and model monitoring.

For CSI compression and Beam management, based on the above discussion, UE side and gNB side need to collect data. Regarding data collection of model training, it is no latency requirement, and collected training data includes ground truth and measurement results (e.g., precoding matrix, channel matrix, beam ID, L1-RSRP and etc), L1 measurement (CSI reporting), L3 measurement (RRM measurements), MDT and UAI can be considered as possible data collection framework. As for data collection of model inference, L1 measurement (CSI reporting) is naturally be regarded as a baseline since short latency and ground truth and measurement results which can be obtained by L1 measurement framework. However, considering the eventual data size requirement and other possible assistant information, L3 measurement and immediate MDT are also possible to further discuss. While for the data collection of model monitoring, if collected data is only ground truth and/or measurement result, in our view, the L1 measurement is also the most suitable framework among the existing data collection frameworks, same as model inference. If collected data includes inference output or other monitoring input, L3 measurement, immediate MDT are also feasible.

Proposal 3: For data collection of model training for CSI compression and BM, L1 measurement (CSI framework reporting), L3 measurement (RRM measurements), MDT and UAI are the possible data collection frameworks.

Proposal 4: For data collection of model inference and monitoring for CSI compression and BM, L1 measurement (CSI reporting) is preferred.

For positioning use case, UE side, gNB side, LMF side need to collect data. LPP framework it is the most suitable among all the existing data collection frameworks.

Proposal 5: For data collection of model training, inference and monitoring for positioning, LPP framework can be considered is preferred.

Mentioned as above, for the data collection framework of CSI and BM case, L1 measurement is preferred as a starting point in our view, it is straightforward that considering the new data content/characteristics seen Table 1 that RAN1 agreed, no matter what L1 downlink reference signal (CSI-RS) measurement and L1 uplink reference signal (SRS) measurement, the legacy L1 measurement configuration and/or reporting are needed to be enhanced. We give a more detailed analysis of data collection framework from UE side model data collection and NW side model data collection, separately.

Table1: Data contents per use case

| Use case | Data contents |
| --- | --- |
| CSI compression using two-sided model use case | Contents of the ground-truth CSI including: Precoding matrix, channel matrix; assistance information (e.g., time stamps, and/or cell ID, assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) |
| Beam management | L1-RSRP; beam ID; additional information for content of the reporting (e.g., timestamps, SNR, data quality.etc) |
| Positioning accuracy enhancement | Ground truth label; measurement (corresponding to model input); Quality indicator; Time stamp; FFS other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.) for data collection; |

- UE side data collection

For the UE side model data collection, the collected data is mainly about model inference input, model monitoring input, also model inference output when model monitoring is located in UE side and model inference is located in NW side. If used downlink reference signal (CSI-RS) measurement, although there is no need to report to gNB, considering the newly agreed collected data content, which is needed to be indicated/included in RRC measurement configuration message. If used uplink reference signal (SRS) measurement, according to the 3GPP protocol, the gNB has control over the uplink data transmission, which implies that the UE does not require any uplink transmission channel state information, therefore, there is no reporting information from UE to gNB. However, for the UE-based AI model operations, such as one-side or two-side, network-side measurement information obtained by UE from the uplink reference signal is mandatory. So it requires to enhance the legacy SRS configuration framework for data collection. Therefore, for this case, a new feedback including collected data content to UE is needed to introduce.

- NW side data collection

Similarly, for the NW side model data collection, the collected data is mainly about model inference input, model monitoring input, also model inference output when model monitoring is located on NW side and model inference is located on UE side. If used downlink reference signal (CSI-RS) measurement, the collected data including new data content would be transferred by CSI reporting message to gNB, legacy L1 measurement configuration and reporting should be enhanced to satisfy these new characteristics requirements, which means the new data content/characteristics should be configured/indicated in measurement configuration and/or reporting configuration. If used uplink reference signal (SRS) measurement, UE only needs to send the RS to gNB, measurement results and other assistant information can be based on gNB implementation. Hence, we think for NW side data collection, if use uplink RS measurement, there seems no need to enhance from data content aspect.

Proposal 6: Assume that regard legacy L1 measurement data collection framework as a starting point

- Measurement configuration and/or reporting configuration needed to be enhanced, at least the new data content indication should be configured;

- A feedback would be introduced of SRS measurement for UE side data collection.

Referring to RAN1 progress about data collection, initiating/triggering data collection will be considered, the related agreements are shown as below:

RAN1 #112 Agreement - In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least - Signaling for triggering the data collection - In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Signaling for triggering the data collection RAN1 #112 bis Agreement Regarding the data collection at UE side for UE-side AI/ML model, study the potential specification impact (if any) to initiate/trigger data collection from RAN1 point of view by considering the following options as a starting point - Option 1: data collection initiated/triggered by configuration from NW - Option 2: request from UE for data collection - FFS: details

Regarding the trigger of data collection raised in RAN1, the following potential options of triggering data collection are easily identified and they are listed below:

For UE side data collection, there are three options for UE side to gather data content:

-Option 1: UE-requested: UE sends a request message to NW to initiate/trigger the procedure of data collection framework, UE collects the data through the triggered data collection framework;

-Option 2: NW-initiated: NW initiates/triggers the procedure of data collection framework by configuration, UE collects the data through the triggered data collection framework;

For NW side data collection, generally, there are three options for NW side to gather data content:

-Option1: UE-requested: UE sends a request message to NW to initiate the procedure of data collection framework. NW collects the data through the data collection framework;

-Option2: NW-initiated: NW initiates/triggers the procedure of data collection framework by configuration, NW collects the data through the triggered data collection framework;

From our perspective, data collection for different LCM purposes does not happen all the time, frequently data collection (e.g., measurement, and/or reporting) will result in significant overhead, resource wastage, transmission conflicts, and may not be conducive to energy efficiency. Considering UE side data collection, the LCM purposes corresponding different uses cases which are located at UE side provide clearer guidance on the specific data content that needs to be collected. In light of this, we prefer option 1 as it is helpful to initiate a suitable data collection framework including RS configuration and/or reporting. For NW side data collection, similarly with UE side data collection, option2 is preferred which facilitates the initiation of a suitable data collection framework including RS configuration/or reporting,

Proposal 7: Regarding UE side and NW side data collection, RAN2 evaluates the trigger of data collection framework, we suggest the following may be taken as the starting points:

- For UE side data collection, study UE-requested data collection framework;

- For NW side data collection, study NW-initiated data collection framework.

- Conclusion

In this contribution, the following observations and proposals have been proposed

Proposal 1: RAN2 is suggested not to make Assumption1 at the moment.

Proposal 2: Prioritize the mapped entities within RAN2 scope (e.g, UE, gNB.) for model training, model inference and model monitoring.

Proposal 3: For data collection of model training for CSI compression and BM, L1 measurement (CSI framework reporting), L3 measurement (RRM measurements), MDT and UAI are the possible data collection frameworks.

Proposal 4: For data collection of model inference and monitoring for CSI compression and BM, L1 measurement (CSI reporting) is preferred.

Proposal 5: For data collection of model training, inference and monitoring for positioning, LPP framework can be considered is preferred.

Proposal 6: Assume that regard legacy L1 measurement data collection framework as a starting point

- Measurement configuration and/or reporting configuration needed to be enhanced, at least the new data content indication should be configured;

- A feedback would be introduced of SRS measurement for UE side data collection.

Proposal 7: Regarding UE side and NW side data collection, RAN2 evaluates the trigger of data collection framework, we suggest the following may be taken as the starting points:

- For UE side data collection, study UE-requested data collection framework;

- For NW side data collection, study NW-initiated data collection framework.

- References

- R2 #122 Chair Notes EOM

- R2-2306906 LS out on Data Collection Requirements and Assumptions(vivo)

- R2-230XXX Report of [Post122][060][AIML] Mapping of functions to physical entities (CMCC)