3GPP TSG-RAN WG2 Meeting #123	R2-2308873

Toulouse, France, Aug. 21st-25th, 2023

Agenda Item:	7.16.2.1

Source: 	China Unicom

Title:  	Discussion on the AI Functional Framework

Document for:	Discussion

Introduction

In the previous meeting, RAN1 and RAN2 has made some progress on R18 AI/ML for Air Interface. In RAN2#122 meeting, RAN2 agreed a general architecture covering the model based and/or functionality based LCM.

In this paper, the functional framework is discussed, specifically regarding the data/information flows and the mapping of functions to physical entities.

Discussion

- 2.1	Functional Framework Details

In RAN2#122 meeting, RAN2 had a discussion on functional architecture. The main blocks/functions of functional framework have already been discussed and agreed online, the agreements of the functional framework are as follows.

Intention is to cover functional arch in general, e.g. covering both be model based and/or functionality based LCM “Model Storage” in the figure is only intended as a reference point (if any) for protocol terminations etc for model transfer/delivery etc. It is not intended to limit where models are actually stored. Add a note for this. Remove “Model” in Model Management and Model Inference and for the actions/the arrow form Management to Inference (to reduce the risk for misunderstanding). Management may be model based management, or functionality-based management. Add a mote for this. With the modifications above Figure 2 from R2-2305327 is agreed

As per these agreements, the agreed functional framework consists of model training, inference, management, data collection and model storage. But there is no consensus on the name and definition of the arrows among different blocks/functions yet. To resolve this issue, [Post122][059][AIML] email discussion [3] is launched to collect companies’ views. In this discussion, four suggestions regarding the arrows among model training, inference, management have been discussed. Additionally, some companies proposed all arrows from/to the “Model Storage” block could use dashed lines, eventually the block itself could also be dashed. However, it’s reasonable to use dashed lines for the block, but only one arrow could use dashed line between the arrow from model training to model storage and the arrow from model storage to inference. The reason is that when model training block and inference block are not mapping to a same physical entity, at least one model transfer/delivery procedure is required from training block to inference block (so at least one solid line is needed).

Besides, arrows among different entities already contain the meaning of transfer/delivery, so the name of the arrow from model storage to inference should remove ‘transfer/delivery’ terminology.

The details of above analysis is reflected in the below figures.

Fig 2.1-1.  Revised AI functional framework (Model Training/Model Storage in one entity)

Fig 2.1-2.  Revised AI functional framework (Inference/Model Storage in one entity)

Proposal 1: “Model Storage” block could use dashed lines to indicate that storage is only intended as a reference point.

Proposal 2: If model training block and inference block are not mapping to a same physical entity, at least one solid line is used for the arrow to/from “Model Storage” block.

Proposal 3: The “transfer/delivery” terminology can be removed from the name of the arrow from “Model Storage” block to “Inference” block, due to the arrows between different entities already contain the meaning of “transfer/delivery”.

- 2.2	Mapping of Functions to Physical Entities

Since the general architecture was agreed in RAN2#122 meeting, it is time to discuss the mapping of functionality to entities. The email discussion [Post122][060][AIML] focus on following LCM purposes:

- Model training

- Inference

- Model transfer/delivery

- Model/functionality monitoring

- Model/functionality control, including selection, (de)activation, switching, fallback

There are many views from various companies on model training (e.g., as mentioned by some companies, since the interaction between OAM and RAN is left to implementation, further discussion is needed on whether OAM can be considered as a mapping entity for model training; CN should be involved model training). Actually, it’s proper to treat OAM as an entity for model training, because OAM has adequate computing resources, which has more computing power than mobile phones and gNB. Besides, OAM can easily collect a large quantity of data through Logged MDT and Immediate MDT procedure.

For CN, since CN never touch the data related to CSI before and the current data collection framework does not support to collect the CSI from UE to CN, CN can be excluded firstly for functionality mapping of the model training at NW side.

Proposal 4: OAM should be considered as an entity of the model training, and CN can be excluded directly for model training at NW side.

Additionally, when UE monitors the performance (UE-sided model/two-sided model), UE should always report performance to the network and NW make decisions of model activation/ deactivation/updating/switching. Different UEs may use different criteria to evaluate the performance of AI-based model/functionality, if different UE vendors adopt criteria to monitor the performance of AI-based model/functionality, it may cause inappropriate decisions, further leading to different user experience among different users. However, users may well come down it to network quality degradation, even though all the AI decisions are made in the UE.

Proposal 5: When UE monitors the performance (UE-sided model/two-sided model), UE should always report performance to the network and NW make decisions of model activation/ deactivation/updating/switching.

Summary

RAN2 is kindly asked to discuss the following proposals:

Proposal 1: “Model Storage” block could use dashed lines to indicate that storage is only intended as a reference point.

Proposal 2: If model training block and inference block are not mapping to a same physical entity, at least one solid line is used for the arrow to/from “Model Storage” block.

Proposal 3: The “transfer/delivery” terminology can be removed from the name of the arrow from “Model Storage” block to “Inference” block, due to the arrows between different entities already contain the meaning of “transfer/delivery”.

Proposal 4: OAM should be considered as an entity of the model training, and CN can be excluded directly for model training at NW side.

Proposal 5: When UE monitors the performance (UE-sided model/two-sided model), UE should always report performance to the network and NW make decisions of model activation/ deactivation/updating/switching.

References

- R2_122_ChairNotes EOM

- [Post122][060][AIML] Mapping of functions to physical entities (CMCC)

- [Post122][059][AIML] TR text proposal (Ericsson)