3GPP TSG-RAN WG2 Meeting #123	R2- 2308878

Toulouse, France, Aug 21th – Aug 25th, 2023

Title	Discussion on MsgA based early indication

Source	LG Electronics

Document for	Discussion and Decision

Agenda Item	8.19.3

1 	Introduction

In RAN1#113[1], it is agreed to support the separated Msg1-based early indication for R18 eRedCap UE.

Agreement - For the “FFS: value(s) of X”, - X = 1/0.5 ms for 15/30 kHz SCS - Legacy default TDRA table and Δ are reused. - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - When Msg1 indication for Rel-18 eRedCap UEs is not configured while Msg1 indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs shall share the PRACH that is configured for Rel-17 RedCap UEs. - Note: Rel-18 eRedCap UEs will be differentiated from Rel-17 RedCap UEs based on Msg3 of Rel-18 eRedCap UEs. - Additional early indication in MsgA PRACH is not supported.

This contribution discusses on MsgA based early indication for Rel-18 eRedCap UE.

2	Discussion

In RAN1#113, it is agreed that MsgA PRACH early indication would not be supported. In other words, RACH partition for Rel-18 early indication cannot be configured for 2-step RA procedure.

Agreement (…omitted) - When Msg1 indication for Rel-18 eRedCap UEs is not configured while Msg1 indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs shall share the PRACH that is configured for Rel-17 RedCap UEs. - Note: Rel-18 eRedCap UEs will be differentiated from Rel-17 RedCap UEs based on Msg3 of Rel-18 eRedCap UEs. - Additional early indication in MsgA PRACH is not supported.

In our understanding, the intention of not supporting the MsgA PRACH in RAN1 is to avoid the duplicated indication via MsgA PRACH and MsgA PUSCH using the separated eLCID value. However, according to the current RACH partitioning framework agreed in Rel-17 [2], the RACH partition selection is performed prior to the RA type selection. Therefore, if the RACH partition for Rel-18 early indication is configured only for 4-step RA, the Rel-18 eRedCap UE cannot perform the 2-step RA procedure regardless of the channel quality, since there is no 2-step RA resource associated with the selected RACH partition (i.e., the RACH partition for Rel-18 early indication)

In detail, if the RACH partition for Rel-18 early indication is configured, the Rel-18 eRedCap UE would perform 4-step RA procedure even though the channel is in good enough to perform the 2-step RA procedure, as follows:

- Selection of UL carrier between NUL/SUL

- BWP selection: performed as specified in clause 5.15 of TS 38.321

- Selection of RACH partition: RACH partition for Rel-18 early indication

- RA type selection based on the selected RACH partition:

  - If the BWP is only configured with 2-step RA resource within the selected RACH partition, perform 2-step RA procedure

  - If the BWP is only configured with 4-step RA resource within the selected RACH partition (i.e., RACH partition for Rel-18 early indication), perform 4-step RA procedure

  - If the BWP is configured with 4-step RA resource and 2-step RA resource within the selected RACH partition, RA type selection is done by RSRP threshold

- Selection of SSB

- …

Observation 1. If the separated RACH partition for Msg1 early indication for Rel-18 eRedCap UE is configured, the eRedCap UE cannot performs 2-step RA procedure, even though the channel quality is good enough to perform 2-step RA.

However, given that the RAN1’s intention is not to restrict the RA type selection for eRedCap UE, the eRedCap UE should be able to perform 2-step RA procedure when the channel quality is good enough, using the Rel-17 RedCap RACH partition or the common RACH partition. However, since this is not aligned with the currant RA procedure with RACH partitioning framework, further RAN2 discussion is needed in order to support 2-step RA procedure for Rel-18 eReCap UE. In RAN2 point of view, following options can be considered:

- Option 1) Modify the RA procedure to allow 2-step RA procedure when RACH partition for Rel-18 early indication is configured, e.g.,

  - Option 1-1) Determine the feature applicability for eRedCap based on RSRP threshold (with same RSRP threshold to determine 2-step RA/4-step RA procedure), i.e., eRedCap is applicable only if the 4-step RA resource can be selected. For example:

    - Rel-18 eRedCap UE determines that the Rel-18 eRedCap early indication (i.e., eRedCap) is applicable only if:

      - If there is no 2-step RA resource in the configured BWP; or

      - If  RSRP of the downlink pathloss reference is less than the threshold

    - Otherwise, Rel-18 eRedCap UE determines that the Rel-18 eRedCap early indication (i.e., eRedCap) is not applicable

  - Option 1-2) During the selection of the RACH partition, the RACH partition for Rel-18 early indication is selected only if the 4-step RA resource can be selected. For example, when there is RACH partition for Rel-18 eRedCap early indication and the RACH partition for Rel-17 RedCap early indication:

    - If there is 2-step RA resource associated with the RACH partition for Rel-17 RedCap early indication and the RSRP of the downlink pathloss reference is larger than the threshold, Rel-18 eRedCap UE selects the the RACH partition for Rel-17 RedCap early indication;

    - Otherwise, Rel-18 eRedCap UE selects the RACH partition for Rel-18 eRedCap early indication

  - Option 1-3) Define a procedure text to determine whether to select RACH partition for Rel-18 early indication after the RA type selection. For example, when there is RACH partition for Rel-18 eRedCap early indication and the RACH partition for Rel-17 RedCap early indication:

    - During the RACH partition selection, Rel-18 eRedCap UE selects two RACH partition i.e., the RACH partition for Rel-18 eRedCap early indication and the RACH partition for Rel-17 RedCap early indication

    - The Rel-18 eRedCap UE performs RA type selection based on selected two RACH partitions

    - Then, the Rel-18 eRedCap finally determines the RACH partition between the RACH partition for Rel-18 eRedCap early indication and the RACH partition for Rel-17 RedCap early indication, based on the selected RA type

- Option 2) Define the RACH partition for early indication of eRedCap UE for 2-step RA and the leave to network implementation to allow the 2-step RA procedure for Rel-18 eRedCap UE.

Observation 2. In order to allow the 2-step RA procedure for Rel-18 eRedCap UE, RACH partitioning framework should be modified or the RACH partition for early indication of eRedCap UE should be supported for 2-step RA.

In our view, modifying the current RA procedure (i.e., Option 1 including Option 1-1 to Option 1-3) would cause very complicated RA procedure, which modifies the RACH partitioning framework defined in Rel-17. In addition, when the fallback procedure is happened from 2-step RA to 4-step RA, the RACH partition should be re-selected in order to perform Msg1-based early indication for Rel-18 eRedCap UE. However, since it is not aligned with the general principle of current RACH partitioning framework, it requires a lot of further discussion to define a procedure for reselection of the RACH partition. Therefore, it would be much simpler to define MsgA PRACH early indication based on the network configuration (i.e., Option 2), and leave to network implementation to support 2-step RA for Rel-18 eRedCap UE. The details on Rel-18 early indication for 2-step RA will be based on the Rel-17 early indication for 2-step RA which is already supported for Rel-17 RedCap UE, so it can be simply implemented without causing the additional discussion.

Proposal 1. Support MsgA PRACH early indication based on the network configuration.

3	Conclusion

In this paper, we discussed our initial view on MsgA early indication for eRedCap UE. The discussion includes the following observations:

Observation 1. If the separated RACH partition for Msg1 early indication for Rel-18 eRedCap UE is configured, the eRedCap UE cannot performs 2-step RA procedure, even though the channel quality is good enough to perform 2-step RA.

Observation 2. In order to allow the 2-step RA procedure for Rel-18 eRedCap UE, RACH partitioning framework should be modified or the RACH partition for early indication of eRedCap UE should be supported for 2-step RA.

Based on the above observations, following is proposed:

Proposal 1. Support MsgA PRACH early indication based on the network configuration.

4	Reference

[1] Draft Report of 3GPP TSG RAN WG1#113 v0.2.0

[2] TS 38.321, v17.5.0, 2023.6