3GPP TSG-RAN WG2 Meeting #123                                                                  R2-2308885

Toulouse, France, August 21-25, 2023

Source:	CATT

Title:	Open Issue on RedCap CFR for MBS Broadcast

Agenda Item:	7.24.2

Document for:	Discussion and Decision

- 1. Introduction

In RAN2#121 meeting, it was agreed to introduce a separate CFR for redcap UEs,

Introduce a separate CFR which can be used when the configured bandwidth for the default CFR in SIB20 exceeds the bandwidth capability of bandwidth limited UEs. This is intended to not have impact on RAN1 or RAN4, and intended to support redcap UEs. CR postponed

In RAN2#122 meeting, the related CR was agreed in principle that,

Agreed in principle (to be resubmitted for final agreement the WG meeting at which R18 TSes shall be created)

This contribution further discusses the open issue on the case that the BWP for RedCap UE is configured but it does not contain CORESET#0.

The proposals are summarized in section 3.

- 2. Discussions

According to current RRC spec as below, the initialDownlinkBWP-RedCap can be configured for RedCap UE.

DownlinkConfigCommonSIB information element

-- ASN1START

-- TAG-DOWNLINKCONFIGCOMMONSIB-START

DownlinkConfigCommonSIB ::=     SEQUENCE {

frequencyInfoDL                 FrequencyInfoDL-SIB,

initialDownlinkBWP              BWP-DownlinkCommon,

bcch-Config                     BCCH-Config,

pcch-Config                     PCCH-Config,

...,

[[

pei-Config-r17                  PEI-Config-r17                         OPTIONAL,     -- Need R

initialDownlinkBWP-RedCap-r17   BWP-DownlinkCommon                     OPTIONAL      -- Need R

]]

}

DownlinkConfigCommonSIB field descriptions

bcch-Config The modification period related configuration.

frequencyInfoDL-SIB Basic parameters of a downlink carrier and transmission thereon.

initialDownlinkBWP The initial downlink BWP configuration for a PCell. The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain. The UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment.

initialDownlinkBWP-RedCap If present, RedCap UEs use this DL BWP instead of initialDownlinkBWP. If the locationAndBandwidth of this BWP contains the entire CORESET#0, the UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment. Otherwise, i.e., if the locationAndBandwidth of this BWP does not contain the entire CORESET#0, the UE uses this BWP for receiving DL messages during initial access (Msg2, MsgB, Msg4) and after initial access. If absent, RedCap UEs use initialDownlinkBWP provided that it does not exceed the RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

According to the field description initialDownlinkBWP-RedCap, it can contain CD-SSB and the entire CORESET#0 or not.

If initialDownlinkBWP-RedCap does not contain CD-SSB and the entire CORESET#0, the search space for broadcast MCCH/MTCH cannot be configured for it. For this case an issue was raised that the current signaling structure does not support to configure the search space for broadcast MCCH/MTCH within the redcap CFR.

Instead considering solution for this issue, it is worth to firstly figure out whether this case is reasonable. On one hand, If initialDownlinkBWP-RedCap does not contain CD-SSB and the entire CORESET#0,it means NW does not want the redcap UE to impact the normal UE,i.e., the initial BWP for normal UEs and the initial BWP for redcap UEs  can be not overlapped due to that the initial BWP for redcap UEs  does not contain CD-SSB and the entire CORESET#0.On the other hand, if redcap CFR is configured in this case, the redcap UEs have to work around CORESET#0 due to that any CFR should contains CORESET#0 according to R17 CFR principle.it defeats the intention of configuring a initialDownlinkBWP-RedCap does not contain CD-SSB and the entire CORESET#0.

Therefore, redcap CFR should not be configured if initialDownlinkBWP-RedCap does not contain CD-SSB and the entire CORESET#0.

Proposal 1: Redcap CFR should not be configured if initialDownlinkBWP-RedCap is configured and it does not contain CD-SSB and the entire CORESET#0.

A TP(in Appendix) based on the IPA CR [1] is also provided to reflect the proposal above.

- 3. Conclusion

Based on the discussions in section 2, the proposals for open issue on RedCap CFR for MBS broadcast are summarized as following,

Proposal 1: Redcap CFR should not be configured if initialDownlinkBWP-RedCap does not contain CD-SSB and the entire CORESET#0.

- 4. Appendix 2: TP to 38.331

### 6.3.1	System information blocks

<<unchanged text skipped>>

#### –	SIB20

SIB20 contains the information required to acquire the MCCH/MTCH configuration for MBS broadcast.

SIB20 information element

-- ASN1START

-- TAG-SIB20-START

SIB20-r17 ::=	SEQUENCE {

mcch-Config-r17                MCCH-Config-r17,

cfr-ConfigMCCH-MTCH-r17        CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

lateNonCriticalExtension       OCTET STRING            OPTIONAL,

...,

[[

cfr-ConfigMCCH-MTCH-RedCap-r18 CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

mcch-ConfigRedCap-r18          MCCH-Config-r17         OPTIONAL   -- Cond CFR-RedCap

]]

}

MCCH-Config-r17 ::= SEQUENCE {

mcch-RepetitionPeriodAndOffset-r17   MCCH-RepetitionPeriodAndOffset-r17,

mcch-WindowStartSlot-r17             INTEGER (0..79),

mcch-WindowDuration-r17              ENUMERATED {sl2, sl4, sl8, sl10, sl20, sl40,sl80, sl160}     OPTIONAL, -- Need S

mcch-ModificationPeriod-r17          ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256,

rf512, rf1024, r2048, rf4096, rf8192, rf16384, rf32768, rf65536}

}

MCCH-RepetitionPeriodAndOffset-r17 ::= CHOICE {

rf1-r17                                INTEGER(0),

rf2-r17                                INTEGER(0..1),

rf4-r17                                INTEGER(0..3),

rf8-r17                                INTEGER(0..7),

rf16-r17                               INTEGER(0..15),

rf32-r17                               INTEGER(0..31),

rf64-r17                               INTEGER(0..63),

rf128-r17                              INTEGER(0..127),

rf256-r17                              INTEGER(0..255)

}

-- TAG-SIB20-STOP

-- ASN1STOP

SIB20 field descriptions

cfr-ConfigMCCH-MTCH Common frequency resource used for MCCH and MTCH reception. If the field is absent, the CFR for broadcast has the same location and size as CORESET#0 and PDSCH configuration of MCCH is the same as PDSCH configuration provided in initialDownlinkBWP in SIB1.

cfr-ConfigMCCH-MTCH-RedCap Common frequency resource used for MCCH and MTCH reception for RedCap UEs. If the field is absent, the RedCap UE can use cfr-ConfigMCCH-MTCH if the UE supports the configured bandwidth. This field should not be present if initialDownlinkBWP-RedCap is configured and it does not contain CD-SSB and the entire CORESET#0.

mcch-WindowDuration Indicates, starting from the slot indicated by mcch-WindowStartSlot, the duration in slots during which MCCH may be scheduled. Absence of this field means that MCCH is only scheduled in the slot indicated by mcch-WindowStartSlot. The network always configures mcch-WindowDuration to be shorter or equal to the length of MCCH repetition period.

mcch-ModificationPeriod Defines periodically appearing boundaries, i.e. radio frames for which SFN mod mcch-ModificationPeriod = 0. The contents of different transmissions of MCCH information can only be different if there is at least one such boundary in-between them. Value rf2 corresponds to two radio frames, value rf4 corresponds to four radio frames and so on.

mcch-RepetitionPeriodAndOffset Defines the length and the offset of the MCCH repetition period. rf1 corresponds to a repetition period length of one radio frame, rf2 corresponds to a repetition period length of two radio frames and so on. The corresponding integer value indicates the offset of the repetition period in the number of radio frames. MCCH is scheduled in the MCCH transmission window starting from each radio frame for which: SFN mod repetition period length = offset of the repetition period.

mcch-WindowStartSlot Indicates the slot in which MCCH transmission window starts.

| Conditional presence | Explanation |
| --- | --- |
| CFR-RedCap | The field is optionally present, Need R, if the configured bandwidth in cfr-ConfigMCCH-MTCH exceeds the RedCap UE capability. It is absent otherwise. |

- 5. Reference

[1] R2-2305955, RedCap CFR for MBS broadcast