3GPP TSG-RAN WG2 Meeting #123	R2-2308891

Toulouse, France, 21st – 25th August, 2023

Agenda item:	7.6.3.1

Source:	Samsung

Title:	On enhancements for neighbour cell measurements

WID/SID:	IoT_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

In RAN#94-e the WID for Rel-18 IoT NTN was agreed [1] and in RAN#98-e it was updated [2]. The updated objectives of the WID is the following:

4.1.1 IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17 This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2] - NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements. 4.1.2 Mobility enhancements The following mobility enhancements objectives are listed. - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Support signalling in system information of neighbour cell ephemeris, for eMTC and NB-IoT [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Define UE RRM core requirements for the above mobility enhancement features [RAN4]. 4.1.3 Further enhancement to discontinuous coverage - Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2. [RAN2, RAN3].

In this contribution we discuss issues related to measurements of neighbouring cells, signaling of neighbour cell ephemeris for IoT NTN and RLF procedures.

# Discussion

- In Rel-17 IoT NTN was introduced with minimal enhancements for mobility purposes. The idea was that mobility enhancements would not be crucial for a first release due to the use case being to deliver small amounts of IoT data. In Rel-18 the use case is for an IoT UE to be connected for a longer time. Regarding performing neighbour cell measurements, there are two objectives considered that are different. One is supporting conditions for performing neighbour cell measurements as those introduced for Rel-17 NB-IoT, the other is supporting the actual signalling as required by RAN4 for neighbour cell measurements in NTN.

## Neighbour cell assistance signaling

- In RAN2#122 the following was agreed:

Agreements:

- Extend the neighbour cell information in existing SIBs (not SIB31) to include satellite ID

- The system Information modification procedure is not triggered for an update of new SIB on neighbor-cell assistance information.

- For NB-IoT, SIBxx is not an essential SIB. UE does not need to consider the cell barred if it is unable to acquire the SIB when scheduled. FFS for eMTC

- In RRC IDLE, how to (re-)acquire neighbour cell assistance information is up to UE’s implementation.

- The satellite ID in the new SIB is an integer of X bits wherein X depends on the maximum number of satellites to be considered for mobility.

- The satellite ID is defined as Radio resource control information element to be used in other configurations.

- If a parameter in the common TA parameters is absent, then the value of the parameter is assumed zero.

- If Kmac is absent, then the value of Kmac for the neighbor satellite in the list is assumed zero. FFS on further optimization on signaling, e.g., signalling explicit value 0 of Kmac.

Agreements:

- Reference location and distanceThresh in SIB31. A change of reference location does not trigger SI modification. A UE does not need to get a new reference location as long as ephemeris and Epoch time are valid (in Connected mode the UE relies on T317)

Agreements:

- For earth-fixed cells, introduce t-ServiceStart for neighbor cells. If UE is aware of the t-ServiceStart of the neighbour cell then may be used (up to UE implementation) to determine when to start measurements of that neighbor cell

- If the serving cell t-service expires, stop T310 (if running) and start T311 (i.e. perform cell search and re-establishment without attempting to recover on the current cell for the duration of T310). FFS on discontinuous coverage

- The distance between the UE and a second reference location (e.g. within a neighbour cell) is not taken into account.

- R18 location and time based trigger for measurements (for connected mode and for idle) apply to both NB-IoT and eMTC.

Proposals for agreement

Proposal 7        (12/16) RAN2 will not consider option (a) Cell stop time of neighbor cell in SIBxx.

Proposal 10     (13/16) For NB-IoT NTN, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN), with the assumption that it is up to the UE to update GNSS location.

Proposal 11     Decide one option (7/15) When the distance between UE and reference location is above a threshold, the UE triggers neighbour cell measurements or (6/15) The same triggered mechanism (connected mode) for neighbor cell measurement can also be reused in RRC_IDLE state.

Proposal 12     Do not consider the option: (2/15) UE starts measurements on a neighbouring cell if the distance between the UE and a first reference location (e.g. within the serving cell) is above a threshold, and the distance between the UE and a second reference location (e.g. within a neighbour cell) is below a threshold.

Proposal 13     (14/16) If ephemeris is absent in a list in SIBxx, serving satellite ephemeris applies. FFS signalling details of other parameters such as validity duration.

Proposals requiring further discussion

Proposal 8        (11/16) Cell start time of quasi-earth fixed neighbor cell is broadcast in SIBxx.

-	Apple thinks we need an exact definition for this.

Proposal 9        (8/16) Reference location + distance threshold of moving neighbor cell is not broadcast in SIBxx.

Proposal 16     (6/16) FFS if the epoch time is absent for a satellite in the list, the implicit epoch time is the end of SI window where the SIB message is received.

Proposal 18     (6/14) FFS whether the satellite ID for the satellite in a list is unique within the context of the identified PLMN.

- For measuring a neighbouring cell it should be possible that the cell is inter- or intra-frequency. This has already been taken into consideration in the latest version of the running RRC CR [3]. In order to integrate NTN with a terrestrial network, it should be possible that inter- or intra-frequency could be either NTN, TN or both.

- Observation 1: To integrate NTN with a terrestrial network, an inter or intra-frequency should be able to be NTN, TN or both NTN and TN.

- If there are terrestrial cells, there is the possibility that a specific frequency only has NTN cells, only has TN cells or has NTN and TN cells. Thus in order for the UE to be able to efficiently detect cells on a frequency efficiently, it needs to be indicated which of these apply for a specific frequency. Without this indication, it would be challenging for a UE to detect a TN cell.

- To signal this, in the appendix an example of how to implement this can be seen.

Proposal 1: Enable signaling that an inter or intra-frequency is NTN, TN or both TN and NTN.

Proposal 2: Introduce choice structure in inter and intra-frequency signaling to indicate that the frequency is NTN, TN or both NTN and TN.

Proposal 3: Agree TP in appendix A1 and A2.

Something else that was discussed is how to indicate that the ephemeris of a cell is the same as the serving satellite. In the new SIBxx only the neighbouring satellite ephemeris is included [3]. This means that it is not reasonable to exclude the ephemeris of a neighbouring satellite because it has the same ephemeris as the serving cell – there is no real system where there are two satellites with exactly identical ephemeris as far as we know. Instead, it may be possible that an intra- or inter-frequency cell may have the ephemeris as the serving cell.

- Observation 2: An intra- or inter-frequency cell may have the same ephemeris as serving cell.

- Thus we think that it should be indicated in SIB3 and SIB5 that ephemeris is the same as the serving cell. This can potentially be done in two ways; 1) explicit indication that ephemeris is the same as serving cell, or 2) introducing a pre-defined satelliteId such as 0 or maxSat-1 that indicates ephemeris being the same as serving cell.

Proposal 4: To indicate a intra or inter-frequency (cell) having the same ephemeris as serving cell, either explicit indication in SIB3 and SIB5, or implicit via pre-defined satelliteId.

- In an e-mail discussion [4] in RAN2#122 there were several agreements on new content for the new SIB or for each individual neighbour satellite assistance information element. One crucial component of that is the validity duration. In NR NTN it was agreed that there will be validity durations for each neighbour satellite information element. The UE may then by implementation decide how to re-acquire SIB19 according to the validity duration. In IoT NTN this cannot work as the UE is not able to read system information in RRC connected mode, so having the UE acquire any type of system information cannot be done by UE implementation.

- Observation 3: SIBxx cannot be acquired by UE implementation in RRC connected mode in IoT NTN.

- For SIB31 there is already T317 and T318 that govern when SIB31 is valid and UE is considered UL synchronized, as well when and during what duration that the SIB31 shall be reacquired. One alternative for SIBxx is to introduce another set of timers to perform the same function. The result of this is that there will be another occasion when the UE tunes away to monitor system information. This makes UE and network scheduler complicated. In order to simplify things, a majority of companies agree that at least T318 can be reused to monitor for SIBxx as well as SIB31. Thus we propose to confirm:

Proposal 5: UE acquires SIBxx during T318, i.e when SIB31 is acquired.

- Having established during what occasion that SIBxx is acquired in RRC connected mode, the question is how long the UE shall consider SIBxx to be valid. This is also equivalent to when the UE shall re-acquire SIBxx. There are two options; 1) UE re-acquires SIBxx in every T318 opportunity, or 2) UE re-acquires SIBxx according to a SIBxx validity duration. The first option simplifies procedures, but also mean that the UE may need to re-acquire the SIBxx at every T318 opportunity. This can be acceptable if the SIB31 validity duration is long, such as for GEO, but in LEO networks this may require the UE to re-acquire the SIBxx wasting battery. This is because the SIBxx will be a rather large SIB compared SIB31 due to multiple ephemeris values. For the second option, this means slightly higher complexity with the benefit of acquiring SIBxx less often. Both of these options are viable, thus we propose that both can be introduced:

Proposal 6: SIBxx validity duration is introduced to guide re-acquiring SIBxx. If not present, the UE acquires SIBxx in every T318 opportunity.

- For the definition of SIBxx validity duration, there needs to be some different considerations. The SIB31 validity duration decides when a UE shall reacquire SIB31, but for the SIBxx validity duration the definition needs to be different. This because T317 already determine when T318 is started, but the SIBxx cannot determine when T318 is started. Thus the SIBxx need to determine when the SIBxx shall be acquired in the next T318 opportunity. This can be seen in Figure 1.

Figure 1. Definition of SIBxx validity duration.

Proposal 7: SIBxx validity duration determines the next T318 duration where SIBxx should be acquired.

For the target cell neighbour cell ephemeris for eMTC, there are also some other considerations needed. In IoT NTN Rel-17, the target cell ephemeris is sent with SIB31 as part of RRCConnectionReconfiguration along with the handover command (mobilityControlInfo) so that IoT NTN UE does not have to read any SIB in connected mode except as indicated by T317. This is because the UE should be configured with all system information required for operating in that cell, so that UE does not need to acquire any system information when having connected. This implies that the SIBxx should be included in the handover command to the UE. If the target cell neighbour cell ephemeris is included in a handover command, it is likely that the handover command may become prohibitively large.

Options can be that a UE acquires the target cell neighbour cell ephemeris for measurements by reading the system information after having connected to the target cell or via dedicated signalling. Thus we propose that RAN2 discuss how the target cell neighbour cell assistance information is acquired when a UE arrives in a cell via handover.

Proposal 8: RAN2 to discuss how UE acquires target cell neighbour cell assistance information during handovers.

## Neighbour cell measurements triggering

For an NB-IoT UE to perform neighbour cell measurements will be a rather time-consuming process involving performing lengthy measurements that in some cases may take a very long time. The UE may also have to acquire neighbour cell ephemeris and synchronize to the moving neighbouring cells to measure them. In many use cases, performing these measurements will not be necessary as the most common IoT use case is to deliver very small packets. Thus to enable power savings, it should be possible for devices to not perform these actions in an NTN network even if the conditions as RAN2 are currently discussing are fulfilled. Thus we propose that RAN2 should consider other conditions for not considering to perform neighbour cell measurements.

Proposal 9: RAN2 to consider use case-based / traffic-based conditions for not performing neighbour cell measurements.

## Idle mode measurement triggering

In the e-mail discussion on mobility enhancements [5], there are the following proposals:

Proposal 8 (14/14): for NB-IoT NTN, location-based measurement initiation can also be optionally used in RRC_IDLE for cell re-selection purposes (like in NR-NTN), with the assumption that it is up to the UE to update GNSS location.

Furthermore, it was agreed in RAN2#122:

- For earth-fixed cells, introduce t-ServiceStart for neighbor cells. If UE is aware of the t-ServiceStart of the neighbour cell then may be used (up to UE implementation) to determine when to start measurements of that neighbor cell

Thus some of the connected mode measurement procedures introduced for RRC connected are also introduced for RRC idle. If t-ServiceStart is introduced for RRC connected, we think that these measurement initiation procedures may also be introduced for RRC idle. The benefit is that the UE can reduce power consumption by avoiding to measure on a frequency or cell before the t-ServiceStart.

Proposal 10: RAN2 to discuss using t-ServiceStart to be used for RRC idle measurement initiation procedures.

## RLF procedures

- The main mechanism for performing connected mode mobility for NB-IoT is through RLF and subsequently the RRC re-establishment procedures. The RRC re-establishment procedures allow for a UE to re-establish its connection to a cell, that may be the same cell or a new cell. During the re-establishment procedure, if the an eNB has the UE context, then the UE does not need to be reconfigured. If an eNB does not have the UE context then there are two alternatives, either the eNB replies with an RRCSetup or the eNB may request the UE context from the eNB that the UE was connected to in the previously. This procedure will be greatly useful for the RLF procedures in an NB-IoT Non-Terrestrial Network.  All together this can be seen in Figure 1.

Figure 2. RRC Re-establishment procedures.

- The enhancements that are currently being discussed with regards to the neighbour cell measurements are targeted at improving the RLF procedures in NB-IoT NTN. In order to improve these procedures, we think that the handling of the UE context can also be improved. One such enhancement is that the UE Context is sent in advance from an eNB that is aware of the potential future eNBs that a UE will connect to after an RLF is triggered. Thus we propose that RAN2 discuss how to improve NB-IoT RLF procedures by sending the UE context in advance:

Proposal 11: Allow sending UE context in advance for faster RLF procedures.

# Conclusion

In this contribution we discussed issues related to enhancements for neighbour cell measurements. We have the following observations and proposals:

- Observation 1: To integrate NTN with a terrestrial network, an inter or intra-frequency should be able to be NTN, TN or both NTN and TN.

- Observation 2: An intra- or inter-frequency cell may have the same ephemeris as serving cell.

- Observation 3: SIBxx cannot be acquired by UE implementation in RRC connected mode in IoT NTN.

Proposal 1: Enable signaling that an inter or intra-frequency is NTN, TN or both TN and NTN.

Proposal 2: Introduce choice structure in inter and intra-frequency signaling to indicate that the frequency is NTN, TN or both NTN and TN.

Proposal 3: Agree TP in appendix A1 and A2.

Proposal 4: To indicate a intra or inter-frequency (cell) having the same ephemeris as serving cell, either explicit indication in SIB3 and SIB5, or implicit via pre-defined satelliteId.

Proposal 5: UE acquires SIBxx during T318, i.e when SIB31 is acquired.

Proposal 6: SIBxx validity duration is introduced to guide re-acquiring SIBxx. If not present, the UE acquires SIBxx in every T318 opportunity.

Proposal 7: SIBxx validity duration determines the next T318 duration where SIBxx should be acquired.

Proposal 8: RAN2 to discuss how UE acquires target cell neighbour cell assistance information during handovers.

Proposal 9: RAN2 to consider use case-based / traffic-based conditions for not performing neighbour cell measurements.

Proposal 10: RAN2 to discuss using t-ServiceStart to be used for RRC idle measurement initiation procedures.

Proposal 11: Allow sending UE context in advance for faster RLF procedures.

# Reference

RP-213596, New WID on IoT NTN enhancements, Mediatek Inc., RAN#94-e, December 2021

RP-223519, Revised WID on IoT NTN enhancements, Mediatek Inc., RAN#98-e, December 2022

R2-2306954, 36331 running CR for IoT NTN, Huawei, RAN2#122, May 2023.

R2-2306664, [offline-102] Mobility enhancements – second round, Qualcomm, RAN2#122, May 2023.

R2-2307192, Report of [Post122][112][IoT NTN enh] Mobility enhancements, Mediatek, RAN2#123, August 2023.

# Appendix

## A1

------------------------ 36.331 R2-2306954  ------------------------

#### –	SystemInformationBlockType3

The IE SystemInformationBlockType3 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SystemInformationBlockType3 information element

-- ASN1START

SystemInformationBlockType3 ::=		SEQUENCE {

cellReselectionInfoCommon			SEQUENCE {

q-Hyst								ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars			SEQUENCE {

mobilityStateParameters				MobilityStateParameters,

q-HystSF						SEQUENCE {

sf-Medium						ENUMERATED {

dB-6, dB-4, dB-2, dB0},

sf-High							ENUMERATED {

dB-6, dB-4, dB-2, dB0}

}

}																OPTIONAL		-- Need OP

},

cellReselectionServingFreqInfo		SEQUENCE {

s-NonIntraSearch					ReselectionThreshold		OPTIONAL,		-- Need OP

threshServingLow					ReselectionThreshold,

cellReselectionPriority				CellReselectionPriority

},

intraFreqCellReselectionInfo		SEQUENCE {

q-RxLevMin							Q-RxLevMin,

p-Max								P-Max						OPTIONAL,		-- Need OP

s-IntraSearch						ReselectionThreshold		OPTIONAL,		-- Need OP

allowedMeasBandwidth				AllowedMeasBandwidth		OPTIONAL,		-- Need OP

presenceAntennaPort1				PresenceAntennaPort1,

neighCellConfig						NeighCellConfig,

t-ReselectionEUTRA					T-Reselection,

t-ReselectionEUTRA-SF				SpeedStateScaleFactors		OPTIONAL		-- Need OP

},

...,

lateNonCriticalExtension				OCTET STRING (CONTAINING SystemInformationBlockType3-v10j0-IEs)	OPTIONAL,

[[	s-IntraSearch-v920					SEQUENCE {

s-IntraSearchP-r9					ReselectionThreshold,

s-IntraSearchQ-r9					ReselectionThresholdQ-r9

}																OPTIONAL,		-- Need OP

s-NonIntraSearch-v920				SEQUENCE {

s-NonIntraSearchP-r9				ReselectionThreshold,

s-NonIntraSearchQ-r9				ReselectionThresholdQ-r9

}																OPTIONAL,		-- Need OP

q-QualMin-r9						Q-QualMin-r9				OPTIONAL,		-- Need OP

threshServingLowQ-r9				ReselectionThresholdQ-r9	OPTIONAL		-- Need OP

]],

[[	q-QualMinWB-r11						Q-QualMin-r9				OPTIONAL	-- Cond WB-RSRQ

]],

[[	q-QualMinRSRQ-OnAllSymbols-r12			Q-QualMin-r9				OPTIONAL			-- Cond RSRQ

]],

[[	cellReselectionServingFreqInfo-v1310 CellReselectionServingFreqInfo-v1310	OPTIONAL,		-- Need OP

redistributionServingInfo-r13			RedistributionServingInfo-r13 OPTIONAL,	--Need OR

cellSelectionInfoCE-r13					CellSelectionInfoCE-r13		OPTIONAL,		-- Need OP

t-ReselectionEUTRA-CE-r13				T-ReselectionEUTRA-CE-r13	OPTIONAL		-- Need OP

]],

[[	cellSelectionInfoCE1-r13				CellSelectionInfoCE1-r13	OPTIONAL	-- Need OP

]],

[[	cellSelectionInfoCE1-v1360			CellSelectionInfoCE1-v1360	OPTIONAL		-- Cond QrxlevminCE1

]],

[[	cellReselectionInfoCommon-v1460		CellReselectionInfoCommon-v1460	OPTIONAL	-- Need OR

]],

[[	cellReselectionInfoHSDN-r15			CellReselectionInfoHSDN-r15	OPTIONAL,		-- Need OR

cellSelectionInfoCE-v1530			CellSelectionInfoCE-v1530		OPTIONAL,		-- Need OP

crs-IntfMitigNeighCellsCE-r15		ENUMERATED {enabled}		OPTIONAL		-- Need OP

]],

[[	cellReselectionServingFreqInfo-v1610	CellReselectionServingFreqInfo-v1610	OPTIONAL	-- Need OR

]],

[[	t-Service-r17						TimeOffsetUTC-r17			OPTIONAL	-- Need OR

]],

[[	t-ServiceStartNeigh-r18				TimeOffsetUTC-r17			OPTIONAL,	-- Need OR

intraFreqNTN-MeasInfo				CHOICE {

ntn-Only							SatelliteAssistanceInfoList-r18,

ntn-Terrestrial						SatelliteAssistanceInfoList-r18,

terrestrialOnly					ENUMERATED {true}

}	 OPTIONAL		-- Need OR

]]

}

SatelliteAssistanceInfoList-r18 ::=			SEQUENCE (SIZE(..maxSat-r18)) OF SatelliteId-r18

--Editor’s Note: FFS whether t-ServiceStartNeigh is per neighbour cell and FFS whether neighbour cell PCIs need to be associated to satellite assistance information.

RedistributionServingInfo-r13 ::=		SEQUENCE {

redistributionFactorServing-r13		INTEGER(0..10),

redistributionFactorCell-r13		ENUMERATED{true}				OPTIONAL,	--Need OP

t360-r13							ENUMERATED {min4, min8, min16, min32,infinity,

spare3,spare2,spare1},

redistrOnPagingOnly-r13				ENUMERATED {true}		OPTIONAL	--Need OP

}

CellReselectionServingFreqInfo-v1310 ::=	SEQUENCE {

cellReselectionSubPriority-r13				CellReselectionSubPriority-r13

}

CellReselectionServingFreqInfo-v1610 ::= SEQUENCE {

altCellReselectionPriority-r16		CellReselectionPriority		OPTIONAL, -- Need OR

altCellReselectionSubPriority-r16	CellReselectionSubPriority-r13	OPTIONAL -- Need OR

}

-- Late non critical extensions

SystemInformationBlockType3-v10j0-IEs ::= SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType3-v10l0-IEs					OPTIONAL

}

SystemInformationBlockType3-v10l0-IEs ::= SEQUENCE {

freqBandInfo-v10l0					NS-PmaxList-v10l0			OPTIONAL,	-- Need OR

multiBandInfoList-v10l0				MultiBandInfoList-v10l0		OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

CellReselectionInfoCommon-v1460 ::=	SEQUENCE {

s-SearchDeltaP-r14					ENUMERATED {dB6, dB9, dB12, dB15}

}

CellReselectionInfoHSDN-r15 ::= SEQUENCE {

cellEquivalentSize-r15				INTEGER(2..16)

}

-- ASN1STOP

------------------------ 36.331 R2-2306954  ------------------------

## A2

------------------------ 36.331 R2-2306954  ------------------------

#### –	SystemInformationBlockType5

The IE SystemInformationBlockType5 contains information relevant for inter-frequency cell re-selection (i.e. information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection) and information relevant for E-UTRA and NR idle/inactive measurements. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

SystemInformationBlockType5 information element

-- ASN1START

SystemInformationBlockType5 ::=		SEQUENCE {

interFreqCarrierFreqList			InterFreqCarrierFreqList,

...,

lateNonCriticalExtension				OCTET STRING	(CONTAINING SystemInformationBlockType5-v8h0-IEs)				OPTIONAL,

[[	interFreqCarrierFreqList-v1250	InterFreqCarrierFreqList-v1250		OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-r12	InterFreqCarrierFreqListExt-r12	OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqListExt-v1280	InterFreqCarrierFreqListExt-v1280	OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqList-v1310		InterFreqCarrierFreqList-v1310		OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-v1310	InterFreqCarrierFreqListExt-v1310	OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqList-v1350		InterFreqCarrierFreqList-v1350	OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-v1350	InterFreqCarrierFreqListExt-v1350	OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqListExt-v1360	InterFreqCarrierFreqListExt-v1360	OPTIONAL	-- Need OR

]],

[[	scptm-FreqOffset-r14				INTEGER (1..8)					OPTIONAL	-- Need OP

]],

[[	interFreqCarrierFreqList-v1530		InterFreqCarrierFreqList-v1530		OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-v1530	InterFreqCarrierFreqListExt-v1530	OPTIONAL,	-- Need OR

measIdleConfigSIB-r15				MeasIdleConfigSIB-r15			OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqList-v1610		InterFreqCarrierFreqList-v1610		OPTIONAL,	-- Need OR

interFreqCarrierFreqListExt-v1610	InterFreqCarrierFreqListExt-v1610	OPTIONAL,	-- Need OR

measIdleConfigSIB-NR-r16			MeasIdleConfigSIB-NR-r16			OPTIONAL	-- Need OR

]],

[[	interFreqCarrierFreqList-v18xy		InterFreqCarrierFreqList-v18xy		OPTIONAL	-- Need OR

]]

}

-- Late non critical extensions

SystemInformationBlockType5-v8h0-IEs ::=	SEQUENCE {

interFreqCarrierFreqList-v8h0 SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v8h0				OPTIONAL,	-- Need OP

nonCriticalExtension			SystemInformationBlockType5-v9e0-IEs							OPTIONAL

}

SystemInformationBlockType5-v9e0-IEs ::=	SEQUENCE {

interFreqCarrierFreqList-v9e0	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v9e0				OPTIONAL,	-- Need OR

nonCriticalExtension			SystemInformationBlockType5-v10j0-IEs	OPTIONAL

}

SystemInformationBlockType5-v10j0-IEs ::=	SEQUENCE {

interFreqCarrierFreqList-v10j0	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v10j0				OPTIONAL,	-- Need OR

nonCriticalExtension			SystemInformationBlockType5-v10l0-IEs		OPTIONAL

}

SystemInformationBlockType5-v10l0-IEs ::=	SEQUENCE {

interFreqCarrierFreqList-v10l0	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v10l0				OPTIONAL,	-- Need OR

nonCriticalExtension			SystemInformationBlockType5-v13a0-IEs			OPTIONAL

}

SystemInformationBlockType5-v13a0-IEs ::=	SEQUENCE {

-- Late non critical extensions from REL-10 upto REL-12

lateNonCriticalExtension		OCTET STRING					OPTIONAL,	-- Need OR

interFreqCarrierFreqList-v13a0	InterFreqCarrierFreqList-v13a0	OPTIONAL,	-- Need OR

-- Late non critical extensions from REL-13

nonCriticalExtension			SEQUENCE {}						OPTIONAL

}

InterFreqCarrierFreqList ::=		SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo

InterFreqCarrierFreqList-v1250 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1250

InterFreqCarrierFreqList-v1310 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1310

InterFreqCarrierFreqList-v1350 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1350

InterFreqCarrierFreqList-v13a0 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1360

InterFreqCarrierFreqList-v1530 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1530

InterFreqCarrierFreqList-v1610 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1610

InterFreqCarrierFreqList-v18xy ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v18xy

InterFreqCarrierFreqListExt-r12 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-r12

InterFreqCarrierFreqListExt-v1280 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v10j0

InterFreqCarrierFreqListExt-v1310 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1310

InterFreqCarrierFreqListExt-v1350 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1350

InterFreqCarrierFreqListExt-v1360 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1360

InterFreqCarrierFreqListExt-v1530 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1530

InterFreqCarrierFreqListExt-v1610 ::=	SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1610

InterFreqCarrierFreqInfo ::=	SEQUENCE {

dl-CarrierFreq						ARFCN-ValueEUTRA,

q-RxLevMin							Q-RxLevMin,

p-Max								P-Max							OPTIONAL,		-- Need OP

t-ReselectionEUTRA					T-Reselection,

t-ReselectionEUTRA-SF				SpeedStateScaleFactors			OPTIONAL,		-- Need OP

threshX-High						ReselectionThreshold,

threshX-Low							ReselectionThreshold,

allowedMeasBandwidth				AllowedMeasBandwidth,

presenceAntennaPort1				PresenceAntennaPort1,

cellReselectionPriority				CellReselectionPriority			OPTIONAL,		-- Need OP

neighCellConfig						NeighCellConfig,

q-OffsetFreq						Q-OffsetRange					DEFAULT dB0,

interFreqNeighCellList				InterFreqNeighCellList			OPTIONAL,		-- Need OR

interFreqExcludedCellList				InterFreqExcludedCellList			OPTIONAL,		-- Need OR

...,

[[	q-QualMin-r9					Q-QualMin-r9					OPTIONAL,		-- Need OP

threshX-Q-r9					SEQUENCE {

threshX-HighQ-r9				ReselectionThresholdQ-r9,

threshX-LowQ-r9					ReselectionThresholdQ-r9

}																OPTIONAL		-- Cond RSRQ

]],

[[	q-QualMinWB-r11					Q-QualMin-r9					OPTIONAL	-- Cond WB-RSRQ

]]

}

InterFreqCarrierFreqInfo-v8h0 ::=		SEQUENCE {

multiBandInfoList					MultiBandInfoList				OPTIONAL	-- Need OR

}

InterFreqCarrierFreqInfo-v9e0 ::=	SEQUENCE {

dl-CarrierFreq-v9e0					ARFCN-ValueEUTRA-v9e0	OPTIONAL,	-- Cond dl-FreqMax

multiBandInfoList-v9e0				MultiBandInfoList-v9e0	OPTIONAL	-- Need OR

}

InterFreqCarrierFreqInfo-v10j0 ::=	SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL	-- Need OR

}

InterFreqCarrierFreqInfo-v10l0 ::=	SEQUENCE {

freqBandInfo-v10l0					NS-PmaxList-v10l0			OPTIONAL,	-- Need OR

multiBandInfoList-v10l0				MultiBandInfoList-v10l0		OPTIONAL	-- Need OR

}

InterFreqCarrierFreqInfo-v1250 ::=		SEQUENCE {

reducedMeasPerformance-r12		ENUMERATED {true}		OPTIONAL,		-- Need OP

q-QualMinRSRQ-OnAllSymbols-r12	Q-QualMin-r9					OPTIONAL	-- Cond RSRQ2

}

InterFreqCarrierFreqInfo-r12 ::=		SEQUENCE {

dl-CarrierFreq-r12					ARFCN-ValueEUTRA-r9,

q-RxLevMin-r12						Q-RxLevMin,

p-Max-r12							P-Max							OPTIONAL,		-- Need OP

t-ReselectionEUTRA-r12				T-Reselection,

t-ReselectionEUTRA-SF-r12			SpeedStateScaleFactors			OPTIONAL,		-- Need OP

threshX-High-r12					ReselectionThreshold,

threshX-Low-r12						ReselectionThreshold,

allowedMeasBandwidth-r12			AllowedMeasBandwidth,

presenceAntennaPort1-r12			PresenceAntennaPort1,

cellReselectionPriority-r12			CellReselectionPriority			OPTIONAL,		-- Need OP

neighCellConfig-r12					NeighCellConfig,

q-OffsetFreq-r12					Q-OffsetRange					DEFAULT dB0,

interFreqNeighCellList-r12			InterFreqNeighCellList			OPTIONAL,		-- Need OR

interFreqExcludedCellList-r12			InterFreqExcludedCellList			OPTIONAL,		-- Need OR

q-QualMin-r12						Q-QualMin-r9					OPTIONAL,		-- Need OP

threshX-Q-r12						SEQUENCE {

threshX-HighQ-r12					ReselectionThresholdQ-r9,

threshX-LowQ-r12					ReselectionThresholdQ-r9

}																	OPTIONAL,	-- Cond RSRQ

q-QualMinWB-r12						Q-QualMin-r9					OPTIONAL,	-- Cond WB-RSRQ

multiBandInfoList-r12				MultiBandInfoList-r11			OPTIONAL,	-- Need OR

reducedMeasPerformance-r12			ENUMERATED {true}				OPTIONAL,	-- Need OP

q-QualMinRSRQ-OnAllSymbols-r12		Q-QualMin-r9					OPTIONAL,	-- Cond RSRQ2

...

}

InterFreqCarrierFreqInfo-v1310	::=	SEQUENCE {

cellReselectionSubPriority-r13		CellReselectionSubPriority-r13		OPTIONAL,		-- Need OP

redistributionInterFreqInfo-r13		RedistributionInterFreqInfo-r13		OPTIONAL, --Need OP

cellSelectionInfoCE-r13				CellSelectionInfoCE-r13			OPTIONAL,	-- Need OP

t-ReselectionEUTRA-CE-r13			T-ReselectionEUTRA-CE-r13		OPTIONAL	-- Need OP

}

InterFreqCarrierFreqInfo-v1350	::= SEQUENCE {

cellSelectionInfoCE1-r13			CellSelectionInfoCE1-r13			OPTIONAL	-- Need OP

}

InterFreqCarrierFreqInfo-v1360	::= SEQUENCE {

cellSelectionInfoCE1-v1360		CellSelectionInfoCE1-v1360	OPTIONAL	-- Cond QrxlevminCE1

}

InterFreqCarrierFreqInfo-v1530	::= SEQUENCE {

hsdn-Indication-r15					BOOLEAN,

interFreqNeighHSDN-CellList-r15		InterFreqNeighHSDN-CellList-r15		OPTIONAL,	-- Need OR

cellSelectionInfoCE-v1530			CellSelectionInfoCE-v1530			OPTIONAL	-- Need OP

}

InterFreqCarrierFreqInfo-v1610	::= SEQUENCE {

altCellReselectionPriority-r16		CellReselectionPriority		OPTIONAL,	-- Need OR

altCellReselectionSubPriority-r16	CellReselectionSubPriority-r13	OPTIONAL,	-- Need OR

rss-ConfigCarrierInfo-r16				RSS-ConfigCarrierInfo-r16		OPTIONAL,	-- Cond RSS

interFreqNeighCellList-v1610			InterFreqNeighCellList-v1610	OPTIONAL	-- Cond RSS

}

InterFreqCarrierFreqInfo-v18xy	::= SEQUENCE {

satelliteAssistanceInfoList-r18		SEQUENCE (SIZE(1..maxSat-r18))  OF SatelliteAssistanceInfo-r18		OPTIONAL 	-- Need OR

}

SatelliteAssistanceInfo-r18 ::=    CHOICE {

ntn-Only						SatelliteId-r18,

ntn-Terrestrial					SatelliteId-r18,

terrestrialOnly						ENUMERATED {true}

}

InterFreqNeighCellList ::=			SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo

InterFreqNeighCellList-v1610 ::=		SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo-v1610

InterFreqNeighHSDN-CellList-r15 ::= SEQUENCE (SIZE (1..maxCellInter)) OF PhysCellIdRange

InterFreqNeighCellInfo ::=			SEQUENCE {

physCellId							PhysCellId,

q-OffsetCell						Q-OffsetRange

}

InterFreqNeighCellInfo-v1610 ::= SEQUENCE {

rss-MeasPowerBias-r16			RSS-MeasPowerBias-r16

}

InterFreqExcludedCellList ::=			SEQUENCE (SIZE (1..maxExcludedCell)) OF PhysCellIdRange

RedistributionInterFreqInfo-r13 ::=		SEQUENCE {

redistributionFactorFreq-r13			RedistributionFactor-r13	OPTIONAL,	--Need OP

redistributionNeighCellList-r13			RedistributionNeighCellList-r13		OPTIONAL	--Need OP

}

RedistributionNeighCellList-r13 ::=		SEQUENCE (SIZE (1..maxCellInter)) OF RedistributionNeighCell-r13

RedistributionNeighCell-r13 ::=		SEQUENCE {

physCellId-r13									PhysCellId,

redistributionFactorCell-r13					RedistributionFactor-r13

}

RedistributionFactor-r13 ::=	INTEGER(1..10)

-- ASN1STOP

------------------------ 36.331 R2-2306954  ------------------------