3GPP TSG-RAN WG2 Meeting #123	R2-2308896

Toulouse, France, 21st – 25th August, 2023

Agenda item:	7.25.1

Source:	Samsung

Title:	On Air to ground system information and adaptations

WID/SID:	NR_ATG

Document for:	Discussion and Decision

# Introduction

In RAN#95-e the WID for Air-to-Ground was agreed. The work item is a RAN4-exclusive work item and the latest WID can be seen below [1]. Furthermore, the work is captured in 38.876 [2].

The core part of this work item includes - Specify features to core specifications for ATG BS and UE [RAN4] - Scenario: - BS on the ground, and the CPE type of UE mounted in the aircraft - A direct radio link between BS on the ground and CPE type of UE mounted in the aircraft - Note: The deployment characteristics described in the justification section shall be taken as a basis for the technical discussion. - Specify core requirements for coexistence between ATG and IMT terrestrial network - Example bands include n1, n78 and n79. - Perform FR1 co-existence evaluation for ATG network (e.g. ACLR, ACS) - Identify key characteristics where it is necessary to differentiate ATG ground-based BS and UEs from conventional ground based BS and UEs - Aim to reuse existing requirements for BS and UE where possible, e.g., - Reuse TN BS requirements for ATG BS - Specify RF requirements for ATG UE/BS - Considering the results of co-existence simulations in terms of impact on emissions and RX requirements, cell sizes and link budgets, technology capabilities, likely BS and UE architectures and other relevant aspects. - Taking into account identified differences between ATG and fully ground based systems - Consider BS type 1-C/1-H/1-O and specify the requirements - Consider conductive requirements for UE - Specify RRM core requirements for ATG UE - Taking into account identified differences between ATG and fully ground based systems - Considering the different nature of ATG UEs and their view of the network, increased cell sizes and other relevant aspects - Specify new UE/BS type(s) for ATG network if necessary

In this contribution we discuss the LSes received from RAN4 ATG, the applicability of NTN SIB19 for ATG and location-based aspects.

# Discussion

## ATG background

- An Air-To-Ground network refers to a network providing connectivity to aircraft in the sky from the ground. This is done via having ground-based cell towers, so-called Air-to-ground base stations (ATG BS) that sends signals up to an aircraft’s antenna of an onboard Air-to-ground terminal (ATG UE). Some of the challenges and characteristics in an Air-to-Ground network are the following:

      - Extremely large inter-site distance and large coverage range. As the number of aircraft are significantly less than handheld UEs, an ATG BS in a cost-effective ATG network may require up to 300 km cell coverage range.

      - Utilizing non-disjoint frequency for deploying both ATG and terrestrial networks. There is an interest in deploying both ATG and terrestrial networks to save frequency resources. But the interference between ATG and terrestrial networks may become non-negligible.

      - More powerful on-board ATG UE. On-board ATG terminals may be much more powerful than a normal terrestrial UE, with higher EIRP due to higher transmission power and larger antenna gains.

- In the RAN4 work item, the following aspects are addressed:

      - Extremely large cell coverage range (up to 300km) and UE speed (up 1200 km/h)

      - Coexistence requirements between ATG and terrestrial network

      - ATG BS/UE core and performance requirement

## LS background

- What can be seen from above is that some of the ATG characteristics are close to that of a 3GPP non-terrestrial network. In a non-terrestrial network, the cells may be very large and the distance between the gNB and UE is also much larger (>100 km) than a terrestrial network, where synchronization using the UE and network node locations are needed. This observation was made in RAN4, which triggered a first LS from RAN4 to RAN2, received in RAN2#121bis-e [3]:

RAN4 agreed ATG UE needs to perform time and/or frequency compensation. ATG BS is expected to provide location information to assist UE-based time and/or frequency pre-compensation. The signaling of BS location could use the IE EphemerisInfo in TS38.331 (as defined for NTN SIB19). Other solutions to provide ATG BS location information are also not precluded in RAN4.

From RAN4 perspective, with the assumption of reusing SIB19 content for ATG, BS shall broadcast EphemerisInfo to ensure ATG BS location can be received for the UEs which rely on SIB EphemerisInfo to do the frequency/time pre-compensation. Note that ATG network will operate on the TN bands and the example bands discussed in RAN4 are including n1, n77 and n79.

RAN4 would like to respectively request RAN2 to check whether SIB19 could be applied for ATG and if not, whether an alternative could be applied to provide the ATG BS location to ATG UE from RAN2 perspective.

- The reply was sent in the same meeting to RAN4 [4]:

RAN2 would like to thank RAN4 for the LS on the applicability of the SIB19 for ATG. RAN2 understands the intention is that ATG BS provides location information to assist UE-based time and/or frequency pre-compensation.

RAN2 will address this and intends to find a solution for Rel-18 (via SIB19 or other SIB). Some companies in RAN2 think sharing exact base station’s location is sensitive. RAN2 kindly asks RAN4 to provide information on any required ATG ground station reference location accuracy (i.e., uncertainty in exact base station location). Meanwhile, RAN2 would like to know if other information except EphemerisInfo of ntn-Config-r17 in SIB19 also needs to be sent to ATG UE (e.g., neighbor ATG cell information ntn-NeighCellConfigList-r17 in SIB19, cell specific koffset in ntn-Config-r17).

- Two new LSes from RAN4 were received in RAN2#123:

- (R4-2310058) [5]:

RAN4 would like to thank RAN2 for the reply LS on the applicability of the SIB19 for ATG. RAN4 has discussed the question from RAN2 on required ATG ground station reference location accuracy and had the following agreements:

- Base station is allowed to not provide the exact base station location. The exact supported accuracy according to RAN4 requirements is still under discussion and it could be sent to RAN2 if it is needed by RAN2.

- RAN4 assumes the BS location accuracy does not need to be specified.

- UE performs the frequency/time pre-compensation based on the indicated BS location and the actual UE location, where the UE frequency/timing pre-compensation does not consider the error due to the misalignment between the indicated BS location and the actual BS location.

In addition, RAN4 would like to respectfully inform RAN2 that RAN4 has agreed to introduce the mechanism of koffset in ATG system as NTN, whether and how to configure koffset is up to network implementation. RAN4 kindly asks RAN2 to take this information into account when considering the applicability of SIB19 for ATG and design corresponding signaling for it.

RAN4 kindly asks RAN2 to take the above information into account.

- (R4-2310152) [6]:

RAN4 has discussed the RRM requirements for ATG UE according to the work item objectives stipulated in RP-230279.

For inter-frequency cell reselection in RRC IDLE and INACTIVE states, RAN4 has agreed to define two sets (set1 and set2) of cell-reselection requirements for ATG UEs (one based on Rel-15 NR reselection requirements and other based on Rel-17 HST requirements). RAN4 has agreed that the network may indicate via system information, whether set1 or set2 requirements apply for a particular cell based on cell configuration.

- Also note that another LS was sent from RAN4 in RAN4#107 to RAN1 with RAN2 in CC in R2-2307037. While no response is expected to be needed, it is something that RAN2 should monitor.

## Applicability of SIB19 for ATG

- In RAN2#121bis-e, whether SIB19 shall be used for ATG was discussed but no consensus was reached. The reply LS stated that RAN2 will look at both options. For using SIB19 in ATG, the benefits mentioned is that re-using SIB19 means that a lot of things from NTN can be reused. For instance, SIB19 already provides the ability of a network to broadcast the location of the synchronization-point via the PVT-field with a zero-speed vector. The cons mentioned is that SIB19 is only for NTN, where a lot of information elements may not be required for ATG.

- For SIB19 for ATG, we believe that re-using SIB19 requires the least specification work. In addition to already providing the location, there are further aspects to consider. SIB19 includes the k-Offset and the ta-Report. K-Offset was agreed to be needed as indicated in the LS from RAN4, while there is a tentative agreement on ta-Report [7]:

Tentative agreement:

Introduce NTN TA reporting for ATG UE.

Whether and how to use the reported TA to avoid UL and DL collision is FFS.

- The SIB19 also includes the neighbour cell ephemeris, which would be needed in order for an ATG to be able to measure neighbour cells, which we assume would be important in an ATG network. We propose that RAN2 either confirms this understanding in RAN2 or sends an LS to RAN4 asking whether neighbour cell location information is needed for RRM purposes.

Observation 1: SIB19 contains k-Offset and ta-Report which have been agreed or tentatively agreed to be introduced for ATG.

Observation 2: SIB19 also contains neighbour cell location info.

Proposal 1: RAN2 to either confirm that neighbour cell location info is needed in ATG for RRM, or send a LS to RAN4 asking to confirm this understanding.

- The only other alternatives for including location information of ATG serving cell and ATG neighbouring cells would be to put all elements in SIB1 or a new SIB. A new SIB will likely require a lot of work, such as defining a new SIB, including SIB during handovers etc. Using SIB1 would unnecessarily bloat SIB1 with location info, where SIB1 mostly contains simple 1-bit flags and similar. Thus we think that SIB19 can be reused.

Observation 3: The alternative option to re-using SIB19 would be to either utilize SIB1 or introduce a new SIB.

Proposal 2: Re-use SIB19 for ATG.

- The work that we think needs to be done are mostly clarifications in relation to NTN and ATG, as RAN2 specifications are dotted with mentions of “NTN”. Instead of going through all procedures and adding “and ATG” where appropriate, we think it can be clarified in Stage 2 on what is applicable for ATG.

Proposal 3: RAN2 to work on clarifying which NTN procedures apply to ATG in Stage 2.

- One issue where there may be a need for procedural simplifications is for uplink sync validity duration. As the ATG base station will not change over time, there is no need to update the ephemeris (=ATG base station position in a ATG network). Thus the procedures for an ATG UE can be to not start the T430 timer. This also means that UL synchronization will not be lost as the timer will not expire.

Proposal 4: Do not start T430 in an ATG network.

## Location-based aspects

### ATG BS location privacy

- One issue that was discussed both in RAN2 and RAN4 is the broadcasting of the base station position. In a satellite network, this is inevitable as the satellite moves and the position is needed for synchronization. However, in a terrestrial network the base station is stationary and thus its position is more sensitive to broadcast. It was for instance not agreed to broadcast the NTN Gateway, which would have been one option instead of providing the TA-Common. In RAN4 the ATG BS location privacy was also discussed and the following agreement was taken [7]:

- Agreement:

  - Base station is allowed to not provide the exact base station location

  - FFS on BS location accuracy

    - Note: Nokia will provide the BS accuracy table with different PRACH formats this week.

    - Do not specify the BS location accuracy in RAN4 specs.

    - Capture the BS location accuracy in reply LS to assist RAN2 signaling design.

- One option to strike a balance between ATG base station location privacy and ability to synchronize is to enable position to be broadcasted with lower accuracy which can then be made more precise when the UE is RRC connected mode and when AS security is established. When the UE synchronizes, the precision will be lower and there will be a hit to performance, but the ATG BS location accuracy is good enough to enable stable communications. When the UE is in connected mode and AS security is established then the ATG UE can be updated with the higher accuracy base station location. This is also reasonable as user plane transmissions are not started before AS security is established, where the more accurate synchronization would be required to enable high performance communication. The high performance communication is likely required for many of the use cases, such as to backhaul passenger traffic on an airliner.

Proposal 5: Allow for lower precision base station position broadcasted in SIB19 that can be updated with higher accuracy in connected mode when AS security has been established.

### Height-components in ATG

- In NTN, the location is used/reported in the following manners [8]:

  - Location-based measurement and CHO triggering

  - Location-based idle/inactive mode measurement-initiation

  - Coarse location reporting, used for network to determine virtual cell ID and tracking area

- For location-based measurement and CHO triggering, the location is encoded using the ReferenceLocation-r17:

- ------- 38.331-h50 -------

–		ReferenceLocation

The IE ReferenceLocation contains location information used as a reference location. The value of the field is same as Ellipsoid-Point defined in TS37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

ReferenceLocation information element

-- ASN1START

-- TAG-REFERENCELOCATION-START

ReferenceLocation-r17 ::= OCTET STRING

-- TAG-REFERENCELOCATION-STOP

-- ASN1STOP

- ------- 38.331-h50 -------

- And the field coarseLocationInfo is defined to be the Ellipsoid-Point as defined in 37.355.  The Ellipsoid point is encoded as follows in 37.355:

- ------- 37.355-h50 -------

–		Ellipsoid-Point

The IE Ellipsoid-Point is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

Ellipsoid-Point ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

- ------- 37.355-h50 -------

- What can be seen from the Ellipsoid-Point is that it only contains the latitude and longitude. In NTN, this makes sense as the UE is expected to be on the ground, but in an ATG network the network is in the sky. This means that when the UE reports its coarse position, this should include the height component. Similarly, for measurement reporting/CHO triggering and location-based measurement initiation using the position, the location should include the height component to enable them to be useful in an ATG network.

Proposal 6: Include height-component in location-based NTN procedures for ATG:

- For coarse location reporting

- Location-based measurement initiation

- Location-based measurement and CHO triggering

# Conclusion

In this contribution we discussed issues related to ATG.

Observation 1: SIB19 contains k-Offset and ta-Report which have been agreed or tentatively agreed to be introduced for ATG.

Observation 2: SIB19 also contains neighbour cell location info.

Observation 3: The option to reusing SIB19 would be to either utilize SIB1 or introduce a new SIB.

Proposal 1: RAN2 to either confirm neighbour cell location info is needed in ATG for RRM, or send LS to RAN4 asking to confirm this.

Proposal 2: Reuse SIB19 for ATG.

Proposal 3: RAN2 to work on clarifying which NTN procedures apply to ATG in Stage 2.

Proposal 4: Do not start T430 in an ATG network.

Proposal 5: Allow for lower precision base station position broadcasted in SIB19 that can be updated with higher accuracy in connected mode when AS security has been established.

Proposal 6: Include height-component in location-based NTN procedures for ATG:

- For coarse location reporting

- Location-based measurement initiation

- Location-based measurement and CHO triggering

# Reference

RP-230279, Revised WID on Air-to-Ground network for NR, CMCC, RAN#99, March, 2023.

3GPP 38.876, “Study on Air-to-Ground network for NR”, Release 18, V0.4.0, May 2023.

R2-2302438, “LS on applicability of SIB19 for NR ATG”, RAN4 (Qualcomm), RAN2#121bis-e, RAN4, April 2023.

R2-2304552, “Reply LS on applicability of SIB19 for NR ATG”, RAN2 (Qualcomm), RAN2#121bis-e, April 2023.

R4-2310058, “Reply LS on applicability of SIB19 for NR ATG”, RAN4 (Qualcomm), RAN4#107, Incheon, Korea, May 2023.

R4-2310152, “LS on signaling for NR ATG”, RAN4 (Ericsson), RAN4#107, Incheon, Korea, May 2023.

R4-2310055, “Adhoc minutes for [107][217] NR_ATG”, Moderator (CMCC), RAN4#107, Incheon, Korea, May 2023.

3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”, V17.5.0, June 2023.