# 3GPP TSG-RAN WG2 Meeting #123	R2-2308909

# Toulouse 21th -25th Aug, 2023

# Source: 			ZTE Corporation, Sanechips

# Title:           	Introduction of FR2 FBG2 CA BW classes

# Agenda item:	6.1.3.2

# Document for: 	Discussion and Decision

- Introduction

In the previous Ran2 meetings, there was a hot discussion on the FR2 FBG2 CA BW classes based on RAN4 LS[1], at last a draft reply LS together with cosigned CRs were provided [2][3][4], but the conclusion is to wait for RAN4’s progress. In this paper, we share our views on this issue based on RAN4’s progress.

- Discussion

During RAN2 discussion, the reason for waiting for RAN4’s progress is that RAN4 was discussing the FBG5. If RAN4 concluded that with the FBG5 there is no need to introduce R/S/T/U in FBG2, then there is no need to discuss it again in the RAN2.

Now according to the RAN4 progress, both the FBG5 bandwidth classes (i.e. R2-R12) and newly introduced FBG2 bandwidth classes (i.e. R/S/T/U) are included in the RAN4’s table as below

Table 5.3A.4-1: CA bandwidth classes (38.101-2)

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| V (Note 4) | 1200 MHz < BWChannel_CA ≤ 1600 MHz | 4 |  |
| W (Note 4) | 1600 MHz < BWChannel_CA ≤ 2000 MHz | 5 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| R | 800 MHz < BWChannel_CA ≤ 1000 MHz | 5 |  |
| S | 1000 MHz < BWChannel_CA ≤ 1200 MHz | 6 |  |
| T | 1200 MHz < BWChannel_CA ≤ 1400 MHz | 7 |  |
| U | 1400 MHz < BWChannel_CA ≤ 1600 MHz | 8 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 |  |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| R2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| R3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| R4 | 400 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| R5 | 500 MHz ≤ BWChannel_CA ≤ 1000 MHz | 5 |  |
| R6 | 600 MHz ≤ BWChannel_CA ≤ 1200 MHz | 6 |  |
| R7 | 700 MHz ≤ BWChannel_CA ≤ 1400 MHz | 7 |  |
| R8 | 800 MHz ≤ BWChannel_CA ≤ 1600 MHz | 8 |  |
| R9 | 900 MHz ≤ BWChannel_CA ≤ 1800 MHz | 9 |  |
| R10 | 1000 MHz ≤ BWChannel_CA ≤ 2000 MHz | 10 |  |
| R11 | 1100 MHz ≤ BWChannel_CA ≤ 2200 MHz | 11 |  |
| R12 | 1200 MHz ≤ BWChannel_CA ≤ 2400 MHz | 12 |  |

Observation 1: Both the FBG5 bandwidth classes (i.e. R2-R12) and newly introduced FBG2 bandwidth classes (i.e. R/S/T/U) were included in RAN4’s spec [5].

Based on the observation 1, to align with RAN4’s progress, RAN2 can approve the CRs together with the Reply LS. For the FBG5 bandwidth classes, it can be discussed independently from the FBG2.

Proposal 1: On introduction of FR2 FBG2 CA BW classes, approve the CRs together with the Reply LS as in [2][3][4]. The introduction of the FBG5 bandwidth classes can be discussed independently from the FBG2.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: Both the FBG5 bandwidth classes (i.e. R2-R12) and newly introduced FBG2 bandwidth classes (i.e. R/S/T/U) were included in RAN4’s spec [5].

Proposal 1: On introduction of FR2 FBG2 CA BW classes, approve the CRs together with the Reply LS as in [2][3][4]. The introduction of the FBG5 bandwidth classes can be discussed independently from the FBG2.

- Reference

- R4-2119966, LS on release independence aspects of newly introduced FR2 CA BW Classes and CBM/IBM UE capability “both”, RAN4.

- R2-2207974, Introduction of FR2 FBG2 CA BW classes, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Ericsson, ZTE Corporation, Sanechips, Qualcomm, Xiaomi Communications	.

- R2-2207975,	Introduction of FR2 FBG2 CA BW classes, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Ericsson, ZTE Corporation, Sanechips, Qualcomm, Xiaomi Communications

- R2-2210241	Reply LS on release independence aspects of newly introduced FR2 CA BW Classes and CBM/IBM UE capability	Nokia, Nokia Shanghai Bell	LS out

- 38101-2 User Equipment (UE) radio transmission and reception v18.2.0 3GPP