3GPP TSG-RAN WG2 Meeting #123	R2-2308911

# Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | DraftCR | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on periodicity for NCR beam configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NetConRepeater |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to an agreement taken by RAN1, periodicity in periodic as well as semi-persistent beam configurations are provided per resource set and the periodicity can be expressed in either slot or millisends. Nevertheless, when the periocity is expressed in millisecond, the maximum value of slot offset should be set according to the reference SCS and not only according to the value of the periodicity. This should be reflected in current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 6.3.2 - Clarified in NCR-PeriodicFwdResourceSet and NCR-SemiPersistentFwdResourceSet that If the periodicity is expressed in [ms], the value of the slot offset should be set according to the following: Slot Offset = Periodicity [ms] * 2^(referenceSCS/15)-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, it would not be possible to set the correct values of slot offset and periodicity for NCR when periodicity is expressed in milliseconds. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is based on the agreed CR for NCR in R2-2306609 with original track changes as accepted. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 6.3.2	Radio resource control information elements

*** Unchanged parts are omitted ***

#### –	NCR-PeriodicFwdResourceSet

The IE NCR-PeriodicFwdResourceSet is used to configure a list of periodic forwarding resources for NCR-Fwd access link. Each periodic forwarding configuration includes a list of periodic forwarding resources, a common periodicity and a common reference SCS.

NCR-PeriodicFwdResourceSet information element

-- ASN1START

-- TAG-NCR-PERIODICFWDRESOURCESET-START

NCR-PeriodicFwdResourceSet-r18 ::=     SEQUENCE {

periodicFwdRsrcSetId-r18                        NCR-PeriodicFwdResourceSetId-r18

periodicFwdRsrcToAdddModList-r18             SEQUENCE (SIZE (1..maxNrofPeriodicFwdResource-r18)) OF NCR-PeriodicFwdResource-r18

OPTIONAL,    -- Need N

periodicFwdRsrcToReleaseList-r18             SEQUENCE (SIZE (1..maxNrofPeriodicFwdResource-r18)) OF NCR-PeriodicFwdResourceId-r18

OPTIONAL,   -- Need N

referenceSCS-r18                           SubcarrierSpacing                       OPTIONAL,    -- Need M

priorityFlag-r18                           ENUMERATED {true}                       OPTIONAL,    -- Need R

...

}

NCR-PeriodicFwdResource-r18 ::=   SEQUENCE {

periodicFwdRsrcId-r18               NCR-PeriodicFwdResourceId-r18,

beamIndex-r18                    INTEGER (0..63),

periodicTimeRsrc-r18             SEQUENCE {

periodicityAndOffset-r18          NCR-PeriodicityAndOffset-r18,

symbolOffset-r18                   INTEGER (0..maxNrofSymbols-1),

durationInSymbols-r18           INTEGER (1..112)

}

}

-- TAG-NCR-PERIODICFWDRESOURCESET-STOP

-- ASN1STOP

| NCR-PeriodicFwdResourceSet field descriptions |  |
| --- | --- |
| beamIndex Indicates logical beam index for NCR-Fwd access link. NCR-Fwd is assumed to be ON over the indicated time domain resource if there is beam indication. |  |
| durationInSymbols Indicates the time duration in number of symbols. |  |
| periodicFwdRsrcToAdddModList List of periodic forwarding resources to be added or modified. |
| periodicFwdRsrcToReleaseList List of periodic forwarding resources to be released. |  |
| periodicityAndOffset Indicates the periodicity and slot offset for the periodic forwarding resource. All the periodic forwarding resources configured within the same resource set should have the same periodicity. If the periodicity is expressed in [ms], the value of the slot offset should be set according to the following: Slot Offset = Periodicity [ms] * 2^(referenceSCS/15)-1 |  |
| priorityFlag Indicates the priority for the list of periodic forwarding resource, if present, the list of periodic forwarding resources will have higher priority over aperiodic beam indication. |  |
| referenceSCS Indicates the reference subcarrier spacing for all the time resource in the list. Only Values kHz15, kHz30, kHz60, kHz120 and kHz240 are applicable. |  |
| symbolOffset Indicates symbol offset in one slot. |  |

#### –	NCR-SemiPersistentFwdResourceSet

The IE NCR-SemiPersistentFwdResourceSet is used to configure a list of semi-persistent forwarding resources for NCR-Fwd access link. Each semi-persistent forwarding resource configuration includes a list of semi-persistent forwarding resources, a common periodicity and a common reference SCS

NCR-SemiPersistentFwdResourceSet information element

-- ASN1START

-- TAG-NCR-SEMIPERSISTENTFWDRESOURCESET-START

NCR-SemiPersistentFwdResourceSet-r18 ::=     SEQUENCE {

semiPersistentFwdRsrcSetId-r18                 NCR-SemiPersistentFwdResourceSetId-r18

semiPersistentFwdRsrcToAdddModList-r18      SEQUENCE (SIZE (1..maxNrofSemiPersistentFwdResource-r18)) OF NCR-SemiPersistentFwdResource-r18

OPTIONAL,   -- Need N

semiPersistentFwdRsrcToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofSemiPersistentFwdResource-r18)) OF

NCR-SemiPersistentFwdResourceId-r18                                                OPTIONAL,   -- Need N

referenceSCS-r18                           SubcarrierSpacing           OPTIONAL,  -- Need M

priorityFlag-r18                           ENUMERATED {true}           OPTIONAL,  -- Need R

...

}

NCR-SemiPersistentFwdResource-r18 ::=   SEQUENCE {

semiPersistentFwdRsrcId-r18                 NCR-SemiPersistentFwdResourceId-r18

beamIndex-r18                             INTEGER (0..63)

semiPersistentTimeRsrc-r18          SEQUENCE {

periodicityAndOffset-r18               NCR-PeriodicityAndOffset-r18,

symbolOffset-r18                         INTEGER (0..maxNrofSymbols-1),

durationInSymbols-r18                INTEGER (1..112)

}

}

-- TAG-NCR-SEMIPERSISTENTFWDRESOURCESET-STOP

-- ASN1STOP

NCR-SemiPersistentFwdResourceSet field descriptions

beamIndex Indicates logical beam index for NCR-Fwd access link. NCR-Fwd is assumed to be ON over the indicated time domain resource if there is beam indication.

durationInSymbols Indicates the time duration in number of symbols.

priorityFlag Indicates the priority for the list of semi-persistent forwarding resource, if present, the list of semi-persistent forwarding resources will have higher priority over aperiodic beam indication.

referenceSCS Indicates the reference subcarrier spacing for all the time resource in the list. Only Values kHz15, kHz30, kHz60, kHz120 and kHz240 are applicable.

semiPersistentFwdRsrcToAdddModList List of semi-persistent forwarding resources to be added or modified.

semiPersistentFwdRsrcToReleaseList List of semi-persistent forwarding resources to be released.

periodicityAndOffset Indicates the periodicity and slot offset for the semi-persistent forwarding resource. All the semi-persistent forwarding resources configured within the same resource set should have the same periodicity. If the periodicity is expressed in [ms], the value of the slot offset should be set according to the following: Slot Offset = Periodicity [ms] * 2^(referenceSCS/15)-1

symbolOffset Indicates symbol offset in one slot.

END OF CHANGES