3GPP TSG-RAN WG2 #123	R2-2308914

Toulouse, France, August 21 – 25, 2023

Agenda Item:	7.16.2.1

Source:	Ericsson

Title:	On UE capability/applicability reporting and functionality-to-entity mapping

Document for:	Discussion, Decision

# 1	Introduction

This is the sixth meeting on which RAN2 discusses the “AI/ML for NR Air Interface” SI (see the approved SID in RP-213599, and the revised version in RP-221348).

During RAN2#122bis-e, this sub-Agenda Item on Architecture matters was briefly discussed, with the main agreements addressing the functional framework.

In this paper we focus on the following:

- UE reporting, including:

  - UE capability reporting, and

  - Reporting of applicability (/suitability) of models/functionalities

- Functionality-to-entity mapping

  - Note: to do this we take on aspects from [Post122][060][AIML]’s discussion.

# 2	Discussion

## 2.1	UE reporting

AIML models/functionalities are developed for specific purposes and under specific conditions, e.g., following certain parameters to adjust to radio scenarios, network deployments, configurations, etc.

However, given the changing nature of the air interface there is of course a risk that these could not always be suitable for the available network context (e.g., the UE can move into a new cell or new environment, there could be radio link issues, lack of feature support, etc…).

Therefore, it appears valid to us to think of means of indicating (non)applicability/(non)suitability of AIML models or functionalities.  

However, it has been brought up by some companies along this SI to introduce a new “dynamic UE capability reporting” mechanisms to address such issue.

We do not believe UE capabilities should be used for this purpose. But RAN2 have not really dig deeper into this and only the following FFS was captured in RAN2#121bis-e notes:

FFS if For UE capability for AIML methods we use the UE capability mechanisms as defined for RRC reported and LPP reported capabilities.

The intention of this document is to address the above FFS and discuss what is needed from a RAN2 perspective.

### 2.1.1	UE capability

We would like to start by emphasizing that even when UE capability-related information can change over time, UEs update this information seldomly. In practice, this information is usually only updated when there is a significant change in the UE's capabilities, e.g., new radio access technology is added or when the UE undergoes a major software or hardware upgrade.

And despite UE capability related matters are to be defined during normative phase, the above needs to be considered now, since just the idea of thinking of introducing a new “dynamic UE capability reporting” framework to RAN2 specification does not appear a reasonable way forward. In short this is due to:

- (as said before) UE capabilities have a “static” nature,

- a dynamic approach would drastically increase the burden on the network, as it increases the complexity in making decisions about how a UE should be configured, and

- (as we discuss below, in Section 2.1.2) there are existing means in RAN2 specification to dynamically point out UE’s needs, or indicate the applicability of features/functionalities under changing scenarios.

Therefore, we suggest starting by agreeing the following:

For UE capability for AIML methods we use the UE capability mechanism defined for RRC and LPP. If needed, RAN2 can study methods to indicate the (non)applicability/(non)suitability of AIML models and/or functionalities after UE capability reporting

Thinking about a potential normative phase, it is now worth discussing what information could be required by RAN2 to be included in the UE capability reporting framework. In this regard, the following is captured by RAN1 in TR 38.843 concerning functionalities and models:

For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. [...] For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. [...] For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

Since it is important to understand what UE capabilities need to describe, to start with, we would like to stress that from the (red) text above, it is not clear to us what RAN1 refers to with these “conditions” indicated/associated with UE capability. At least from a RAN2 perspective some rewording seems needed.

In clause 4.2 of the TR, for functionality and model identification it is not clear what RAN1 refers to with “conditions indicated/associated with UE capability”.

Then, from the TR’s text above we propose for RAN2 to agree on the following understanding. Note that at this point RAN2 should not agree on any specific details regarding UE capability information. Hence the next proposal’s wording.

RAN2 understands that:
A) AIML functionalities refer to features and can be considered “static”, i.e., a UE supports an AIML functionality during a similar timeframe as other UE capabilities.
B) AIML models might be associated to AIML functionalities and can be considered “dynamic”, i.e., models (model IDs) linked to functionalities can change frequently compared to a UE capability update timeframe.

From B) in the Proposal above it is clear that RAN2 should study mechanisms to achieve model identification and the mapping (/relationship) between models and functionalities.

It is FFS how AIML models are identified, and how these could be mapped/related to functionalities.

### 2.1.2	Indicating the (non)applicability of models and functionalities

As described in Proposal 1 above, RAN2 can study methods to indicate the (non)applicability/(non)suitability of AIML models and/or functionalities after UE capability reporting. On this matter, the following is currently mentioned in the TR:

After functionality identification, necessity, mechanisms, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities are studied. [...] After model identification, necessity, mechanisms, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models are studied.

Note that, this should address companies’ initial concerns with respect to models and functionalities not being suitable under changing scenarios or conditions.

And as highlighted in section 2.1.1, there are existing means in RAN2 specification to do so. Below we give some examples.

#### 2.1.2.1	UE Assistance Information

UE Assistance Information can be used by UEs to provide e.g., the (non)applicability of functionalities/models or other concerning information that could be useful towards understanding what UEs support under different conditions.

We could think of two approaches to do so:

- a "reactive” approach, and

- a “proactive” approach.

On the one hand, a reactive reporting would involve the UE to provide information to the NW upon receiving an action from it, e.g., after being configured with a functionality for which its model is not applicable. A UE reacting to a certain configuration could further translate as a simple indication which informs of no applicability. For which later the network can decide what to do with the AIML model/functionality.

While on the other hand, a proactive reporting would involve the UE indicating needs or changes to the network without being prompted. For which RAN2 should study:

- How to handle the potential overhead that could be generated, and

- How to understand the UE’s message.

Note that item “b)” above can prove a strong link with RAN1’s or RAN4’s study. For which we believe that ultimately, RAN2 should prioritize considering reactive UE Assistance Information based solutions.

RAN2 to consider using UE Assistance Information based solutions to indicate needs or updates regarding the applicability of AIML models and functionalities. Prioritize reactive approaches that do not show dependency on other WGs’ discussions.

#### 2.1.2.2	Embedding information in “RRC Complete” messages

In Rel-16, RAN2 introduced the NeedForGaps indication, which is not included in the capability signaling but instead embedded into the RRCReconfiguration procedure. The way this works is that when the network (re)configures the UE with Scells and/or an SCG, it configures the UE to indicate in its response (i.e., RRCReconfigurationComplete) the inter-frequency measurements for which it requires gaps based on the current configuration. One could describe this as a snapshot of the capabilities that is only applicable in the current situation.

A similar approach could be adopted for our SI.

RAN2 to consider using RRC Complete (and related) embedded messages (e.g., similar to NeedForGaps) to indicate needs or updates regarding the applicability of AIML models and functionalities in a given situation/context/scenario.

## 2.2	Mapping of functionalities to entities

The functionality-to-entity mapping is covered in “[Post122][060][AIML] Mapping of functions to physical entities (CMCC)”. We would like to emphasize the importance of this topic as it impacts RAN2’s data collection and model transfer/delivery discussion.

For each scenario (i.e., UE-side or NW- side models), we focus on:

- training, and

- performance monitoring.

And we observe that for data collection, the outcome of this discussion will define the entities handling the data collection for the various LCM phases in each use case, while for model transfer/delivery, there is a large dependency on the entities handling the training.

With regards to the inference phase, this should be less controversial and barely affect RAN2 work.

### 2.2.1	CSI and Beam management use cases

In this section, we describe the entities that should be involved in the CSI/beam management use cases for NW-side models and UE-side models.

#### 2.2.1.1	NW-side model training

When it comes to training, the following was agreed in RAN2#122:

P5a: For the data generation entity and termination entity deployed at different entities, RAN2 assumes: - For CSI enhancement and beam management use cases: - - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server. - For positioning enhancement use case: - - For model training, training data can be generated by UE/gNB and terminated at LMF/OTT server.

Note that these agreements do not differentiate between UE-side models and NW-side models, for which one cannot proceed with the termination point analysis for each scenario. In our view this should be clarified.

For NW-side models, it seems natural to assume, from the use cases in the SI, that for training the data collection initiation/termination point can be in the gNB. A potential OAM involvement in our view does not affect in any way 3GPP specifications. The interaction between OAM and gNB is largely left to NW implementation, and there is no need to define protocols and procedures for the OAM to build a NW-side model or to specify a method to deliver a NW-side model from the OAM to the gNB. 
If OAM is involved, i.e., OAM collecting information for a NW-side model, the only potential standard impact is in the gNB model monitoring and in the UE data collection, for which enhancements to the current observability framework (specified by SA5 in TS 28.552) and MDT (for the UE data collection) may be considered. However, which specific performance measurements taken by the gNB or by the UE would be needed by the OAM to observe the performance of a NW-side model feature is still unknown. If that is of interest should be first studied by RAN1 and then coordinated with SA5.

When it comes to the involvement of a NW-controlled OTT server, we believe that this is certainly possible, but this results in more implementation-based solution that should be left to vendors and that does not impact 3GPP/RAN2.

Furthermore, we see no clear motivation to consider the CN as initiation/termination point for data collection for NW-side models. As said, these are RAN-centric use cases, and the training sessions (data required, triggering conditions) are strictly dependent on the RAN capabilities/implementations and performance monitoring operations that are naturally not available at the CN.

Hence, introducing a framework to exchange such information between CN and RAN proves not to be needed, as it does not provide technical advantages compared to the straightforward solution in which the data collection is handled by gNB. In the latter case, the specification impact is also much lower and confined in RAN1/RAN2.

For training of NW-side models, there is no technical motivation to consider CN involvement.

For training of NW-side models, RAN2 to focus on the gNB as a logical entity for the model training.

Potential OAM involvement in the NW-side model training does not have 3GPP specification impact. Impact on RAN2 protocols for data collection, such as MDT, should be first assessed in RAN1.

For training of NW-side models, OTT alternatives are out of 3GPP scope and should not be further discussed.

#### 2.2.1.2	NW-side model performance monitoring

For performance monitoring of NW-side models, the NW can obviously use off-the-shelf techniques to evaluate the performance of its own models, e.g., legacy L1/L3 measurements. Whether enhancements to the current L1/L3 framework are needed can be studied. On this matter, it is clear that the gNB should be the node handling the performance monitoring of NW-side models.

For performance monitoring of NW-side models, RAN2 considers the gNB as performance monitoring entity.

For NW-side models, if training is done in the OAM, as previously mentioned, we do not expect standardization impact. The need of enhancements to the observability framework defined by SA5 in TS 28.552 should be studied in SA5. However, the input necessary for this study should come from RAN1, and it is too early for RAN2 to take any conclusion on the need of such enhancements.

For performance monitoring of NW-side models at the OAM, e.g., enhancements to the legacy observability framework defined in SA5, RAN2 should wait for progress in RAN1 before assessing such need.

#### 2.2.1.3	UE-side model training

In our view, for UE-side models, the natural assumption is that the AIML training function resides in the UE and/or in an OTT server. The UE can for example perform data collection based on radio configurations and measurements to enrich a model locally stored in the UE, and/or transfer this information to an OTT server which implements the model storage and the model training potentially based on inputs received from different UEs.

Alternative options discussed during the SI consider NW-centric approaches for AIML training of UE-side models (e.g. gNB/OAM centric or CN-centric). However, the advantages of those approaches over the UE/OTT-centric approaches are not clear. The training of UE-side models depends on radio configurations and measurements of the radio environment taken by the UE, which ultimately depend on the very specific UE characteristics (hardware, software). Storing and then training the UE models in the network would be prohibitive from a computation complexity point of view, since a large number of different UE models are expected, considering factors like different UE vendors, different UE releases, different PHY functionalities, different deployment scenarios, etc. Besides, that would imply the need to specify the exact data that the UE should collect for UE-side models and the data collection transferring procedures.

When it comes to CN-centric approaches, it has been discussed to enhance existing frameworks, e.g., EVEX. It is now clear that the EVEX data collection framework is not RAN-centric, since the configuration, and the initiation of the data collection is performed outside the RAN, as well as the data collection itself is terminated outside the RAN. Additionally, the use cases currently addressed by this framework are certainly not RAN-centric, as the cases studied in RAN1 (especially the CSI and beam management use case). So, we see no real benefit of enhancing this framework for the sake of our SI, as there are already straightforward approaches to do so (e.g., storing models in UE-controlled OTT server).

Nonetheless, potential enhancements to EVEX should not affect RAN2’s work, and the analysis of CN-centric approaches for UE-side training should be studied by SA WGs.

For UE-side model training, RAN2 considers the UE as the model training entity. Whether the UE-side model training occurs in a UE-controlled OTT servers is transparent from a 3GPP standpoint.

For UE-side model training, it is left to SA WGs to study whether there is a need to involve CN.

#### 2.2.1.4	UE-side performance monitoring

For performance monitoring of UE-side models, the following was agreed in RAN2#122:

- - For model monitoring at NW side, performance metrics can be generated by UE and terminated at gNB.

In general, the UE can independently monitor its own system/link level performance and can detect or predict potential performance degradation, i.e., UE-side performance monitoring at UE side. However, the network might also be able to monitor UE performances of AIML functionalities (UE-side performance monitoring at NW side). For example, by configuring the UE to report performance metrics concerning a certain AIML functionality, as agreed in RAN2#122. No other entities should be involved in this LCM procedure.

For UE-side performance monitoring at UE side, RAN2 considers the UE as a performance monitoring entity.

For UE-side performance monitoring at NW side, RAN2 considers the gNB as a performance monitoring entity (e.g., UE providing performance metrics to the gNB).

### 2.2.2	Positioning use cases

The following use cases were agreed in RAN1 for positioning:

Agreement (RAN1#110bis) - Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

For positioning, from the above use cases, we can identify UE-, gNB- and LMF-side models. The entities involved for these use cases are described below.

#### 2.2.2.1	UE-side model training

For positioning cases 1 and 2a, UE-side models are used. Following the same logic as for CSI and beam management (see section 2.2.1.3), the natural assumption would be that for positioning UE-side model training the AIML training function resides in the UE and/or in an OTT server. It is not clear why the LMF or the CN should be involved.

For Case 2a, the LMF can certainly ask the UE-side model to generate certain measurements (e.g., timing information of received paths at the UE) to estimate the UE’s location, but such measurements are simply fed to conventional positioning methods (e.g., triangulation) to determine UE location.

For training of UE-side model (case 1/2a), the LMF may provide ground truth labels for building up the training dataset. But the model training is up to each UE vendor, and LMF or CN is not responsible for training the UE-side model.

When it comes to CN-centric approaches, it is not clear which tool should be used for that. If the intention is to use EVEX (as some companies are proposing for the CSI/beam management use case) there is no impact to RAN2’s work, and such an analysis needs to be carried over in SA WGs.

For training of positioning UE-side models, RAN2 considers the UE as the model training entity. Whether the UE-side model training occurs in a UE-controlled OTT servers is transparent from a 3GPP standpoint.

For training of positioning UE-side models, it is left to SA WGs to study whether there is a need to involve CN.

#### 2.2.2.2	UE-side model performance monitoring

For this case we can follow the same logic as for CSI and beam management (see section 2.2.1.4), i.e., the UE can autonomously perform monitoring and generate model monitoring metrics. The final model monitoring decision (e.g., activate or deactivate a UE-side model) could also eventually be done by the NW. Since the LMF is the NW entity involved in positioning handling, the LMF would be the natural candidate. In particular, in RAN2#123 it was agreed that for model monitoring at NW side of UE-side models, the performance metrics can be generated by UE and terminated at LMF.

For performance monitoring at UE side of positioning UE-side models, RAN2 considers the UE as the model monitoring entity.

For performance monitoring at NW side of positioning UE-side models, RAN2 considers the LMF as the NW entity to take final model monitoring decisions (e.g., UE providing model monitoring metrics to the LMF).

#### 2.2.2.3	LMF-side model training

As for the NW-side models for CSI and beam management, the LMF should be the natural termination entity for this case. Whether the LMF can leverage on an OTT server for training/model storing is something that should be left to vendors and does not impact 3GPP/RAN2.

OAM involvement can also be considered, but there is no RAN2 specifications impact either. If any that is for performance monitoring, i.e., enhancements to the current observability framework. However, more investigation is needed in RAN1 before involving SA5.

As for considering the CN as initiation/termination point for LMF training, there is no clear advantage as discussed before for CSI and beam management use cases.

For training of LMF-side models, there is no technical motivation to consider CN involvement.

For training of positioning LMF-side models, RAN2 to focus on the LMF as the model training entity. Potential OAM involvement (if any) is outside RAN2 scope.

For training of positioning LMF-side models, OTT alternatives are out of 3GPP scope and should not be further discussed.

#### 2.2.2.4	LMF-side model performance monitoring

For LMF-side model performance monitoring, LMF is obviously the only entity involved.

For performance monitoring of positioning LMF-side models, RAN2 only considers the LMF as the model monitoring entity.

#### 2.2.2.5	gNB-side model training

For gNB-side model training, the possibility to involve the LMF or even other CN node is also discussed in [Post122][060][AIML]. In our view, like for CSI and beam management training at NW-side, the need of such interaction between the gNB and LMF/CN nodes does not show to be advantageous.

In a typical gNB-side model that has been evaluated by RAN1, a gNB-side model is used to estimate an unobserved direct path ToAs (TimeOfArrival). The gNB is connected for example to a certain number of TRPs, and for each of them, it performs the channel measurements to use as model input, from which it can derive (as model output) a certain number of unobserved direct path ToAs.

The estimated unobserved direct path ToAs are forwarded to the LMF to obtain UE’s positions using conventional positioning algorithms. Hence, to us, the gNB-side AIML positioning is used to assist positioning estimation at the LMF, which is ultimately the recipient of the gNB-side AIML positioning. Since the LMF is ultimately in charge of deriving the UE physical position, we believe that the LMF can provide the gNB with the ground truth labels corresponding to the gNB-side model output (e.g., ToA). After gNB receive the ground truth labels from LMF, the ground truth labels can be paired with the uplink measurements (e.g., UL SRS) to build training data samples, based on which the gNB can then train its model. Thus, the gNB is the model training entity for the gNB-side model.

Otherwise, if the LMF is required to build a gNB-side model, new signalling should be specified on the interface between the gNB and the LMF, e.g., to convey the channel measurements (e.g. UL SRS) from gNB to LMF for building up a training dataset. For model monitoring / model update / model fine-tuning, continuous signalling from gNB to LMF is required, e.g., feedbacks of how the gNB-side model provided by the LMF is behaving which may lead to a quite complicated and inefficient protocol. Moreover, the LMF may not be aware of all the gNBs properties which are necessary to make a model work. At this stage, besides the said measurements, it is not clear what other information the LMF would need from the gNB to build a model suitable for the gNB. 
Hence, we believe that this is an unnecessary complication at this stage, and additionally the study of such protocols is not in the RAN2 domain (it should be done by RAN3).

The LMF may be in charge of providing ground-truth labels to the gNB for the gNB-side models, but the need for the LMF to build a gNB-side model for the gNB is unclear and it should not be studied in RAN2.

Thus, given the gNB-side positioning models use case, we propose the following.

For training of positioning gNB-side models, RAN2 only considers gNB as the entity performing the training. Potential OAM involvement is outside RAN2 scope.

#### 2.2.2.6	gNB-side model performance monitoring

As per the above arguments, the only node that performs performance monitoring of gNB-side models is the gNB. Involvement of other nodes does not make sense, since all the information needed to perform performance monitoring is already available at the gNB.

For performance monitoring of positioning gNB-side models, RAN2 only considers gNB as the model monitoring entity.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In clause 4.2 of the TR, for functionality and model identification it is not clear what RAN1 refers to with “conditions indicated/associated with UE capability”.

Observation 2	For training of NW-side models, there is no technical motivation to consider CN involvement.

Observation 3	For performance monitoring of NW-side models at the OAM, e.g., enhancements to the legacy observability framework defined in SA5, RAN2 should wait for progress in RAN1 before assessing such need.

Observation 4	For training of LMF-side models, there is no technical motivation to consider CN involvement.

Observation 5	The LMF may be in charge of providing ground-truth labels to the gNB for the gNB-side models, but the need for the LMF to build a gNB-side model for the gNB is unclear and it should not be studied in RAN2.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For UE capability for AIML methods we use the UE capability mechanism defined for RRC and LPP. If needed, RAN2 can study methods to indicate the (non)applicability/(non)suitability of AIML models and/or functionalities after UE capability reporting

Proposal 2	RAN2 understands that: A) AIML functionalities refer to features and can be considered “static”, i.e., a UE supports an AIML functionality during a similar timeframe as other UE capabilities. B) AIML models might be associated to AIML functionalities and can be considered “dynamic”, i.e., models (model IDs) linked to functionalities can change frequently compared to a UE capability update timeframe.

Proposal 3	It is FFS how AIML models are identified, and how these could be mapped/related to functionalities.

Proposal 4	RAN2 to consider using UE Assistance Information based solutions to indicate needs or updates regarding the applicability of AIML models and functionalities. Prioritize reactive approaches that do not show dependency on other WGs’ discussions.

Proposal 5	RAN2 to consider using RRC Complete (and related) embedded messages (e.g., similar to NeedForGaps) to indicate needs or updates regarding the applicability of AIML models and functionalities in a given situation/context/scenario.

Proposal 6	For training of NW-side models, RAN2 to focus on the gNB as a logical entity for the model training.

Proposal 7	Potential OAM involvement in the NW-side model training does not have 3GPP specification impact. Impact on RAN2 protocols for data collection, such as MDT, should be first assessed in RAN1.

Proposal 8	For training of NW-side models, OTT alternatives are out of 3GPP scope and should not be further discussed.

Proposal 9	For performance monitoring of NW-side models, RAN2 considers the gNB as performance monitoring entity.

Proposal 10	For UE-side model training, RAN2 considers the UE as the model training entity. Whether the UE-side model training occurs in a UE-controlled OTT servers is transparent from a 3GPP standpoint.

Proposal 11	For UE-side model training, it is left to SA WGs to study whether there is a need to involve CN.

Proposal 12	For UE-side performance monitoring at UE side, RAN2 considers the UE as a performance monitoring entity.

Proposal 13	For UE-side performance monitoring at NW side, RAN2 considers the gNB as a performance monitoring entity (e.g., UE providing performance metrics to the gNB).

Proposal 14	For training of positioning UE-side models, RAN2 considers the UE as the model training entity. Whether the UE-side model training occurs in a UE-controlled OTT servers is transparent from a 3GPP standpoint.

Proposal 15	For training of positioning UE-side models, it is left to SA WGs to study whether there is a need to involve CN.

Proposal 16	For performance monitoring at UE side of positioning UE-side models, RAN2 considers the UE as the model monitoring entity.

Proposal 17	For performance monitoring at NW side of positioning UE-side models, RAN2 considers the LMF as the NW entity to take final model monitoring decisions (e.g., UE providing model monitoring metrics to the LMF).

Proposal 18	For training of positioning LMF-side models, RAN2 to focus on the LMF as the model training entity. Potential OAM involvement (if any) is outside RAN2 scope.

Proposal 19	For training of positioning LMF-side models, OTT alternatives are out of 3GPP scope and should not be further discussed.

Proposal 20	For performance monitoring of positioning LMF-side models, RAN2 only considers the LMF as the model monitoring entity.

Proposal 21	For training of positioning gNB-side models, RAN2 only considers gNB as the entity performing the training. Potential OAM involvement is outside RAN2 scope.

Proposal 22	For performance monitoring of positioning gNB-side models, RAN2 only considers gNB as the model monitoring entity.

# 4	References

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec 2021

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#96, Budapest, Hungary, June 2022

TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, Release 18, v0.1.0, June 2023