3GPP TSG-RAN WG2 #123	R2-2308916

Toulouse, France, August 21 – 25, 2023

Agenda Item:	7.16.2.4

Source:	Ericsson

Title:	Control and monitoring responsibility

Document for:	Discussion, Decision

# 1	Introduction

RAN2 have discussed the NW and UE’s role in model/functionality control and monitoring. On control related matters, the following was agreed during RAN2#121bis-e:

For the CSI compression and beam management use cases, model/function selection/(de)activation/switching/fallback can be UE-initiated or gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model). For the positioning use case, model/function selection/(de)activation/switching/fallback can be UE-initiated or LMF-/ gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model).

Without considering the model sidedness, the above set of agreements paved the way to considering that for all use cases, either the UE or NW can initiate the control of models/functionalities.

Further discussion was carried during RAN2#122 when addressing data collection-related aspects (see the concerning LS to RAN1 in R2-2306906) which added on matters concerning an entity’s ‘monitoring’ responsibilities:

RAN 2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive): - - For model inference of UE-sided model, input data for model inference is available inside the UE. - - For UE-side (real time) monitoring of UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW. P4a: For the latency requirement of data collection, RAN2 assumes: - - for all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection - - for model inference, when required data comes from other entities, there is a latency requirement for data collection - - for model monitoring, when required monitoring data (e.g., performance metric) comes from the other entities, there is a latency requirement for data collection. P5a: For the data generation entity and termination entity deployed at different entities, RAN2 assumes: - For CSI enhancement and beam management use cases: - - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server. - - For NW-sided model inference, input data can be generated by UE and terminated at gNB. - - For UE-side model inference, input data/assistance information can be generated by gNB and terminated at UE. - - For model monitoring at NW side, performance metrics can be generated by UE and terminated at gNB. - For positioning enhancement use case: - - For model training, training data can be generated by UE/gNB and terminated at LMF/OTT server. - - For NW-sided model inference, input data can be generated by UE/gNB and terminated at LMF and/or gNB. - - For UE-side model inference, input data/assistance information can be generated by LMF/gNB and terminated at the UE. - - For model monitoring at NW side, performance metrics can be generated by UE/gNB and terminated at LMF.

However, the agreements do not appear to be fully correlated (even when they are) and RAN2 have not clearly drawn a line when it comes to addressing responsibilities for the various control and monitoring procedures captured in the agreements above. 

The below discussion brings to the table proposals trying to cover the above issue.

# 2	Discussion

## 2.1	Two LCM mechanisms

As per TR 38.843, two different LCM approaches are considered:

- Functionality-based LCM, and

- Model ID-based LCM.

From a RAN2 perspective, the difference between these two boils down to:

- the desired level of detail in managing AIML-related features, and in a certain manner to,

- who oversees the LCM.

In this regard, the following has been captured by RAN1 in TR 38.843:

For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models may not be identified at the Network, and UE may perform model-level LCM. Whether and how much awareness/interaction NW should have about model-level LCM will be studied. For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. [...] In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

One could then say that:

- AIML functionalities refer to features that can eventually be included in UE capabilities.

- AIML models might be associated to AIML functionalities (either intrinsically or explicitly). These could be “dynamic”, i.e., models (model IDs) linked to functionalities can change over time. For which these should not be included in UE capability.

The above can be represented by Figure 1 below, where an AIML-enabled feature (e.g., use case) in the UE can be described by functionalities (e.g., sub use cases) that work according to different AIML models.

AIML feature in the UE (“use case”)

AI/ML Functionality 1
(“sub use case”)

AI/ML Functionality 2
(“sub use case”)

Model ID 1

Model ID 2

Model ID 3

Model ID a

Model ID b

Model ID c

Figure 1. Description of an AIML-enabled feature.

Below we discuss more about control and monitoring responsibilities for model-ID- and functionality-based LCM.

## 2.2	Model-ID-based LCM

The main responsibility for model LCM should in principle fall on the side that performs the inference, since overall, this side would be more familiar with the model’s characteristics (e.g., how it was developed) and its intended use. We do not see why this should not be the case.

The side that performs the inference is familiar with the model’s characteristics and its intended use.

However, as per RAN2’s agreements and discussion, there is still a debate on whether the entity performing inference receives supplementary support (i.e., from another entity) for management/control purposes.

This can be seen in the agreed functional framework in Figure 2 below, since the “Management” and “Inference” blocks are separate blocks.

Figure 2: Functional framework for AI/ML for NR Air Interface

However, we believe that the side performing the inference is at least responsible for data collection for model training and inference which is not clear from the functional framework, i.e.:

- for a NW(UE)-sided model, the NW(UE) side oversees the data collection, the model training, and the model inference.

To achieve the above, there appears to be a need to add some clarifying notes to the TR. This could come from a Note, or eventually when describing the data flows (i.e., the arrows) of the functional framework.

Clarify in the TR that the side performing the inference should at least be responsible for data collection for model training and for inference. RAN2 to discuss whether how this is done, e.g., in the main text, with a note, or when describing the data flows (arrows).

For UE-sided models, we understand that there is a motivation to consider “external” monitoring and control (e.g., for model (de)activation, switching, fallback operation, etc). Let us then consider the three following scenarios:

- a UE is able to effectively monitor and act upon its models, or

- a UE is not able to effectively monitor its models, or

- the NW wants to have awareness/control of the performance of UE-sided models and instruct the UE what to do.

For a), the UE could in principle operate without additional network support and only need to indicate its actions if crucial for a feature’s correct functioning. This is likely to aid the operation of AIML-enabled features, as imposing the network to monitor the performance of several different models trained and implemented on the users is a complex task.

For b), the UE needs network support to setup AIML-enabled features. While for c), the network prefers to support the UE on the features’ operation.

Hence, for a), there is no need for the network to be aware of the models (i.e., model IDs) supported by the UE. While for both b) and c) there is. Therefore, we propose the following.

In a model-ID-based LCM for UE-sided models, there are two cases to consider:
a) UE-sided model monitoring at the UE. Here there is no need for model-ID-awareness in the NW. For this case, the UE can effectively monitor its models and can autonomously trigger required actions (e.g., deactivation, switching, etc). It is FFS whether (and how) the UE needs indicate this to the NW. 
b) UE-sided model monitoring at the NW. Here the model-ID-awareness might be used to facilitate model managing/configurations by the NW (e.g. (non)applicability, model monitoring performance results). It is FFS whether (and how) the UE indicates (or the NW requests) this information.

## 2.3	Functionality-based LCM

As discussed in the beginning (see Proposal 1), for functionality-based management, the NW may only need to be aware AIML-enabled functionalities supported by UEs and there is no need for model-ID-awareness in the NW.

In this regard, functionalities should not be dynamic like the models (i.e., since they ultimately represent features). For which this should then be “business as usual” for RAN2, since in principle it results in the UE indicating to the network its supported (quasi-static) AIML-enabled functionalities (e.g., using the UE capability reporting framework).

There is no motivation to introduce a functionality ID, as this might very well relate to the UE’s capability reporting of AIML-enabled features.

Now, we can follow a similar logic as the one we adopted for model-based LCM, for this case. Hence, the following set of proposals.

The responsibility for functionality LCM is on the side performing the inference.

In a functionality-based LCM for UE-sided models, there are two cases to consider:
a) UE-sided functionality monitoring at the UE. Here, the UE monitors or manages an AIML functionality on its own. So, the UE can autonomously trigger required actions (e.g., deactivation, switching, fallback, etc). It is FFS whether there is a need for the UE to indicate this. 
b) UE-sided functionality monitoring at the NW. Here, the UE reports the outcome of functionality monitoring to the NW, (e.g. (non)applicability, functionality monitoring performance results).

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The side that performs the inference is familiar with the model’s characteristics and its intended use.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Clarify in the TR that the side performing the inference should at least be responsible for data collection for model training and for inference. RAN2 to discuss whether how this is done, e.g., in the main text, with a note, or when describing the data flows (arrows).

Proposal 2	In a model-ID-based LCM for UE-sided models, there are two cases to consider:
a) UE-sided model monitoring at the UE. Here there is no need for model-ID-awareness in the NW. For this case, the UE can effectively monitor its models and can autonomously trigger required actions (e.g., deactivation, switching, etc). It is FFS whether (and how) the UE needs indicate this to the NW. 
b) UE-sided model monitoring at the NW. Here the model-ID-awareness might be used to facilitate model managing/configurations by the NW (e.g. (non)applicability, model monitoring performance results). It is FFS whether (and how) the UE indicates (or the NW requests) this information.

Proposal 3	There is no motivation to introduce a functionality ID, as this might very well relate to the UE’s capability reporting of AIML-enabled features.

Proposal 4	The responsibility for functionality LCM is on the side performing the inference.

Proposal 5	In a functionality-based LCM for UE-sided models, there are two cases to consider: a) UE-sided functionality monitoring at the UE. Here, the UE monitors or manages an AIML functionality on its own. So, the UE can autonomously trigger required actions (e.g., deactivation, switching, fallback, etc). It is FFS whether there is a need for the UE to indicate this.
b) UE-sided functionality monitoring at the NW. Here, the UE reports the outcome of functionality monitoring to the NW, (e.g. (non)applicability, functionality monitoring performance results).

# 4	References

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec 2021

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#96, Budapest, Hungary, June 2022

TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, Release 18, v0.1.0, June 2023