3GPP TSG-RAN WG2 Meeting #123	R2-2308925

# Toulouse, France, 21– 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1658 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on BFI_COUNTER at SCG activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When beam failure is detected while SCG is deactivated (BFI_COUNTER >= beamFailureInstanceMaxCount for the PSCell), the UE initiates RA procedure upon SCG activation. However, the RA procedure is not initiated for SpCell beam failure recovery if beam failure was detected which has some side effects: - CFRA preambles configured for BFR cannot be used in the RA procedure; and - BFI_COUNTER is not set to zero upon RA procedure completion which means that the MAC will trigger a new BFR in case even a single BFI is indicated by the lower layers to MAC. By triggering the RA procedure for SpCell beam failure recovery upon SCG activation in case BFI_COUNTER >= beamFailureInstanceMaxCount for the PSCell, these issues can be avoided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Upon SCG activation, if BFI_COUNTER >= beamFailureInstanceMaxCount for the PSCell, MAC layer indicates to upper layers that a Random Access Procedure (as specified in clause 5.1.1) for SpCell beam failure recovery is needed for SCG activation. Impact analysis Impacted functionality: SCG activation. Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE cannot use network configured CFRA preambles regardless of BFR situation at the PSCell. - If the UE is implemented according to the CR and the network is not, no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - CFRA preambles configured for BFR cannot be used in the RA procedure upon SCG activation; and - BFI_COUNTER is not set to zero upon RA procedure completion which means that the MAC will trigger a new BFR in case even a single BFI is indicated by the lower layers to MAC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 5.29	Activation/Deactivation of SCG

The network may activate and deactivate the configured SCG.

The MAC entity shall for the configured SCG:

1>	if upper layers indicate that SCG is activated:

2>	if BFI_COUNTER >= beamFailureInstanceMaxCount for the PSCell:

3>	indicate to upper layers that a Random Access Procedure (as specified in clause 5.1.1) for SpCell beam failure recovery is needed for SCG activation.

2>	else if the timeAlignmentTimer associated with PTAG is not running:

3>	indicate to upper layers that a Random Access Procedure (as specified in clause 5.1.1) is needed for SCG activation.

2>	activate the SCG according to the timing defined in TS 38.133 [11].

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 associated with this PSCell according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2.2;

2>	apply normal SCG operation including:

3>	SRS transmissions on the PSCell;

3>	CSI reporting for the PSCell;

3>	PDCCH monitoring on the PSCell;

3>	PUCCH transmissions on the PSCell;

3>	transmit on RACH on the PSCell;

3>	initialize Bj for each logical channel to zero.

1>	else if upper layers indicate that the SCG is deactivated:

2>	deactivate all the SCells of the SCG according to clause 5.9;

2>	deactivate SCG according to the timing defined in TS 38.133 [11];

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the PSCell respectively;

2>	suspend any configured uplink grant Type 1 associated with the PSCell;

2>	reset MAC according to clause 5.12.

1>	if the SCG is deactivated:

2>	not transmit SRS on the PSCell;

2>	not report CSI for the PSCell;

2>	not transmit on UL-SCH on the PSCell;

2>	not transmit PUCCH on the PSCell;

2>	not transmit on RACH on the PSCell;

2>	not monitor the PDCCH on the PSCell.

End of Changes