3GPP TSG-RAN WG2 Meeting #123	R2-2308929

Toulouse, France, 21 – 25 August 2023	Revision of R2-2305753

Agenda item:	7.21.3

Source:	Nokia, Nokia Shanghai Bell

Title:	UP impacts of PRACH CE

WID/SID:	NR_cov_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

New work item on further NR coverage enhancements has been approved in RP-221858 and RAN1 has discussed the PRACH CE issues already. For instance, the following agreements have been made by RAN1:

Agreement For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions. - If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported. - FFS: details Agreement Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx beams. Agreement For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions. Note 1: All ROs in one RO group is associated with the same SSB(s). Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. Note 3: whether/how to define “RO group” in specification will be discussed separately Note 4: Valid RO(s) refers to what is defined in existing specification Agreement For multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power ramping is not applied within one RACH attempt. Agreement For multiple PRACH transmissions with same Tx beam, only one RAR window is supported for RAR monitoring for one RACH attempt. - FFS: the start position of the RAR window. - FFS: RA-RNTI. Conclusion For multiple PRACH transmissions within one RACH attempt, they are only transmitted over ROs associated with the same SSB/CSI-RS. Note: This applies for multiple PRACH transmissions with same Tx beam, and also applies for multiple PRACH transmissions with different Tx beam (if supported).

RAN1#112-bis-e agreed the following:

Agreement - Multiple PRACH transmissions within one RACH attempt are only performed within one RO group. - The number of valid ROs in the RO group is equal to one of the configured number(s) of multiple PRACH transmissions. - Note1: If only one value is configured for multiple PRACH transmissions, then the number of valid ROs in the RO group is equal to this value. - Note2: If multiple values are configured for multiple PRACH transmissions, for each value, the number of valid ROs in the RO group is equal to the corresponding number of multiple PRACH transmissions. - Note 3: Valid RO(s) refers to what is defined in existing specification. Agreement The starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions. Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213. Note: The last valid RO is irrespective of whether the PRACH transmission on the last valid RO in the RO group is dropped or not Conclusion There is no consensus to support utilizing different preambles during the multiple PRACH transmissions with the same Tx beam in one attempt.

# 2	Discussion

## 2.1	RAR window

Since single RAR window is applied for the set of PRACH preamble repetitions and the RAR window is started after all of the PRACH preamble transmissions of the set, there is no reason to extend the RAR window for the purpose of PRACH preamble repetitions but the maximum of 10ms RAR window (as in legacy) can be applied.

Proposal 1: RAR window length is not extended for PRACH preamble repetitions, ie., maximum RAR window length is 10ms (as in legacy).

## 2.2	Conditions for using PRACH repetition feature

Since PRACH repetition feature drains lots of RA resources due to one UE transmitting multiple preambles at a time. Furthermore, this naturally generates more UL interference which the gNB needs to handle. Hence, it needs to be defined with care when the UE would be allowed to utilize the PRACH repetition feature for its RA. As discussed in the previous meeting, the RSRP threshold(s) could be used to select between the different number of PRACH repetitions, however, exact absolute RSRP threshold may not be appropriate to decide in the beginning of the RA procedure whether to apply PRACH repetition or not – this is because the pathloss for the UE could be still relatively good and it could survive with normal RA procedure. Given that for normal RA procedure, UE can ramp up its transmission power for each re-attempt, increasing the number of PRACH transmissions would seem beneficial to be only performed if there is no possibility for power ramping, ie., the UE is transmitting with its maximum UL power.

Proposal 2: UE can select PRACH repetition feature at initiation of the RA procedure only if it would need to operate with its maximum UL transmit power for the RA procedure.

# 3	Conclusion

Proposal 1: RAR window length is not extended for PRACH preamble repetitions, ie., maximum RAR window length is 10ms (as in legacy).

Proposal 2: UE can select PRACH repetition feature at initiation of the RA procedure only if it would need to operate with its maximum UL transmit power for the RA procedure.