3GPP TSG-RAN WG2 Meeting #123	R2-2308930

Toulouse, France, 21 – 25 August 2023	Revision of R2-2305754

Agenda item:	7.21.3

Source:	Nokia, Nokia Shanghai Bell

Title:	PRACH CE fallback cases

WID/SID:	NR_cov_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN2 discussed in RAN2#121-bis-e meeting the following RA fallback cases when PRACH preamble repetition is configured in a cell:

• Case 1: Fallback from legacy RA to Msg1 repetition; • Case 2: Fallback from Msg1 repetition with lower number to Msg1 repetition with higher number; • Case 3: Fallback from 2-step RA to 4-step RA with Msg1 repetition; • Case 4: Fallback from CFRA to CBRA with Msg1 repetition.

RAN2 agreed the following:

Agreements - Msg1 repetition with different repetition number {2, 4, 8} are treated a separate feature, and a RACH partition is associated with a specific repetition number (Stage 3 details are FFS, e.g. we should not use all the spare values in the current IE) - RAN2 will not support the fallback from legacy RA to Msg1 repetition and vice versa; Other fall back scenarios are FFS Agreements - RAN2 to further discuss fallback from lower number of MSG1 repetition to higher number which is also FFS for now. We need to understand how to signal this and how this impacts MAC procedure. Agreements - RAN2 to agree to configure multiple RSRP thresholds for different repetition numbers - The RSRP threshold(s) for triggering Msg1 repetition are configured per-BWP

This TDoc discusses the Msg1 repetition fallback cases.

# 2	Discussion

Generally, considering any of the fallback cases above, they require NW to configure exactly the same feature combinations for different type of random accesses (similarly to 2-step vs. 4-step RA in Rel-17 in case of fallback). This would explode the number of required combinations.

Observation 1: Since any fallback requires same feature group combinations to be configured, required number of feature groups would be plenty as long as even a few features are deployed in a cell.

Given the issue of configuring plenty of different feature combinations as well as the complexity of changing the used combination on the fly during the RA procedure, similarly to legacy, the feature combination to be used for the RA procedure should be selected only at the initiation of the RA procedure. Since RAN1 is also discussing different kind of fallback scenarios, RAN2 should indicate to RAN1 that the set(s) of RA resources are only selected at the initiation of the RA procedure.

Proposal 1: Similarly to legacy, set(s) of RA resources are only selected at the initiation of the RA procedure, ie., used feature combination cannot change within the RA procedure. Indicate this to RAN1.

## 2.1	Fallback from 2-step RA to 4-step RA with Msg1 repetition

As discussed, generally, if the UE had selected 2-step RA to begin the RA procedure with, the need to fall back down to 4-step RA with PRACH repetition seems unlikely. This would also mandate the NW to configure same feature combinations for 2-step RA and 4-step RA with Msg1 repetition to allow this which would explode the need for different feature combinations should the NW enable even very few of them. Given that the fallback from 2-step RA to 4-step RA with PRACH repetition could be seen even more complex than fallback from legacy RA to PRACH repetition, it seems reasonable to forget such option.

Proposal 2: Do not support fallback from 2-step RA to 4-step RA with Msg1 repetition.

## 2.2	Fallback from lower number of Msg1 repetitions to higher number of Msg1 repetitions

Since different repetition numbers (2, 4, or 8) require different preambles to be used, requiring to support fallback from lower number of Msg1 repetitions to higher number of Msg1 repetitions would similarly mean exactly the same feature combinations to be configured for all the different repetition numbers. However, if Msg1 repetition (regardless of the number of repetitions) are considered as one feature, this would be simpler. Within the feature (ie., Msg1 repetitions), NW could then configure any number of repetitions possible which the UE then selects based on rules that are to be defined, e.g., by RAN1. Since the UE should be already running with full UL power for the case of Msg1 repetitions, there would be no effect of power ramping in the RA procedure re-attempt and, hence, increasing the number of repetitions would be only possibility to increase the likelihood of successful PRACH attempt.

Proposal 3: Define different number of Msg1 repetitions as single feature in RRC and NW may associate one or multiple number(s) of repetitions for a given RACH partition.

Proposal 4: Support fallback from lower number of Msg1 repetitions to higher number of Msg1 repetitions.

Multiple RSRP thresholds to determine the number of repetitions required at least for the initial RA attempt was agreed by RAN2 in the previous RAN2#122 meeting. These thresholds could be exploited also to determine when the fallback for a higher number of repetitions is meaningful during the RA procedure. Also, the number of attempts can be considered to increase the likelihood of increasing the number of repetitions for the subsequent attempts. For example, to increase the number msg1 repetitions after failing a RA attempt would be for a UE to decrease the value of the measured RSRP by a certain configurable amount even if the actual RSRP value at the time of the RA re-attempt has not changed, ie., the RSRP would be scaled down in terms of the number of RA attempts the UE has performed.

Proposal 5: The determination to increase the number of msg1 repetitions during the RA procedure is based on both the number of RA attempts and the scaled RSRP thresholds at each RA attempt.

# 3	Conclusion

Observation 1: Since any fallback requires same feature group combinations to be configured, required feature groups would be plenty.

Proposal 1: Similarly to legacy, set(s) of RA resources are only selected at the initiation of the RA procedure, ie., used feature combination cannot change within the RA procedure. Indicate this to RAN1.

Proposal 2: Do not support fallback from 2-step RA to 4-step RA with Msg1 repetition.

Proposal 3: Define different number of Msg1 repetitions as single feature in RRC and NW may associate one or multiple number(s) of repetitions for a given RACH partition.

Proposal 4: Support fallback from lower number of Msg1 repetitions to higher number of Msg1 repetitions.

Proposal 5: The determination to increase the number of msg1 repetitions during the RA procedure is based on both the number of RA attempts and the scaled RSRP thresholds at each RA attempt.